PACKAGE- 8D

(Re-Tender)

EPC TENDER FOR INTERIOR & FURNISHING WORKS FOR NALANDA UNIVERSITY LIBRARY BUILDING

NOTICE INVITING TENDER

NALANDA UNIVERSITY, RAJGIR, BIHAR.

Registration with M/s ITI Ltd: - Intending bidders are requested to register themselves with M/s ITI Ltd (if not registered earlier) through https://nalandauniv.ewizard.in/ for obtaining user-id, by paying a registration fee (As given in the e-portal), online tender processing fee etc. Bidders are also required to obtain Digital Signature for participating in the e-tender.

E-Tender Processing Fee (As given in the e-portal), pay to “ITI LTD”. Through e-payment gateway.

For participating in the e-Tendering process of Nalanda University, Rajgir, Bihar, the contractor shall have to get them registered on the site https://nalandauniv.ewizard.in/ by making required payment through only online payment mode so that they will get user ID and Password. This will enable them to access the website, https://nalandauniv.ewizard.in/ with the help of Digital Signature by which they can participate in e-Tender of Nalanda University, Rajgir, Bihar. For this intending bidder may contact following e-Wizard Helpdesk numbers.

E-Wizard Helpdesk

301-302, 3rd Floor,

The Cloverleaf, Plot no.37,

Sector-11, Dwarka,

New Delhi - 110075

Tel: 011-49606060, 8448288980, 8448288984, 8448288982

MOST IMPORTANT

Firms who wish to participate in this tender should be able to demonstrate their ability to conceptualize and deliver a Library Interior that connects with the legacy of the ancient Nalanda Library which forms the central design essence of the tender. The bidders’ potential to deliver the Library interior design in EPC mode shall be adjudicated during their presentations and walkthrough at the technical bid evaluation stage. Failing to present any coherent or creative concept reflecting this legacy-driven vision and lacking cultural integration, thematic narration, and conceptual originality, which are essential to the project’s identity, shall result in disqualification on technical grounds. The bidders should accord utmost importance in this aspect and accordingly prepare their design presentations which shall be called for during technical evaluation process. The bidders should reckon time for the presentation date w.e.f. the date of uploading of the pre-bid minutes / clarifications, if any, by the University. No additional time shall be given once technical presentations date is announced.

NALANDA UNIVERSITY

#### Notice Inviting e-Tenders

The Registrar, Nalanda University invites online the bids from reputed firms/contractors in two bid system in EPC mode for the following work:

NIT No: NU/Engg/2025-26/EPC-03 DATED 05.12.2025

NAME OF WORK: Engineering, Procurement and Construction (EPC) of Interior Works for the Nalanda Library Building at the Main Campus of Nalanda University at Rajgir, Bihar including wooden partitions, cabinets for FCUs, service bands for electrical, HVAC, CCTV, Data, DALI etc., fixed cupboards, storages (normal storage & compactors etc.), wooden flooring (wherever applicable), carpets, acoustic works, wall paneling, false ceiling, blinds, white boards, tack boards, skylights, glazing work, aluminum work, fabrication work, work stations, office tables, customized conference tables, signage & wayfinding, complete reception arrangements, supply and installation of double sided steel book racks, ergonomic reading chair, reading tables, research cubicles, lounge furniture with comfortable seating, heavy duty book trolley / cart, Magazine display racks, Newspaper stands, library grade step-stools, digital library workstations (wire- managed), and any other fixed furniture/interior items as per requirement.

Estimated Cost: INR. 31,04,23,816/- Earnest Money: INR 41,04,238/-

Period of Completion: 06 Months

Last time and date of submission of bid: 31.12.2025 UP TO 03:00 PM

Summary of Scope of Work

This tender is invited for Engineering, Procurement and Construction (EPC) of Interior and furnishing works for the Nalanda Library Building of approximately 16,844 sqm of built up area at Nalanda University Main Campus at Rajgir, Bihar. The conceptual drawings, floor plans are enclosed separately for reference.

The project covers design, procurement, supply, installation, integration, testing, and commissioning of Library furniture and interiors.

The intent is to create a modern, fully automated, accessible, and sustainable library ecosystem supporting academic, research, cultural, and community functions.

- General Requirements

  - All items shall conform to BIS/ISO standards and applicable local building codes.

  - Furniture and materials shall be of ergonomic design, fire-retardant, eco- friendly, durable, and easy to maintain. Please refer to the detailed technical specifications as stipulated in the Tender.

- Library Furniture & Interiors

  - Double-sided steel book rack bay (6 shelves/side, 900mm)

  - Ergonomic reading chair with metal body

  - Wire-managed reading tables (4 seater, 6 seater)

  - Wire-managed customized conference tables

  - Acoustic panel carrels for research cubical

  - Lounge seating with comfortable chairs

  - Book trolley/cart (heavy duty)

  - Magazine display rack (5-shelf, single-sided)

  - Newspaper stand (6-rod)

  - Step stool (library-grade)

  - Digital Library Workstations - modular, wire-managed

  - Interior works in partitions with gypsum/wood/engineered wood, wall paneling, false ceiling, veneering with laminate/wood veneer, wooden flooring, carpeting, glass work, DGU paneling, acoustic treatment, blinds, service bands for electrical, CCTV, Data, DALI etc., signage, POP works, stone work, tack board, glazing work, aluminium works, fabrication work, customized conference tables, etc. all kinds of interior works solutions

  - All kinds of acoustic works

- Museum & Exhibition Zone

  - Modular partitions, illuminated showcases, pedestals, and movable display panels.

- Periodicals & Reference Sections

  - Slanted display racks, reference desks, newspaper stands.

- Circulation & Issue Counters

  - Custom-designed counters to be with integrated RFID staff station (not in scope).

  - Barrier-free design for specially challenged users.

- Reading Zones with Charging Facilities

  - Lounge comfortable seating for long hours

- Children’s Corner

  - Child-safe furniture, low-height shelves, colorful interiors.

- Group Discussion Rooms, Recording Studio & AV Rooms

  - Acoustic wall panels, ergonomic furniture.

  - Glass partitions with soundproofing.

Table of Contents

TENDER NOTICE	9

SECTION 1	11

GENERAL INFORMATION	11

1.1	CHECKLIST BEFORE SUBMISSION OF BIDS	12

1.4	INTRODUCTION TO PROJECT –	25

1.6	GUIDELINES / PROCEDURE TO BE FOLLOWED IN INTRODUCTION OF ‘E’- PROCUREMENT SOLUTION	28

NOTICE INVITING E-TENDER (CPWD-6)	30

EPC TENDER & CONTRACT FOR WORK	39

GENERAL GUIDELINES FOR BIDDERS	45

INFORMATION REGARDING ELIGIBILITY CRITERION	54

FORM A	56

FORM B	57

SOLVENCY CERTIFICATE	57

FORM C	58

FORM E	61

FORM G	66

FORM H	67

INTEGRITY PACT DECLARATION	69

INTEGRITY PACT DECLARATION	70

INTEGRITY PACT DECLARATION (BIDDERS)	71

INTEGRITY AGREEMENT	72

FORMAT OF AGREEMENT	78

PROFORMA OF SCHEDULES: A TO F	80

SCHEDULE A	81

SCHEDULE OF QUANTITIES	81

SCHEDULE B	82

SCHEDULE C	82

SCHEDULE D	83

SCHEDULE E	84

SCHEDULE F (GENERAL RULES & DIRECTIONS)	85

4.1	Minimum Acceptable Specifications for interior works –	98

4.2	Minimum Acceptable Specifications for Supply & Installation of Movable Furniture –	133

4.1.1	General Quality Standards and Certifications	133

4.1.2	Library Book Racks (Fixed Shelving)	134

4.1.3	Comfortable Seating (Reading Area Chairs)	136

4.1.4	Ergonomic Chairs (Adjustable Work/Study Chairs)	137

4.1.5	Study Tables (Reading Tables)	138

4.1.6	Office Tables (Work Tables)	140

4.1.7	Executive Tables (Executive Desks)	141

4.1.8	Workstations (Modular Office Workstations)	142

4.1.9	Storage Units (Cabinets, Shelves, and Filing)	144

4.1.10	Mobile Compactors (High-Density Storage System)	145

4.1.11	Lounge Furniture (Sofas and Soft Seating)	147

4.1.12	Classroom Furniture (Training Room Setup)	148

4.1.13	Museum-Grade Showcases	150

4.1.14	Newspaper and Magazine Display Units	152

4.1.15	Manuscript Display and Storage Units	155

4.1.16	Microfilm Storage Cabinets	158

4.1.17	Retractable Awnings (Motorized Cassette-Type)	161

4.1.18	Heavy-Duty Library Book Trolley	165

1. Scope - Design, manufacture, supply, and installation of heavy-duty, low-noise, maneuverable book trolleys for library operations (shelving, circulation, sorting, inter-stack movement). Trolleys shall be robust for continuous daily use, ergonomically safe, and finished for long service life in an indoor, air-conditioned environment.	165

2. Capacity & Configuration	165

3. Ergonomics & Handling	166

4. Materials & Construction	166

5. Surface Preparation & Finish	166

6. Mobility System (Casters)	167

7. Stability & Safety	167

8. Dimensions (reference standard model)	167

9. Optional Variants / Add-Ons	167

10. Testing & Acceptance (factory + site)	168

11. Compliance & References	168

12. Documentation & Deliverables	168

13. Execution	169

CLAUSE.5.1 EARNEST MONEY	191

GUARANTEE FOR WATER PROOFING TREATMENT	203

ADDITIONAL CONDITIONS FOR GREEN BUILDING PRACTICES	230

C.P.W.D. SAFETY CODES AND RULES	243

C.P.W.D. Contractor’s Labour Regulations	259

LIST OF ACTS AND OMISSONS FOR WHICH FINES CAN BE IMPOSED	266

TENTATIVE SPACE MANAGEMENT PLAN	269

ARCHITECTURAL PLAN OF THE BUILDING	270

LIST OF APPROVED MAKES	274

(INTERIOR WORKS AND ASOCIATED CIVIL WORKS)	274

### TENDER NOTICE

The Registrar, on behalf of Nalanda University, Rajgir, District – Nalanda, Bihar invites online bids basis from reputed, approved, and eligible contractors of CPWD or any State/Central Govt. Departments /PSUs /Institutions /Autonomous Bodies /other reputed firms fulfilling the set eligibility criteria, in two bid system (Eligibility Bid & Financial bid) for the following work:

| Name of Work |  | : | Engineering, Procurement and Construction (EPC) of Interior Works for the Nalanda Library Building at the Main Campus of Nalanda University at Rajgir, Bihar including wooden partitions, cabinets for FCUs, service bands for electrical, HVAC, CCTV, Data, DALI etc., fixed cupboards, storages (normal storage & compactors etc.), wooden flooring (wherever applicable), carpets, acoustic works, wall paneling, false ceiling, blinds, white boards, tack boards, skylights, glazing work, aluminum work, fabrication work, work stations, office tables, customized conference tables, signage & wayfinding, complete reception arrangements, supply and installation of double sided steel book racks, ergonomic reading chair, reading tables, research cubicles, lounge furniture with comfortable seating, heavy duty book trolley / cart, Magazine display racks, Newspaper stands, library grade step-stools, digital library workstations (wire-managed), for Nalanda University Library Building and any other fixed furniture/interior items as per requirement. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Estimated Cost |  | : | Rs. 31,04,23,816/- |  |  |  |
| Earnest Money |  | : | Rs. 41,04,238/- (RTGS/NEFT or 50% RTGS/NEFT and 50% BG) |  |  |  |
| E-Tender Processing fee |  | : | Rs 10,000/- (RTGS/NEFT) plus GST @ 18% |  |  |  |
| Performance Guarantee |  | : | 5% of Tendered Value |  |  |  |
| Security Deposit |  | : | 2.5% of Tendered Value |  |  |  |
|  | Period of Work Completion | : | 06 Months |  |  |  |
|  | Pre- Bid Meeting | : | On 22/12/2025 | at 03:00 PM at NU Project Office |  |  |
|  |  |  | (online meeting link shall be provided separately) |  |  |  |
| Last date & time of Online submission of Bids |  | : | On 31.12.2025 up to 15:00 Hrs. |  |  |  |
| Opening of Technical Bids Online |  | : | On 31.12.2025 15:30 Hrs. |  |  |  |

The bid forms and other details can be obtained from the website https://nalandauniv.edu.in/tenders/ and https://nalandauniv.ewizard.in/

Registrar Nalanda University

Rajgir, Bihar

## SECTION 1

## GENERAL INFORMATION

### CHECKLIST BEFORE SUBMISSION OF BIDS

- Firms who wish to participate in this tender should be able to demonstrate their ability to conceptualize and deliver a Library Interior that connects with the legacy of the ancient Nalanda Library which forms the central design essence of the tender. The bidders’ potential to deliver the Library interior design in EPC mode shall be adjudicated during their presentations. Failing to present any coherent or creative concept reflecting this legacy-driven vision and lacking cultural integration, thematic narration, and conceptual originality, which are essential to the project’s identity, shall result in disqualification on technical grounds. The bidders should accord utmost importance in this aspect and accordingly prepare their design presentations which shall be called for during technical evaluation process. The bidders should reckon time for the presentation date w.e.f. the date of uploading of the pre-bid minutes / clarifications, if any, by the University. No additional time shall be given once technical presentations date is announced.

- The bidders should read all the instructions, terms & conditions, contract clauses, nomenclature of items, specifications etc. contained in the tender documents very carefully, before quoting the rates. The bidder should also read the General Conditions of Contract 2024 for EPC Project (with correction slips up to last date of submission of tenders) which will be part of the agreement.

- The agency shall quote the rate for complete scope of work for both in words and figures in the financial bid.

- The EPC contractor shall quote his rates keeping in mind the scope of work, specifications, terms & conditions, additional conditions, and special conditions etc. and nothing shall be payable extra whatsoever unless otherwise specified.

- The EPC contractor shall also furnish Performance Guarantee of 5% of the tendered amount in addition to the other deposits mentioned elsewhere in this document for proper performance of the agreement. The Performance Guarantee shall be in the shape of Bank guarantee as per Performa given in Annexure-V enclosed. 50% of the performance guarantee shall be released after completion of work and balance will be released after 1 year of successful completion of work. However, this part of performance guarantee will be released against Bank Guarantee which will be kept valid time to time by the bidder as per the direction of Engineer-in-charge.

- In the event of the tender being submitted by a firm, it must be signed separately by each partner thereof or in the event of the absence of any partner, it must be signed on his behalf by a person holding a power of attorney authorizing him to do so. Such power of attorney should be produced with the tender and it must be disclosed that the firm is duly registered under the Indian partnership act, 1952.

- The bidder shall quote their rates considering all prevalent taxes/Cess like GST, Labour Cess or any other tax on material/work as applicable as per the GoI norms and nothing extra shall be paid to the contractor on this account. The University shall deduct from the R/A bills, the TDS as applicable as per the prevailing rates as prescribed by the Government of India time to time.

- The tender, which is not duly signed by authorized signatory or is conditional shall be treated as non-responsive and shall be summarily rejected.

- Online bid documents submitted by intending bidders shall be opened only of those bidders, whose EMD is deposited through NEFT/RTGS or in combination of 50% NEFT/RTGS and 50% in BG (in original) and other documents scanned and uploaded are found in order.

- The contractor shall barricade the workplace (s) confirming to CPCB and NGT guidelines to avoid any pollution at site and the surroundings to the subject building. The contractor’s rate deemed to be inclusive of the barricading of the site as per requirement of these bodies of Central Govt. Apart from this the contractor has to comply with the guidelines issued from time to time by Bihar State Govt. to ensure pollution control measures at site and surroundings at their own cost.

- The arrangement of water suitable to construction works as well as other usage i/c testing of equipment will be arranged by the contractor at site at his own cost from. The contractor will obtain necessary permission from E-in-C with test result of water from source proposed by contractor duly chemical testing report i/c pH value. The department will get independent physical and chemical testing of such source at an interval of every four months. The testing charges of the water tests done will be paid by the department and will be recovered from contractor’s bill. Power supply required for construction; testing & commissioning shall have to be arranged by the bidder at his own costs.

#### INFORMATION & INSTRUCTIONS TO BIDDERS FOR e-TENDERING

The Registrar, Nalanda University, Rajgir, District – Nalanda, Bihar on behalf of Nalanda University invites online composite bids on Engineering Procurement Construction (EPC) basis from reputed, approved and eligible composite category contractors of CPWD or any State/Central Govt. Departments/PSUs/Institutions/Autonomous Bodies/ other reputed firms fulfilling the set eligibility criteria, in two bid system (Eligibility Bid & Financial bid) for following work:

| NIT No. | NU/Engg/2025-26/EPC-03 dated 05.12.2025 |
| --- | --- |
| Name of Work | Engineering, Procurement and Construction (EPC) of Interior Works for the Nalanda Library Building at the Main Campus of Nalanda University at Rajgir, Bihar including wooden partitions, cabinets for FCUs, service bands for electrical, HVAC, CCTV, Data, DALI etc., fixed cupboards, storages (normal storage & compactors etc.), wooden flooring (wherever applicable), carpets, acoustic works, wall paneling, false ceiling, blinds, white boards, tack boards, skylights, glazing work, aluminum work, fabrication work, work stations, office tables, customized conference tables, signage & wayfinding, complete reception arrangements, supply and installation of double sided steel book racks, ergonomic reading chair, reading tables, research cubicles, lounge furniture with comfortable seating, heavy duty book trolley / cart, Magazine display racks, Newspaper stands, library grade step-stools, digital library workstations (wire-managed), for Nalanda University Library Building and any other fixed furniture/interior items as per requirement. |
| Estimated Cost Put to Tender | Rs. 31,04,23,816/- |
| Earnest Money | Rs. 41,04,238/- (RTGS/NEFT or 50% RTGS/NEFT and 50% BG) |
| E-tender processing Fee | Rs. 10,000/- plus GST @ 18% |
| Period of Work Completion | 6 months (1 month for planning & designing and obtaining approvals for commencement of work + 5 months for execution of original work including all documentations complete |
| Pre-bid Conference | Pre-bid conference shall be held with the intending bidders in the Conference Hall, Project Office of Nalanda University at Rajgir at 03:30 PM on 22.12.2025. Only prospective bidders’ EPC Contractors’ authorized representative is allowed to attend. |
| Last date & time of online submission of technical and financial bids | Up to 03:00 PM on 31.12.2025 through online in https://nalandauniv.ewizard.in/. The tender documents can be viewed and downloaded from Nalanda University website https://nalandauniv.edu.in/tenders/ also. |
| Time and date of opening of Technical Bids | 03:30 PM on 31.12.2025 |

###### ELIGIBILITY CRITERIA

Contractors who fulfill the following requirements shall be eligible to apply. Joint ventures/Consortium and special purpose vehicles are not accepted. For this purpose, cost of work shall mean gross value of the completed work escalated @ 7% per annum from the date of completion of the work up to last date of submission of the tender including extensions, if any.

    - Should have satisfactorily completed the works as mentioned below during the last Seven years ending previous day of last date of submission of bids. Three similar works (40%) each costing not less than Rs. 12.42 Crore.

OR

Two similar works (60%) each costing not less than Rs. 18.62 Crore.

OR

One similar work costing (80%) not less than Rs. 24.83 Crore.

For the purpose of this clause, “similar work” shall mean “Interior and furnishing works for buildings like Public Libraries/ Research & Development Centers / IITs/ IIMs/ Central Universities / Deemed Universities / Colleges. Interior works should cover not less than 15,000 sqm of built-up area in a single campus under one agreement covering works in fixed partition with wood, glass, aluminum or uPVC work, false ceiling works, flooring, carpets, acoustic works, wall paneling, glazing, blinds, white boards, tack boards, skylights, glazing work, aluminum work, fabrication works, customized work stations, office tables, customized conference tables, signage & wayfinding, complete reception arrangements etc. in last seven years in India.”

Note:-

- The value of executed works shall be brought to current costing level by enhancing the actual value of work at simple rate of 7% per annum; calculated from the date of completion to previous day of last date of submission of tenders. Qualified similar works may be physically inspected, if required, by a Technical Expert Committee constituted by the Competent Authority at Nalanda University to ascertain the completion, performance on quality of works for finalizing the technical bid and their report will be final and binding.

- In case of works executed in Private Sector, Completion Certificate should be accompanied with TDS certificates.

    - Should have minimum average annual financial turn over (gross) of Rs.

15.52 Crore on interior works during the last available three consecutive financial year ending March, 2025. Year in which no turnover is shown would also be considered for working out the average. (The bidder should upload financial information about turnover as per Form – ‘A’ only and should not upload any other financial sheets like balance sheets etc.)

    - The multiplication factor of 7% per annum simple interest is not applicable on the Annual Financial Turnover figures.

    - Should not have incurred any loss (profit after tax should be positive) in more than two years during the last five consecutive years ending 31st March 2025. The contractor should upload financial information about profit/loss only and should not upload any other financial sheets like Balance Sheets etc.

    - Should have a minimum solvency of Rs. 12.42 Crore (Scanned copy of original solvency certificate to be uploaded). Such solvency certificate should not have been issued by the Bank beyond 6 months from the date of last submission of bids.

    - The eligibility bid shall be opened first on due date and time as mentioned above. The time and date for opening of financial of contractors qualifying the eligibility bid shall be communicated to them at later date.

    - Online financial bid document submitted by the bidders shall be opened only of those bidders who on the basis of pre -qualification documents uploaded by them within the period of bid submission, qualify in accordance with the provision of eligibility bid. The financial bid shall be opened online at the notified time and date in presence of qualified bidders or their representative.

    - Agreement shall be drawn with the successful bidders on prescribed Form GCC-2024 for EPC contracts in CPWD which is available as a Govt. of India Publication and also available on website www.cpwd.gov.in. Bidders shall quote his rates as per various terms and conditions of the said form which will form part of the agreement. A copy of the document is also enclosed with this NIT.

    - The site for the work is available. The time allowed for carrying out the

work will be 06 (six) Months (1 month for finalization of design and approval of competent authority and 5 months for execution of original work, from the date of start as defined in tender or from the first date of handing over of the site, whichever is later, in accordance with the phasing, if any, indicated in the bid documents.

    - The bidder should have sufficient number of Technical and Administrative Employees for the proper execution of the contract. The bidder shall have to submit a list of these employees stating clearly how these would be involved in this work within 15 days of award of work.

    - The contractor will take all necessary guards and precautions during the course of execution to ensure safety and stability of the adjoining building structures. All legal consequences of any danger/threat to the safety and stability of adjoining structures rest with the contractor and department stands absolved from all legal liabilities and consequences thereof if any, initiated by the owner/contractor of these building structures.

    - After submission of the bid, the contractor can resubmit revised bid any number of times but before last time and date of submission of bid as notified. Contractor must ensure to quote his rate in the attached schedules online only. The bids submitted online shall only be considered for evaluation.

    - Online bid documents submitted by intending bidders shall be opened only of those bidders, whose EMD submission details uploaded to the e-Tendering website and other documents scanned and uploaded are found in order.

    - The bid submitted shall become invalid, if:

      - The bidder is found ineligible.

      - The bidder does not deposit EMD.

      - The bidder does not upload all the documents (including GST Registration) as stipulated in the bid document including the copy of proof of EMD submission.

      - If a tenderer quotes nil rates against any item of work of tender, the tender shall be treated as invalid and will not be considered as lowest tenderer.

    - The contractor whose bid is accepted will be required to furnish performance guarantee of 5% (Five Percent) of the bid amount within the period specified in Schedule F in the form of Demand Draft of any scheduled bank/ Pay order of any Scheduled Bank /Fixed Deposit Receipts or Guarantee Bonds of any Scheduled Bank in accordance with the prescribed form. In case the contractor fails to deposit the said performance guarantee within the period as indicated in tender, including the extended period if any, the contractor will be debarred from tendering in Nalanda University as per the undertaking submitted by the contractor.

    - The contractor whose bid is accepted will also be required to furnish either copy of applicable licenses/ registrations or proof of applying for obtaining labour licenses, registration with EPFO, ESIC and BOCW Welfare Board including Provident Fund Code No. If applicable and also ensure the compliance of aforesaid provisions by the sub- contractors, if any engaged by the contractor for the said work within the period specified in tender.

    - The intending bidder must read the terms and conditions of CPWD-6 carefully. He should only submit his bid if he considers himself eligible and he is in possession of all the documents required.

    - Information and instructions for bidders posted on website shall form part of bid document.

    - The bid document consisting of plans, specifications, items to be executed and the set of terms & conditions of the contract to be complied with and other necessary documents can be seen from website www.nalandauniv.edu.in/tenders/ or https://nalandauniv.ewizard.in/

    - The bid can only be submitted after uploading the proof of submission of E- Tender Processing Fee (copy of proof of payment / bank transfer shall be submitted) and other documents as specified.

    - Those Bidders who are not registered in https://nalandauniv.ewizard.in website mentioned above, are required to get registered themselves beforehand. The intending bidder must have valid Class-III digital signature to submit the bid.

    - On opening date, the Bidder can login and see the bid opening process. After opening of bids, he will receive the competitor bid sheets.

    - Bidder can upload documents in the form of PDF format.

    - Bidder should ensure that the document uploaded is legible and full documents page is properly scanned.

    - Certificate of Financial Turnover: At the time of submission of bid, bidder may upload affidavit/certificate from CA mentioning Financial Turn Over of

last 5 years or for the period as specified in the bid document duly certified by the chartered accountant. There is no need to upload entire voluminous balance sheet.

    - The eligibility (Technical) bid shall be opened first on due date and time as mentioned above. The time and date of opening of financial bid of bidders qualifying the eligibility (Technical) bid shall be communicated to them at a later date.

    - Pre-bid conference shall be held with the intending bidders in the Conference Hall, Project Office of Nalanda University at Rajgir, District Nalanda, Bihar – 803116 at 03:30 PM on 22.12.2025. After pre-bid conference, modifications if required in the bidding documents and clarifications to the queries raised by intending bidders will be issued to all participating bidders by the University by e-mail. Same will also be uploaded to the website.

    - All modifications/addendums/corrigendum issued regarding this bidding process, shall be uploaded on website only and shall not be published in any Newspaper. Therefore, prospective bidders must see the website regularly for any update.

    - The University reserves the right to reject any prospective application without assigning any reason thereof and to restrict the list of qualified bidders to any number deemed suitable by it, if too many bids are received satisfying the minimum laid down criteria.

    - The eligible bidders shall quote rate per square meter only, which shall include all components of the work as per the financial bid format.

    - If any information furnished by the applicant is found incorrect at a later stage, he shall be liable to be debarred from tendering/taking up of works in the University. The University reserves the right to verify the particulars furnished by the applicant independently.

    - Intending Bidders are advised to inspect and examine the site and its

surroundings and satisfy themselves before submitting their bids as to the nature of the ground and sub-soil (so far as is practicable), the form and nature of the site, the means of access to the site, the accommodation they may require and in general shall themselves obtain all necessary information as to risks, contingencies and other circumstances which may influence or affect their bid. A bidders shall be deemed to have full knowledge of the site whether he inspects it or not and no extra charge consequent on any misunderstanding or otherwise shall be allowed. The bidders shall be responsible for arranging and maintaining at his own cost all materials, tools & plants, water, electricity access, facilities for workers and all other services required for executing the work unless otherwise specifically provided for in the contract documents. Submission of a bid by a bidders implies that he has read this notice and all other contract documents and has made himself aware of the scope and specifications of the work to be done and of conditions and rates at which stores, tools and plant, etc. will be issued to him by the Government and local conditions and other factors having a bearing on the execution of the work.

    - After submission of the bid the agency can re-submit revised bid any number of times but before last time and date of submission of bid as notified.

    - While submitting the revised bid, agency can revise the rate of one or more item(s) any number of times (he need not re-enter rate of all the items) but before last time and date of submission of bid as notified.

    - Any dispute arising out of this tender including dispute related to encashment of any Bank Guarantee/ FDR etc., shall be subject to the jurisdiction of courts of Patna High Court only.

    - Details of the Bank for submission of Performance Guarantee:

| Name of A/c holder | : | NALANDA UNIVERSITY |
| --- | --- | --- |
| Name & Address of HDFC Bank Branch |  | DHARAMSHALA ROAD, DIST NALANDA, City: RAJGIR 803116, State: BIHAR |
| Type of Account | : | Current Account |
| Branch Code | : | 2059 |
| IFSC Code | : | HDFC0002059 |

    - List of documents to be filled in by the bidders in various forms as indicated in Section 3, to be scanned and uploaded within the period of bid submission –

| Sl. No. | Details of Documents to be submitted |
| --- | --- |
| 1. | Letter of transmittal |
| 2. | Copy of receipt for deposition of original E-Tender Processing Fee and EMD (through NEFT / RTGS only) |
| 3. | List of eligible similar nature of works completed during the last seven years from date of completion to previous day of last date of submission of bid in Form -C. (If private works are shown in support of eligibility, certified copy of the tax deducted at source certificate (TDS) shall be submitted along with the experience certificate and the TDS amount shall tally with the actual amount of work done). |
| 4. | Certificate of Financial Turnover from CA in Form –A. Bank Solvency Certificate in Form-B. |
| 5. | Performance report of works (mentioned in Form-C) in Form-D. Structure and Organization of the bidder in Form-E. |  |
| 6. | Permanent Account Number (PAN) as issued by the Income Tax Department and GST Number along with Certification |  |
| 7. | Signed copy of Integrity Agreement. |  |
| 8. | Undertaking of Structural/ Services Design as per proforma in para 1.3 below. |  |

###### CERTIFICATE REGARDING STRUCTURAL DESIGN

The following certificates are to be submitted along with the technical bid of the tender: (This affidavit/certificate is to be submitted in non-judicial stamp paper of appropriate value and duly notarized)

It is certified that the structural design and drawings with respect to the works under the scope of this tender shall be prepared in conformance with the concept plan as published by the University along with this tender, including safety norms from natural hazards like seismic, wind, fire etc., coordinated with all existing wired & piped services drawings (executed by other agencies), by the associate agency(s) conforming to eligibility criteria as defined in the NIT having duly qualified Architect and the Structural Engineer respectively for specialized component (s) including GRIHA Consultant /Facilitator as per norms prescribed in N.B.C/B.I.S/I.R.C./ GRIHA etc.

Accordingly, I/ we shall get the buildings designed from qualified Architect, Structural Engineer, Integrated Services Consultant and GRIHA Consultant

/Facilitator.

It is further certified that the structural engineer and other consultants hired by us for carrying out the Interior Works design along with structural design shall have following experience:

BE/B. Tech. from Any reputed Government Engineering College like IIT/NIT/Govt. Engineering College.

OR

Under the panel list of CDO/CPWD (mention as the case may be).

AND

The structural consultant will have at least degree of M. Tech (Structure) &

M. Tech (E&M service) or equivalent and has got 15 years or more experience in the field of structural design and services design of similar nature of works. Documents in support of the above will be submitted at appropriate time for approval of the department.

OR

Should have the design experience by serving & designing in CPWD for at least 15 years & should have been retired at least from SE level in CPWD.

The structural designer firm of above should have an experience of structural design of completed non-residential or residential building of minimum 5 storied building of minimum plinth area as below:

3 RCC framed structure of having 5000 sqm build up area each.

OR

2 RCC framed structure of having 8000 sqm build up area each.

OR

1 RCC framed structure of having 10000 sqm build up area each.

The GRIHA Consultant and facilitator hired by bidder shall be well qualified having minimum experience of 07 years in GRIHA consultancy and shall be on the approved list of the GRIHA authorities.

Signature of the contractor with date Name in Blocks letters ------------------

Address:

### INTRODUCTION TO PROJECT –

- Nalanda University was established in November 2010. It came into being by an Act of the Indian Parliament – a testimony to the important status that Nalanda University occupies in the Indian intellectual landscape. Nalanda University is a standalone international university unlike any other established in the country. Located in the town of Rajgir, in the northern Indian State of Bihar, Nalanda University is mandated to be “an international institution for the pursuit of intellectual, philosophical, historical and spiritual studies”. This new university contains within it a memory of the ancient Nalanda University and is premised on the shared desire of Member States of the East Asia Summit countries to re-discover and strengthen “educational co-operation by tapping the East Asia Regions centres of excellence in education ... [and] to improve understanding and the appreciation of one another’s heritage and history”

The proposed site for the University is located at Rajgir in the Nalanda district of Bihar and fronts the State Highway 71. The site for the University is on the Southwest outskirts of Rajgir town at a distance of 3.5 kilometres from the current urban edge of the town.

- The present tender is for Engineering, Procurement and Construction (EPC) of Interior Works for the Nalanda Library building at the Permanent Campus of Nalanda University at Rajgir, Bihar including wooden partitions, facades, cabinets for FCUs, service bands for electrical, HVAC, CCTV, Data, DALI etc., fixed cupboards, storages (normal storage & compactors etc. in all offices), wooden flooring (wherever applicable), carpets, acoustic works, wall paneling, false ceiling, blinds, white boards, tack boards, skylights, glazing work, aluminum work, fabrication work, work stations, Office tables, customized conference tables, signage, complete reception arrangements for the Library building and other fixed and movable furniture/interior items as per requirement stated in the schedule of quantities.

- Being an EPC tender, the bidder is advised to work out the detailing as per requirement given in tender documents and considering the same along with the relevant IS codes, NBC-2016, Latest CPWD specification and latest technology etc. Indicative Items of various works covered under scope of work in this tender are given below for reference purpose only. The bidders, before quoting the tender are deemed to have ascertained/verified/worked out all the items & quantities etc. from site inspection for complete scope of work. No claim whatsoever on account of any discrepancies, changes in nomenclature, variation, addition, alteration, modification, left over items etc. shall be entertained by the University. The contractor is required to execute all the works/items and quantities as per Scope of work based on Site Execution, Drawings & whatever is essential for successful completion of the project and the bidder’s quoted rates are deemed to include everything as mentioned and including testing, commissioning & handing over of the completed project, defect liability period and Operation and Maintenance period (if applicable) as per satisfaction of the Owner. In case of contravention of stipulation of this clause from any other clause/provision elsewhere in the tender document, then this clause shall have superseding effect to the extent of contravention and decision of Engineer-in-charge shall be final & binding on the EPC Contractor.

- The Bidder has to take care (i.e., not to damage) the existing flora and fauna in the campus if any, during execution of the works as per this E-Tender.

- The scope of work, within quoted cost, also includes the following apart from the mentioned details elsewhere in Financial Bid:

The drawings/layout /working details etc. for the Interior works shall be prepared by the bidder through a reputed Interior Design Consultant, and approval shall be obtained from the University before execution.

- The movement plan of machineries shall first get approved from the University before execution.

- Bidder has to arrange temporary electric connection either from state electricity board or through own DG set as applicable situation, for the execution of the Interior works. No extra claim for the same shall be entertained.

- The Concept design, drawings and documents attached in the tender are for adhering to minimum requirement of project reference which is indicative and not exhaustive. Bidder has to work out with reference to details provided in scope, and drawings in the tender document for quoting their rates. No claim whatsoever will be entertained at a later stage in this regard.

###### BROAD MILESTONES UNDER SCOPE OF WORK

- The EPC Contractor shall prepare of all designs/drawings, Design Basis Report (DBR) conforming to the relevant building bye laws/statutory provisions.

- The contractor shall give the presentation of Interior design to Nalanda University for approval from competent authority.

- Upon obtaining approval of the concept design the EPC Contractor shall carry out structural design wherever applicable and obtain approval and based on the approved designs prepare the working/GFC Drawings for execution of work.

- All the technical component drawings shall be vetted by the Consultants appointed by University or any IIT/NIT/Govt. organization after acceptance from the University.

- The contractor will submit the schedule containing the item, rates (i.e. DSR reference, if available), and quantities of items, detailed measurements & specifications (four sets) within three months from the date of start of the work.

- Project Management of the work including day to day supervision during execution.

- The specialized works shall be done by reputed agencies who has carried out similar type of work with high standards and having statutory registrations.

- The EPC Contractor may recommend any innovative, sustainable, environmental friendly, easy for installation substitute item having minimum same standards as per relevant code mentioned in tender document.

- The EPC Contractor shall execute the project as per the provisions of CPWD Manual 2019 including subsequent amendments and under the ambit of GFR 2017 including subsequent amendments.

- The EPC Contractor shall ensure that the quality of work is executed as per specified specifications and norms and as laid down in latest CPWD specifications including correction slips, if any, BIS Code and other specifications but within the guidelines as laid down by CVC. The routine testing in respect of materials shall be carried out in recognized laboratory/ institution concerned as per CPWD norms in force.

- The Contractor shall complete the project as per the broad specifications which are given in the Preliminary Estimates.

- Intimating the detailed physical and financial progress regularly at the end of each month.

- The contractor shall hand over the completed work including the As Built Drawings in soft as well as hard copies, a detailed maintenance manual, warranties if any and all other relevant documents. The Contractor shall provide three sets of completion drawings as built to the University at the time of handing over of completed works. The Contractor also handover the copy of guarantee bonds of the specialized items as per CPWD works manual.

- Submission of Completion Certificate duly issued by the Project Engineer and the Architect that the work has been executed as per the approved architectural/ structural drawings and as per terms of the contract along with the final bill.

- The Contractor shall indemnify and keep indemnified Nalanda University against all losses and claims for injuries or damages to any person or any property whatsoever which may arise out of or in consequence of the construction against all claims, demands, proceedings, damages, costs, charges and expenses whatsoever in respect of or in relation thereto.

- On completion of the work, the Contractor shall clear away and remove from the site all constructional plants and machines surplus materials, rubbish and temporary works of every kind and leave the whole of the site and works, to the satisfaction of Nalanda University.

Note: The EPC Contractor shall handover all warranty & original certificates, servicing manuals, company routine maintenance etc. for future operation & maintenance well within time limit under the Contractor’s Scope of Work.

### GUIDELINES / PROCEDURE TO BE FOLLOWED IN INTRODUCTION OF ‘E’- PROCUREMENT SOLUTION

    - The	bidders	can	view	/	download	the	tender	documents,	from	the	 www.nalandauniv.edu.in/tenders or https://nalandauniv.ewizard.in/ or https://eprocure.gov.in/epublish/app.

    - Submission of Bids: The bidders who are desirous of participating in this e- Tender shall submit their price bids in the standard formats prescribed in the Tender		documents,	displayed		at	www.nalandauniv.edu.in/tenders		or https://nalandauniv.ewizard.in/ The bidder should upload the scanned copies	of	all	the		relevant	certificates,	documents	etc.,	in	the https://nalandauniv.ewizard.in/ in support of their bids. The bidder shall sign on all the statements, documents, certificates, uploaded by him, owning responsibility for their correctness / authenticity.

    - Payment of Bid Security (Earnest Money Deposit):- Rs 41,04,238/- to be submitted through NEFT or RTGS in favour of Nalanda University to the following bank account –

A/c Name : Nalanda University Bank	: State Bank of India Account No. : 37646810882 Branch		: Rajgir Branch IFSC CODE : SBIN0003499

Alternatively, the bidders can submit the EMD in a combination of NEFT/RTGS and Bank Guarantee. In such case at least 50% of the EMD to be submitted through NEFT/RTGS in the above mentioned bank account and the rest through Bank Guarantee issued by a Scheduled Bank in favour of Nalanda University, Rajgir, Bihar.

    - Processing of Tenders:- The concerned Committee as appointed by the Competent Authority at Nalanda University will evaluate and process the tenders as done in the conventional tenders and the decisions will be communicated to the bidder online. Any correspondences or queries related to this tender shall be submitted to the University in the email address – phase1.tender@nalandauniv.edu.in only.

    - Price Bid opening:-The Price bid will be opened online by the Committee at the specified date and time and the result will be displayed on the website www.nalandauniv.edu.in/tenders and https://nalandauniv.ewizard.in/ , which can be seen by all the bidders who participated in the tender.

    - Payment of performance Guarantee: -The bidder shall submit irrevocable performance guarantee of 3% (three percent) of the tendered amount in addition to other deposits mentioned elsewhere in the contract for his proper performance of the contract. This guarantee shall be in the form of Banker Cheque / pay order / FDR / guarantee bonds of any scheduled bank in favour of Nalanda University, payable at Rajgir. 100% of the performance guarantee shall be released after completion of work .However, the Performance Bank Guarantee will be kept valid time to time by the bidder as per the direction of Engineer-in-charge.

    - Participation of Bidders at the time of opening of bids - Bidders have two options to participate in tendering process at the time of opening of Bids as stated below -

    - Bidders can come at the place of opening of bids (electronically) by the Tender Opening Committee as done in the conventional tender process.

    - Bidders can attend the process online.

    - Signing of agreement: -After the award of the contract, an agreement will be signed as done in Conventional Tenders.

Sd/- Registrar

Nalanda University

### NOTICE INVITING E-TENDER (CPWD-6)

The Registrar, Nalanda University, Rajgir, District – Nalanda, Bihar on behalf of Nalanda University invites online composite bids on Engineering Procurement Construction (EPC) basis from reputed, approved and eligible composite category contractors of CPWD or any State / Central Govt. Departments / PSUs / Institutions / Autonomous Bodies / other reputed firms fulfilling the set eligibility criteria, in two bid system (Eligibility Bid & Financial bid) for following work:

- Name of Work: Engineering, Procurement and Construction (EPC) of Interior Works for the Nalanda Library Building at the Main Campus of Nalanda University at Rajgir, Bihar including wooden partitions, cabinets for FCUs, service bands for electrical, HVAC, CCTV, Data, DALI etc., fixed cupboards, storages (normal storage & compactors etc.), wooden flooring (wherever applicable), carpets, acoustic works, wall paneling, false ceiling, blinds, white boards, tack boards, skylights, glazing work, aluminum work, fabrication work, work stations, office tables, customized conference tables, signage & wayfinding, complete reception arrangements, supply and installation of double sided steel book racks, ergonomic reading chair, reading tables, research cubicles, lounge furniture with comfortable seating, heavy duty book trolley / cart, Magazine display racks, Newspaper stands, library grade step-stools, digital library workstations (wire-managed), for Nalanda University Library Building and any other fixed furniture/interior items as per requirement.

- The estimated cost of Work is Rs. 31,04,23,816/- (Rupees Thirty-One  Crore Four Lakh Twenty-Three Thousand Eight Hundred Sixteen Only/-).

- The Intending Bidder shall be eligible to submit the bid provided he has definite proof from the appropriate authority, which shall be to the satisfaction of the Competent Authority at Nalanda University, of having satisfactorily completed similar works of magnitude as specified below –

Three similar works (40%) each costing not less than Rs. 12.42 Crore. OR

Two similar works (60%) each costing not less than Rs. 18.62 Crore. OR

One similar work costing (80%) not less than Rs. 24.83 Crore.

For the purpose of this clause, “similar work” shall mean “Interior and furnishing works for buildings like Public Libraries/ Research & Development Centers / IITs/ IIMs/ Central Universities / Deemed Universities / Colleges. Interior works should cover not less than 15,000 sqm of built-up area in a single campus under one agreement covering works in fixed partition with wood, glass, aluminum or uPVC work, false ceiling works, flooring, carpets, acoustic works, wall paneling, glazing, blinds, white boards, tack boards, skylights, glazing work, aluminum work, fabrication works, customized work stations, office tables, customized conference tables, signage & wayfinding, complete reception arrangements etc. in last seven years in India.”

Note:-

- The value of executed works shall be brought to current costing level by enhancing the actual value of work at simple rate of 7% per annum; calculated from the date of completion to previous day of last date of submission of tenders. Qualified similar works may be physically inspected, if required, by a Technical Expert Committee constituted by the Competent Authority at Nalanda University to ascertain the completion, performance on quality of works for finalizing the technical bid and their report will be final and binding.

- In case of works executed in Private Sector, Completion Certificate should be accompanied with TDS certificates.

- Should have minimum average annual financial turn over (gross) of Rs.

15.52 Crore on interior works during the last available three consecutive financial year ending March, 2025. Year in which no turnover is shown would also be considered for working out the average. (The bidder should upload financial information about turnover as per Form – ‘A’ only and should not upload any other financial sheets like balance sheets etc.)

- The multiplication factor of 7% per annum simple interest is not applicable on the Annual Financial Turnover figures.

- Should not have incurred any loss (profit after tax should be positive) in more than two years during the last five consecutive years ending 31st March 2025. The contractor should upload financial information about profit/loss only and should not upload any other financial sheets like Balance Sheets etc.

- Should have a minimum solvency of Rs. 12.42 Crore (Scanned copy of original solvency certificate to be uploaded). Such solvency certificate should not have been issued by the Bank beyond 6 months from the date of last submission of bids.

- The eligibility bid shall be opened first on due date and time as mentioned above. The time and date for opening of financial of contractors qualifying the eligibility bid shall be communicated to them at later date.

- Online financial bid document submitted by the bidders shall be opened only of those bidders who on the basis of pre -qualification documents uploaded by them within the period of bid submission, qualify in accordance with the provision of eligibility bid. The financial bid shall be opened online at the notified time and date in presence of qualified bidders or their representative.

- Agreement shall be drawn with the successful bidders on prescribed Form GCC-2024 for EPC contracts in CPWD which is available as a Govt. of India Publication and also available on website www.cpwd.gov.in. Bidders shall quote his rates as per various terms and conditions of the said form which will form part of the agreement. A copy of the document is also enclosed with this NIT.

- The site for the work is available. The time allowed for carrying out the

- work will be 06 (six) Months (1 month for finalization of design and approval of competent authority and 5 months for execution of original work, from the date of start as defined in tender or from the first date of handing over of the site, whichever is later, in accordance with the phasing, if any, indicated in the bid documents.

- The bidder should have sufficient number of Technical and Administrative Employees for the proper execution of the contract. The bidder shall have to submit a list of these employees stating clearly how these would be involved in this work within 15 days of award of work.

- The contractor will take all necessary guards and precautions during the course of execution to ensure safety and stability of the adjoining building structures. All legal consequences of any danger/threat to the safety and stability of adjoining structures rest with the contractor and department stands absolved from all legal liabilities and consequences thereof if any, initiated by the owner/contractor of these building structures.

- After submission of the bid, the contractor can resubmit revised bid any number of times but before last time and date of submission of bid as notified. Contractor must ensure to quote his rate in the attached schedules online only. The bids submitted online shall only be considered for evaluation.

- Online bid documents submitted by intending bidders shall be opened only of those bidders, whose EMD submission details uploaded to the e-Tendering website and other documents scanned and uploaded are found in order.

- The bid submitted shall become invalid, if:

  - The bidder is found ineligible.

  - The bidder does not deposit EMD.

  - The bidder does not upload all the documents (including GST Registration) as stipulated in the bid document including the copy of proof of EMD submission.

  - If a tenderer quotes nil rates against any item of work of tender, the tender shall be treated as invalid and will not be considered as lowest tenderer.

- The contractor whose bid is accepted will be required to furnish performance guarantee of 5% (Five Percent) of the bid amount within the period specified in Schedule F in the form of Demand Draft of any scheduled bank/ Pay order of any Scheduled Bank /Fixed Deposit Receipts or Guarantee Bonds of any Scheduled Bank in accordance with the prescribed form. In case the contractor fails to deposit the said performance guarantee within the period as indicated in tender, including the extended period if any, the contractor will be debarred from tendering in Nalanda University as per the undertaking submitted by the contractor.

- The contractor whose bid is accepted will also be required to furnish either copy of applicable licenses/ registrations or proof of applying for obtaining labour licenses, registration with EPFO, ESIC and BOCW Welfare Board including Provident Fund Code No. If applicable and also ensure the compliance of aforesaid provisions by the sub- contractors, if any engaged by the contractor for the said work within the period specified in tender.

- The intending bidder must read the terms and conditions of CPWD-6 carefully. He should only submit his bid if he considers himself eligible and he is in possession of all the documents required.

- Information and instructions for bidders posted on website shall form part of bid document.

- The bid document consisting of plans, specifications, items to be executed and the set of terms & conditions of the contract to be complied with and other necessary documents can be seen from website www.nalandauniv.edu.in/tenders/ or https://nalandauniv.ewizard.in/

- The bid can only be submitted after uploading the proof of submission of E- Tender Processing Fee (copy of proof of payment / bank transfer shall be submitted) and other documents as specified.

- Those Bidders who are not registered in https://nalandauniv.ewizard.in website mentioned above, are required to get registered themselves beforehand. The intending bidder must have valid Class-III digital signature to submit the bid.

- On opening date, the Bidder can login and see the bid opening process. After opening of bids, he will receive the competitor bid sheets.

- Bidder can upload documents in the form of PDF format.

- Bidder should ensure that the document uploaded is legible and full documents page is properly scanned.

- Certificate of Financial Turnover: At the time of submission of bid, bidder may upload affidavit/certificate from CA mentioning Financial Turn Over of

- last 5 years or for the period as specified in the bid document duly certified by the chartered accountant. There is no need to upload entire voluminous balance sheet.

- The eligibility (Technical) bid shall be opened first on due date and time as mentioned above. The time and date of opening of financial bid of bidders qualifying the eligibility (Technical) bid shall be communicated to them at a later date.

- Pre-bid conference shall be held with the intending bidders in the Conference Hall, Project Office of Nalanda University at Rajgir, District Nalanda, Bihar – 803116 at 03:30 PM on 22.12.2025. After pre-bid conference, modifications if required in the bidding documents and clarifications to the queries raised by intending bidders will be issued to all participating bidders by the University by e-mail. Same will also be uploaded to the website.

- All modifications/addendums/corrigendum issued regarding this bidding process, shall be uploaded on website only and shall not be published in any Newspaper. Therefore, prospective bidders must see the website regularly for any update.

- The University reserves the right to reject any prospective application without assigning any reason thereof and to restrict the list of qualified bidders to any number deemed suitable by it, if too many bids are received satisfying the minimum laid down criteria.

- The eligible bidders shall quote rate per square meter only, which shall include all components of the work as per the financial bid format.

- If any information furnished by the applicant is found incorrect at a later stage, he shall be liable to be debarred from tendering/taking up of works in the University. The University reserves the right to verify the particulars furnished by the applicant independently.

- Intending Bidders are advised to inspect and examine the site and its surroundings and satisfy themselves before submitting their bids as to the nature of the ground and sub-soil (so far as is practicable), the form and nature of the site, the means of access to the site, the accommodation they may require and in general shall themselves obtain all necessary information as to risks, contingencies and other circumstances which may influence or affect their bid. A bidders shall be deemed to have full knowledge of the site whether he inspects it or not and no extra charge consequent on any misunderstanding or otherwise shall be allowed. The bidders shall be responsible for arranging and maintaining at his own cost all materials, tools & plants, water, electricity access, facilities for workers and all other services required for executing the work unless otherwise specifically provided for in the contract documents. Submission of a bid by a bidders implies that he has read this notice and all other contract documents and has made himself aware of the scope and specifications of the work to be done and of conditions and rates at which stores, tools and plant, etc. will be issued to him by the Government and local conditions and other factors having a bearing on the execution of the work.

- After submission of the bid the agency can re-submit revised bid any number of times but before last time and date of submission of bid as notified.

- While submitting the revised bid, agency can revise the rate of one or more item(s) any number of times (he need not re-enter rate of all the items) but before last time and date of submission of bid as notified.

- Any dispute arising out of this tender including dispute related to encashment of any Bank Guarantee/ FDR etc., shall be subject to the jurisdiction of courts of Patna High Court only. Bidder shall submit the Bid Security Declaration as per FORM I. The intending bidder has to scan and upload the signed and notarized document to the e-Tendering website within the period of tender submission and original should be deposited at project office of Nalanda University.

Bank Details of Nalanda University is as follows:

  - Name of Bank: HDFC BANK LTD

  - Name	&	Address	of	Branch:	DHARAMSHALA	ROAD,	DIST NALANDA, BIHAR, PIN – 803116

  - Branch Code :2059

  - IFSC Code of respective branch: HDFC0002059

- Interested bidders who wish to participate in the tender shall pay Rs. 10,000/ (Rupees Fifteen Thousand only) plus GST @ 18% as e-Tender Processing Fee of Nalanda University, Rajgir, Bihar through NEFT or RTGS to the following bank account. A copy of the bank transfer / online payment shall have to be uploaded to the e-Tendering website before tender submission.

- The tender shall be accompanied with the following documents:

- Proof of EMD and e-Tender Processing Fee shall be uploaded in the e-Tendering portal with name of Work and due date of opening of the tender also mentioned thereon.

- Copies of certificates of work experience and other prequalifying documents as specified in the NIT shall be uploaded in the e-procurement as a single pdf file under "Eligibility Documents".

- All the tender documents (Notice Inviting Tender, Schedule of Quantities, BOQ, Technical Specifications and Tender drawings) uploaded by the University shall be duly signed on each page by the authorized signatory of the bidder, stamped and the scanned copies of which shall be uploaded in the e-procurement portal and labeled as “Signed Tender Documents”.

- Price Bid shall be uploaded in the e-procurement portal (https://nalandauniv.ewizard.in/) directly as per proforma given therein.

- Technical tender documents submitted by intending bidders shall be opened only of those bidders, whose EMD and e- Tender Processing Fee are found in order.

- The tender submitted shall become invalid if:

- The bidder does not deposit EMD and Tender Processing Fee as prescribed above

- The bidders are found ineligible;

- The bidders fail to upload all the documents (Including GST registration as stipulated in this tender document;

- The bidder fails to provide presentation and walkthrough of the conceptual interior design during technical evaluation stage or the design is found to be unsatisfactory.

- If a tenderer quotes nil rates against each item in item rate tender or does not quote any percentage above/below on the total amount of the tender of any section/sub head in percentage rate tender, the tender shall be treated as invalid and will not be considered as lowest bidder.

- The contractor whose bid is accepted will be required to furnish performance guarantee of 5% (Five Percent) of the tender amount within the period as specified in Schedule F. This guarantee shall be in the form of Deposit at Call receipt of any scheduled bank/Banker's cheque of any scheduled bank/Demand Draft of any scheduled bank or Government Securities or Fixed Deposit Receipts or Guarantee Bonds of any Scheduled Bank or the State Bank of India in accordance with the prescribed form. In case the contractor fails to deposit the said performance guarantee within the period as indicated in Schedule 'F', including the extended period if any, suitable penal action shall be taken as per Bid Security Declaration form.

- The contractor whose tender is accepted will also be required to furnish either copy of applicable licenses/registrations or proof of applying for EPFO, ESIC and BOCW Welfare Board including Provident Fund Code No. if applicable and also ensure the compliance of aforesaid provisions by the sub-contractors, if any engaged by the contractor for the said work and Programme Chart (Time and Progress) within the period specified in Schedule F.

- Intending bidders must inspect and examine the Site and its surroundings and satisfy themselves before submitting their tenders as to the nature of the ground and sub-soil (so far as is practicable), form and nature of the Site, the means of access to the Site, the accommodation as may be required and in general, shall obtain all necessary information as to the risks involved, contingencies and other circumstances which may have a bearing or influence or affect the decision of submission of tender. A bidder shall be deemed to have full knowledge of the Site whether or not an inspection has been undertaken by the bidder or not and no extra charge consequent on any misunderstanding or otherwise shall be allowed in the amount quoted in the tender. The bidder shall be responsible for arranging and maintaining cost of all materials, tools & plants, water, electricity, access, facilities for workers and all other services required for executing the Work unless otherwise specifically provided for in this document and/or the agreement and/or any other letter, circular, notification issued, or document released by NU. Submission of a tender by a bidder implies that he has read this notice and all other contract documents and has made himself aware of the scope and specifications of the Work to be executed and of conditions and local conditions and other factors having a bearing on the execution of the Work. Bidder shall submit the Affidavit for Site Visit in the form as prescribed in FORM “G”. The bidder submitting bid shall be presumed that they are well aware of the site and accepting all the actual site conditions.

- The University is not bound to accept the lowest or any other tender and reserves to itself the right to reject any or all the tenders received without assigning any reason thereof. Owner also reserves the right to restrict the list of qualified contractors to any number deemed suitable by it, if too many tenders are received satisfying the laid down criteria. All tenders in which any of the prescribed conditions are not fulfilled or any condition including that of conditional rebate is put forth by the bidder shall be rejected summarily. Tenders subject to any conditions proposed by the bidder shall not be accepted and shall be liable to be rejected.

- Canvassing either directly or indirectly, in connection with tender is strictly prohibited and the tenders submitted by the bidders who resort to canvassing shall be liable to be rejected.

- Owner reserves the right of accepting the whole or any part of the tender and the bidders shall be bound to perform the same at the quoted rate.

- The bidder shall intimate the names of persons who are or were working in any capacity with the bidder or are subsequently employed by the bidder and who are or have been an officer in NU.

- The contractor shall not be permitted to tender for this work if his near relative is employed by Nalanda University in any capacity. He shall also intimate the names of persons who are working with him in any capacity or are subsequently employed by him and who are near relatives to any officerin Nalanda University or in the Ministry of External Affairs. Any breach of this condition by the contractor would render liable to be debarred from participating in tender process.

- No engineer of Gazetted rank or other Gazetted officer employed in Engineering or Administrative duties in an Engineering department of Government of India is allowed to work as a contractor for a period of one year after his retirement from Government service, without the previous permission of the Government of India in writing. This contract is liable to be cancelled if either the contractor or any of his employees is found to be such a person who had not obtained permission of the Government of India as aforesaid before submission of the tender or engagement in the contractor’s service.

- The tender for the Work shall remain open for acceptance for a period of 90 (Ninety) days from the date of opening of Technical tender. In case any bidder withdraws its tender before the end of Ninety days or issue of letter of acceptance, whichever is earlier, or makes such modifications in the terms and conditions of the tender which are not acceptable to the Owner, then the Owner shall, without prejudice to any other right or remedy available under the contract or law, be at liberty to take suitable actions as per Bid Security Declaration. Further, such bidder shall be debarred from participating in the re-tendering process of the Work.

- The receipt of the bid will be presumed that the bidder has accepted all the terms and condition of this NIT read with the corrigendum/addendum issued by the University. This notice-inviting Tender shall form a part of the agreement and shall be read and construed accordingly. The successful bidder, on acceptance of its tender by the Owner, shall within 15 days from the stipulated date of initiation of the Work, execute an agreement consisting of: -

- The Notice Inviting Tender, general conditions, all the documents including additional conditions, specifications, drawings and minutes of pre-tender meeting, addendums, and corrigendum, if any, forming the tender as issued at the time of invitation of tender and acceptance thereof together with any correspondence leading thereto.

Note: Integrity Agreement will be signed and sealed by the Authorized Representative of bidder as per the format of NIT on bidder's letter head initially. Formal agreement on stamp paper shall be signed between NU and the successful bidder only, at the time of Signing of agreement for the work.

Sd/- Registrar

## EPC TENDER & CONTRACT FOR WORK

Tender for the work of: - This tender is invited online for the “Name of work”

To be uploaded by the bidders in e-procurement portal of NU up to 03:00 PM on 31.12.2025.

To be opened at 03:30 PM on 31.12.2025 online.

###### Letter of Transmittal

I/We have read and examined the notice inviting tender, bill of quantity, Specifications, Drawings & Designs, General Rules and Directions, Conditions of Contract, clauses of contract, Special conditions& other documents and Rules referred to in the conditions of contract and all other contents in the tender document for the work.

I/We hereby tender to execute the work of NU mentioned above and detailed in the schedule of quantities within the time frame specified in Schedule F of the tender documents, and in accordance with the specifications, designs, drawing and instructions/orders of the engineer in charge meeting all the Conditions of Contract with such materials as are provided for In the drawings, technical specifications or the schedule of quantities.

I/We agree to keep the tender open and valid for 90 (Ninety) days from the due date of opening of Technical tender and shall not make any modifications in its terms and conditions.

We submit herewith the Bid Security Declaration as per FORM I. If I/We, fail to furnish the prescribed performance guarantee within prescribed period and form, I/We agree that NU shall without prejudice to any other right or remedy, be at liberty to take suitable action as per Bid Security Declaration. Further, if I/We fail to commence work as specified, I/We agree that NU shall without prejudice to any other right or remedy available in law, be at liberty to take suitable action as per Bid Security Declaration and forfeit the performance guarantee absolutely, otherwise the said Performance Guarantee shall be retained by Owner towards security deposit to execute all the works referred to in the tender documents upon the terms and conditions contained or referred to those in excess of that limit at the rates to be determined in accordance with the provision contained in Clause Deviations/Variations Extent and Pricing of the tender form. Further, I/We agree that in case of forfeiture Performance Guarantee as aforesaid, I/We shall be debarred from participating in the re-tendering process of the work.

I/We undertake and confirm that eligible similar work(s) has/have not been got executed through another contractor on back-to-back basis. Further that, if such a violation comes to the notice of university, then I/We shall be debarred for tendering in Nalanda University in future forever. Also, if such a violation comes to the notice of Nalanda University before date of start of work, the Registrar shall be free to forfeit the entire amount of Performance Guarantee.

I/We hereby declare that I/we shall treat the Tender Documents, drawings and other records connected with the Work as secret / confidential documents and shall not communicate nor use any information derived therefrom to any persons other than a person to whom I/we am/are authorized to communicate the same or use the information in any manner prejudicial to the interest of Nalanda University.

Dated	Signature of Contractor

Witness

Name ………………………………	Name ………………………………

Address ………………………………	Address ………………………………

Occupation ………………………………	Telephone ………………………………

###### ACCEPTANCE

The above offer is hereby accepted by me on behalf of The Nalanda University Rajgir, Bihar. For a sum of Rs. -------------------------- (Rupees	)

The letters referred to below shall from part of this agreement:

Dated	For & on behalf of NU

Signature

Form of Performance Security (Guarantee)Bank Guarantee Bond

Whereas the Registrar, Nalanda University, on behalf of Vice Chancellor, NU (hereinafter called “The Owner”) has entered into an agreement bearing number ………………. with……………….. (name and address of the contractor) (hereinafter called “the contractor”) for the execution of the work………………………………………. (name of the work)………………………………………….. The Owner has further agreed to accept an irrevocable Bank Guarantee for Rs…………………….(Rupees………………….only) valid up to……………..(date)……………… as Performance Guarantee/Security Deposit/ Mobilization Advance (Strikeout whichever is not applicable) from the said Contractor for compliance of his obligations in accordance with the terms and conditions of the agreement.

- We,………..(indicate the name of the bank)……………………(hereinafter referred to as “the Bank”), hereby undertake to pay to the Owner an amount not exceeding Rs…………………………. (Rupees……………………..only) on demand by the Owner within 10 days of the demand.

- We,……….….. (indicate the name of the bank)……………………, do here by undertake to pay the amount due and payable under this guarantee without any demur, merely on a demand from the Owner stating that the amount claimed is required to meet the recoveries due or likely to be due from the said Contractor. Any such demand made on the Bank shall be conclusive as regards the amount due and payable by the Bank under this Guarantee. However, our liability under this guarantee shall be restricted to an amount not exceeding Rs…………….. (Rupees…	only).

- We,…………… (indicate the name of the bank)…………………, further undertake to pay the Owner any money so demanded notwithstanding any dispute or disputes raised by the contractor in any suit or proceeding pending before any Court or Tribunal, our liability under this Bank guarantee being absolute and unequivocal. The payment so made by us under this Bank Guarantee shall be a valid discharge of our liability for payment there under and the Contractor shall have no claim against us for making such payment.

- We,………………. (indicate the name of the bank)……………………, further agree that the Owner shall have the fullest liberty without our consent and without affecting in any manner our obligation here under to vary any of the terms and conditions of the said agreement of to extend time of performance by the said Contractor from time to time or to postpone for any time of from time to time any of the powers exercisable by the Owner against the said contractor and to forbear or enforce any of the terms and conditions relating to the said agreement and we shall ne be relieved from our liability by reason of any such variation or extension being granted to the said Contractor of for any forbearance, cat of omission on the part of the Owner or any indulgence by the Owner to the said Contractor or by any such matter or thing whatsoever which under the lay relating to sureties would, but for this provision, have effect of so relieving us.

- We,……………………… (indicate the name of the bank)……………………, Further agree that the Owner at its option shall be entitled to enforce this Guarantee against the Bank as a principal debtor at the first instance without proceeding against the Contractor and notwithstanding any security or other guarantee the Owner may have in relation to the Contractor’s liabilities.

- This guarantee will not be discharged due to the change in the constitution of the Bank or the Contractor.

- We,…………. (indicate the name of the bank)……………………, undertake not to revoke this guarantee except with the consent of the Owner in writing.

- This Bank Guarantee shall be valid up to	unless

extender on demand by the Owner. Notwithstanding anything mentioned above, our liability against this guarantee is restricted to Rs……………………… (Rupees………………………………only) and unless a claim in writing is lodged with us within the date of expiry or extender date of expiry of this guarantee, all our liabilities under this guarantee shall stand discharged.

Date: ……………………………………

Witnesses:

  - Signature……………………………….. Name and Address

- Signature…………………………………… Name and Address

Authorized Signatory Name:

Designation: Staff Code No:

Bank Seal:

* Date to be worked out on the basis of validity period of 90 days where only financial bids are invited and 180 days for two/three bid system from the date of submission of the tender.

#### Brief Particulars of the Work

Summary of Scope of Work

This tender is invited for Engineering, Procurement and Construction (EPC) of Interior and furnishing works for the Nalanda Library Building of approximately 16,844 sqm of built up area at Nalanda University Main Campus at Rajgir, Bihar. The conceptual drawings, floor plans are enclosed separately for reference.

The project covers design, procurement, supply, installation, integration, testing, and commissioning of Library furniture and interiors.

The intent is to create a modern, fully automated, accessible, and sustainable library ecosystem supporting academic, research, cultural, and community functions.

- General Requirements

  - All items shall conform to BIS/ISO standards and applicable local building codes.

  - Furniture and materials shall be of ergonomic design, fire-retardant, eco- friendly, durable, and easy to maintain. Please refer to the detailed technical specifications as stipulated in the Tender.

- Library Furniture & Interiors

  - Double-sided steel book rack bay (6 shelves/side, 900mm)

  - Ergonomic reading chair with metal body

  - Wire-managed reading tables (4 seater, 6 seater)

  - Wire-managed customized conference tables

  - Acoustic panel carrels for research cubical

  - Lounge seating with comfortable chairs

  - Book trolley/cart (heavy duty)

  - Magazine display rack (5-shelf, single-sided)

  - Newspaper stand (6-rod)

  - Step stool (library-grade)

  - Digital Library Workstations - modular, wire-managed

  - Interior works in partitions with gypsum/wood/engineered wood, wall paneling, false ceiling, veneering with laminate/wood veneer, wooden flooring, carpeting, glass work, DGU paneling, acoustic treatment, blinds, service bands for electrical, CCTV, Data, DALI etc., signage, POP works, stone work, tack board, glazing work, aluminium works, fabrication work, customized conference tables, etc. all kinds of interior works solutions

  - All kinds of acoustic works

- Museum & Exhibition Zone

  - Modular partitions, illuminated showcases, pedestals, and movable display panels.

- Periodicals & Reference Sections

  - Slanted display racks, reference desks, newspaper stands.

- Circulation & Issue Counters

  - Custom-designed counters to be with integrated RFID staff station (not in scope).

  - Barrier-free design for specially challenged users.

- Reading Zones with Charging Facilities

  - Lounge comfortable seating for long hours

- Children’s Corner

  - Child-safe furniture, low-height shelves, colorful interiors.

- Group Discussion Rooms, Recording Studio & AV Rooms

  - Acoustic wall panels, ergonomic furniture.

  - Glass partitions with soundproofing.

### GENERAL GUIDELINES FOR BIDDERS

###### GENERAL:

  - Letter of transmittal and forms, terms and conditions for deciding eligibility are given in this tender document.

  - All information called for in the enclosed forms should be furnished against the relevant columns in the forms. If for any reason, information is furnished on a separate sheet, this fact should be mentioned against the relevant column. In case no information is to be provided in a particular column, “nil” or “no such case” or “not available” entry should be made in that column. If any particulars/query is not applicable in case of a bidder, “not applicable” shall be mentioned against such particular/query. The bidders are cautioned that not giving complete information called for in the application forms or not giving it in clear terms or making any change in the prescribed forms or deliberately suppressing the information may result in the tender being summarily disqualified and rejected entitling the Owner to take suitable action as per Bid Security Declaration form and forfeit e-Tender Processing Fee without any further notice to the bidder. Tenders made by telegram or telex and including those received late shall not be entertained and returned unaccepted.

  - The tender should be type written. The bidder should sign each page of application. Scanned copies of these signed documents should be uploaded for online submission.

  - Overwriting should be avoided. Corrections, if any, should be made by neatly crossing out, initiating, dating and rewriting. Pages of eligibility criteria are numbered. Additional sheets, if any added by the bidder, should also be numbered by him. They should be submitted as a package with signed letter of transmittal.

  - The bidder may furnish any additional information, which he thinks is necessary to establish his capabilities to successfully complete the envisaged Work. Bidders are, however, advised not to furnish any superfluous information. No information shall be entertained after uploading of eligibility criteria document unless it is called for by the Engineer-in-Charge.

  - It is desirable that the bidder is not contesting or defending any legal proceeding before a court of law or tribunal or quasi-judicial or administrative authority (“Legal Proceedings”). The bidder must submit information of all on-going Legal Proceedings and for the past seven (7) years. In the event the bidder has not been subject to any such Legal Proceedings either in process or in the past seven (7) years, an affidavit to this effect, duly notarized shall be submitted in original.

  - Any information furnished by the bidder found to be incorrect either immediately or at a later date, shall render such bidder liable to be debarred from tendering/taking up of the Work.

  - The bidder shall not have been blacklisted/debarred by any State/Central Government Department or Public Sector Undertaking or any autonomous body. The bidder shall submit a duly notarized affidavit, stating on oath that the bidder has not been blacklisted. Applications received without such affidavit in original shall stand automatically rejected. NU shall be entitled to take suitable action as per Bid Security Declaration and forfeit e-Tender Processing Fee on account of any such rejection.

  - The Contract means the documents forming the tender and acceptance thereof and the formal agreement executed between the competent authority on behalf of the President of India and the Contractor, together with the documents referred to therein including these conditions, the specifications, designs, drawings and instructions issued from time to time by the Engineer-in- Charge and all these documents taken together, shall be deemed to form one contract and shall be complementary to one another.

###### DEFINITIONS:

In this document the following words and expressions shall carry the meaning hereby assigned to them:

  - “Owner/Purchaser/Employer/NU”	shall	mean	and	refer	to	the	Nalanda University, Rajgir, Bihar

    - “Registrar” shall mean and refer to the Registrar, NU

    - “Project Manager” shall mean and refer to the person/Project Management Consultants (PMC) nominated as Project Manager by NU and shall report to Engineer in Charge.

    - “Architect Consultants” shall mean and refer to the Consultants appointed by NU as Architect Consultants.

    - "MEP Consultant" shall mean and refer to the Consultants appointed by “Architect Consultants” as MEP Consultants.

    - “Engineer-in-Charge” shall mean and refer to an authorized Engineer Officer appointed by NU who shall supervise and be In-Charge of the work.

    - “Bidder” shall mean and refer to an individual, proprietary firm, firm in partnership, limited company (private or public) or corporation but shall not include a joint venture and special purpose vehicle.

    - “Year” means financial year unless stated otherwise.

    - “Market Rate” shall be the rate as decided by the Engineer-in-Charge on the basis of the prevailing cost of materials and labour at the site (where the work is being executed) involved in executing that item plus the percentage mentioned in Schedule ‘F’ to cover, all overheads and profits, provided that no extra overheads and profits shall be payable on the part(s) of work assigned to other agency(s) by the contractor as per terms of contract.

###### METHOD OF APPLICATION:

  - If the bidder is an individual, the application for submitting tender shall be signed by such individual above the full type-written name and current address.

  - If the bidder is a proprietary concern, the application for submitting tender shall be signed by the proprietor above full type written name and the full name of the proprietor firm with its current address.

  - If the bidder is a partnership firm, the application for submitting tender shall be signed by all the partners of the partnership firm above their full typewritten names and current address, or, alternatively, by one or more partners holding power of attorney for the firm and/or other partners. A notarized copy of such power of attorney shall be submitted along with the application and it must be disclosed that the firm is duly registered under the INDIAN PARTNERSHIP ACT 1932. The application shall also be accompanied with a notarized copy of the partnership deed and current addresses of all the partners of the firm.

  - If the bidder is a limited company or a corporation, the application for submitting tender shall be signed by a duly authorized person holding power of

attorney for signing the application accompanied by a copy of the power of attorney. The bidder shall also furnish a notarized copy of the Memorandum and Articles of Association of the company.

  - FINAL DECISION MAKING AUTHORITY - NU reserves the right to accept or reject any tender and to annul the process and reject all tenders at any time without assigning any reason thereof or incurring any liability to the bidders.

  - Particulars of the work given in Brief are to be considered Provisional. They are liable to change and must be considered only as advance information to assist the Bidders.

###### SITE VISIT

  - The bidder shall visit the Site of Work, at his own cost, and examine the Site and its surroundings to satisfy himself and collect all information that may be considered necessary for proper assessment of the scope of Work enumerated in the this tender document. Bidders are requested to provide prior intimation before site visit through email at engineering@nalandauniv.edu.in.

###### CRITERIA FOR ELIGIBILITY (TECHNICAL TENDER)

  - The eligibility criteria applicable for the bidders for the present tender in terms of the scope of Work shall be:

Three similar works (40%) each costing not less than Rs. 12.42 Crore. OR

Two similar works (60%) each costing not less than Rs. 18.62 Crore. OR

One similar work costing (80%) not less than Rs. 24.83 Crore.

For the purpose of this clause, “similar work” shall mean “Interior and furnishing works for buildings like Public Libraries/ Research & Development Centers / IITs/ IIMs/ Central Universities / Deemed Universities / Colleges. Interior works should cover not less than 15,000 sqm of built-up area in a single campus under one agreement covering works in fixed partition with wood, glass, aluminum or uPVC work, false ceiling works, flooring, carpets, acoustic works, wall paneling, glazing, blinds, white boards, tack boards, skylights, glazing work, aluminum work, fabrication works, customized work stations, office tables, customized conference tables, signage & wayfinding, complete reception arrangements etc. in last seven years in India.”

Note:-

- The value of executed works shall be brought to current costing level by enhancing the actual value of work at simple rate of 7% per annum; calculated from the date of completion to previous day of last date of submission of tenders. Qualified similar works may be physically inspected, if required, by a Technical Expert Committee constituted by the Competent Authority at Nalanda University to ascertain the completion, performance on quality of works for finalizing the technical bid and their report will be final and binding.

- In case of works executed in Private Sector, Completion Certificate should be accompanied with TDS certificates.

  - The value of executed works shall be brought to current costing level by enhancing the actual value of work at simple rate of 7% per annum, calculated from the date of completion to the previous day of the last day of submission of the Tender.

  - The bidder should have had average annual financial turnover (gross) of Rs. 15.52 Crores of similar works during the immediate last five consecutive financial years balance sheets, ending 31st March 2024, duly audited by Charted Accountant. Year in which no turnover is shown would also be considered for working out the average.

  - The bidder should not have incurred any loss (Profit after tax should be positive) in more than two (2) years during available last five consecutive financial years balance sheets, ending 31st March 2020 , duly certified and audited by the Chartered Accountant.

  - The bidder shall furnish a solvency certificate of value not less than Rs. 12.42 Crores certified by his bankers as per Form-B.

  - The bidder shall have sufficient number of technical and administrative employees for proper execution of the Contract. The bidder shall have to submit a list of these employees stating clearly how these would be involved in this work within 15 days of award of work.

  - The agency/ Firm who have been debarred/ blacklisted by the Nalanda University during the past 2 years shall not be eligible to bid for this tender and if submitted, their bid shall be summarily rejected.

- Evaluation Criteria for Eligibility (Technical Tender)

  - The details submitted by the bidders will be evaluated in the following manner:

    - The initial criteria for eligibility prescribed in para 5 above in respect of experience of eligible similar class of works completed, loss, solvency and financial turn over etc. shall be first scrutinized and the bidder’s eligibility for the work shall be determined.

    - The bidders qualifying the initial criteria as set out in Para 5 above will be evaluated by scoring method on the basis of details furnished by such bidders. The detailed scoring method including all parameters & attributes have been placed under FORM F below.

    - The owner reserves the right to restrict the list of such qualified contractor to any number deemed suitable by it.

    - Evaluation of Performance: The NU reserves the right of physical inspection of completed projects of bidders conducted by a tender evaluation committee constituted by the Competent Authority, NU for evaluating the quality of work. The tender evaluation committee may visit and inspect all the eligible works that have been executed and submitted by the bidders in the Technical tender. The marks for quality will be given based on this inspection, if inspection is carried out. The Committee may also at its discretion, call for information from owners of eligible works carried out by bidders or the works in progress by bidders and evaluate the projects regarding all submission done by the bidder including litigations. The bidder shall submit a self-certified self- evaluation calculation sheet for evaluation parameters mentioned in Form F.

    - To become eligible for short-listing, the bidder shall secure at least 50

% marks in each section A, B, C, and D and 70% marks in aggregate. However, the qualifying 50% marks must be on both the attributes of section A separately. In case of eligible similar works being more than one, average value of performance of works for time overrun and quality shall be taken on the basis of performance report of these works.

###### FINANCIAL INFORMATION

The bidder shall furnish the Annual Financial Statements for the last five (5) years ending 31st March 2024 in “Form A” and Solvency Certificate in “Form B”.

###### DISCLOSURE OF EXPERIENCE IN SIMILAR WORKS

The bidder shall furnish the List of eligible similar works successfully completed during the preceding seven years (ending previous day of the last day of submission of tenders) in ‘Form C’.

###### ORGANISATION INFORMATION

The bidder shall be required to submit the information in respect of its organization in

Form E.

###### LETTER OF TRANSMITTAL

The bidder shall submit the Letter of Transmittal in the format as prescribed in this document.

###### OPENING OF THE FINANCIAL TENDER

After evaluation of applications, a list of short listed agencies will be prepared. Thereafter, the financial tenders of only the qualified and technically acceptable bidders shall be opened at the notified time, date and place in the presence of the qualified bidders or their representatives. The validity of the tenders shall be 90 (Ninety) days and shall be reckoned from the date of opening of the Technical Tender.

###### AWARD CRITERIA

  - The Owner reserves the right, without being liable for any damages or any obligation to inform the bidder, to:

    - Amend the scope and value of Work to the bidder.

    - Reject any or all of the applications without assigning any reason.

    - Any effort on the part of the bidder or his agent to exercise any influence or to pressurize the Owner shall result in rejection of his tender. The Owner clarifies that any kind of canvassing or any act of similar nature is expressly prohibited.

###### CRITERIA FOR EVALUATION IN QUALITY AND COST BASED SELECTION

(QCBS) PROCESS (as per GFR Rules) –

  - The quality of proposals received through this tender shall be assessed through QUALITY AND COST BASED SELECTION (QCBS) process (as per GFR 2017 Rule 192). Initially the quality of technical proposals shall be scored as per criteria mentioned under Para 7 & 8 above. Only those responsive proposals that have achieved at least minimum specified qualifying score in quality of the technical proposals shall be considered further.

  - Thus the score achieved by the respective technically qualified bidder,

after evaluation of the technical bids based on the criteria stipulated under Para 6 above, shall be denoted as Tn.

  - After opening and scoring the financial proposals of technically qualified responsive bidders, a final combined score shall be arrived at by giving predefined relative weight ages for the score of quality of the technical proposal and the score of financial proposal.

  - The Financial Bids of technically qualified bidders will be opened on the prescribed date in the presence of bidder representatives.

  - Financial bids which appear to be either grossly under quoted or over quoted are liable to be rejected and this will be at the sole discretion of the Owner.

  - The bidder with lowest qualifying financial bid (L1) will be awarded 100% score (amongst the bidders which did not get disqualified on the basis of para 16.5 above). Financial Scores for other than L1 bidders will be evaluated using the following formula –

Normalized Financial Score of a Bidder (Fn) = {(Quoted Price of

L1 / Quoted Price of the respective Bidder) X 100} %

(Adjusted to two decimal places)

  - Only fixed price financial bids indicating total price for all the deliverables and services specified in this NIT of EPC tender will be considered.

  - The bid price will include all taxes and levies including GST and shall be in Indian Rupees and mentioned separately.

  - Any conditional bid would be summarily rejected.

  - Errors & Rectification: Arithmetical errors will be rectified on the following basis: “If there is a discrepancy between the unit price and the total price that is obtained by multiplying the unit price and quantity, the unit price shall prevail and the total price shall be corrected.

  - The Tender Evaluation Committee appointed by the Competent Authority at Nalanda University would evaluate the commercial bids in isolation and in comparison, with other commercial bids to confirm whether all foreseeable & probable risks have been factored in appropriately at the fair market price. Also the ability of the bidder to absorb the adverse risk position shall also be evaluated.

  - In case the Tender Evaluation Committee feels that the commercial risks have not factored in all such costs & risks mitigation plan and necessary contingency, the commercial bid proposal may be rejected.

  - The Proposal Evaluation Committee shall also confirm whether any new

avenues of costs OR revenues should not be mentioned in the Price Bid, which was not detailed out previously in the Technical Bid.

###### COMBINED AND FINAL EVALUATION

  - The technical and financial scores secured by each bidder will be added using weight age of 70% and 30% respectively to compute a Composite Bid Score. The bidder securing the highest Composite Bid Score will be adjudicated as the most responsive Bidder for award of the Project. The overall score will be calculated as follows:

Bn = 0.70 * Tn + 0.30 * Fn

Where,

Bn = overall score of the Bidder

Tn = Technical score of the Bidder (out of maximum of 100 marks achieved based on technical bid evaluation criteria under Para 8 above)

Fn = Normalized Financial score of the Bidder (as mentioned in Para

13.6 above)

  - In the event the bid composite scores are tied, the bidder securing the highest technical score will be adjudicated as the Best Value Bidder for award of the Project

From: To

### INFORMATION REGARDING ELIGIBILITY CRITERION

###### LETTER OF TRANSMITTAL

The Registrar,

Nalanda University, Rajgir, Bihar.

Dear Sir,

Subject: (Name of Work)

###### TENDER NO :

Having examined the details given in Press –Notice and Technical Tender document for the above work in the subject herein above, I/we hereby submit the relevant information as hereunder:

I/We hereby certify that all the statement made and information supplied in the enclosed Forms A to J and accompanying statement are true and correct in all respects and no information has been concealed and misrepresented.

I/We have furnished all information and details necessary for certifying the eligibility and that no further pertinent information required to be shared remains undisclosed.

I/We hereby submit the requisite certified solvency certificate and authorize the Vice Chancellor, NU to approach the bank, issuing the solvency certificate to confirm the correctness and veracity thereof. I/We also authorize Vice Chancellor, NU to approach individuals, owners, firms and corporations to verify my/our competence and reputation.

I/We submit the following certificates in support of my/our suitability, technical knowledge and capability for having successfully completed the following works:

| Sr. No. | Name of Work | Certificate from |
| --- | --- | --- |

Certificate:

It is certified that the information given in the enclosed Eligibility Tender are correct. It is also certified that I/we shall be liable to be debarred, disqualified/cancellation of enlistment in case any information furnished by me/us is found to be incorrect.

Enclosures: Seal of bidder Date of submission

SIGNATURE (S) OF BIDDER (S)

### FORM A

FINANCIAL INFORMATION

Financial Analysis-Details to be furnished duly supported by figures in balance sheet/profit & loss account for the last five (5) years duly certified by the Chartered Accountant, as submitted by the bidder to the Income Tax Department (copies of all the documents to be attached).

| Sl. No. | Description | Years |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 2020-21 | 2021-22 | 2022-23 | 2023-24 | 2024-25 |
| i) | Gross Annual turnover |  |  |  |  |  |
| ii) | Turnover on similar works |  |  |  |  |  |
| iii) | Profit/Loss |  |  |  |  |  |

Financial arrangements for carrying out the proposed work. Solvency Certificate from Bankers of bidder in the prescribed Form B

SIGNATURE OF BIDDER (S)

Signature of Chartered Accountant with Seal

### FORM B

(FORM OF BANKER’S SOLVENCY CERTIFICATE FROM A SCHEDULED BANK)

### SOLVENCY CERTIFICATE

This is to certify that to the best of our knowledge and information that M/s.

/Shri……………………………………… having marginally noted address, a customer of our bank are/is respectable and can be treated as good for any engagement up to a limit of Rs……………(Rupees…	).

This certificate has been issued at the specific request for limited purpose of submitting the Tender to the Nalanda University, Rajgir, Bihar and shall not be used for any other purpose whatsoever.

This certificate is issued without any guarantee or responsibility on the bank or any of our officers and employees.

(Signature) For the Bank

GENERAL INSTURCTIONS:

- Banker’s Solvency Certificate must be on the letterhead of the bank issuing such certificate, sealed in a cover addressed to the Vice Chancellor, NU.

- In case the Solvency Certificate is issued at the request of a partnership firm, such certificate must include names of all partners as recorded with the Bank.

- The Solvency Certificate shall not be more than 6 months old from the original date of submission of tender.

### FORM C

###### LIST OF SIMILAR WORKS COMPLETED DURING THE LAST SEVEN YEARS  ENDING PREVIOUS DAY OF LAST DATE OF SUBMISSION OF TENDER

| A | B | C | D | E | F | G | H | I | J |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Name |  |
|  |  |  |  |  |  |  | Litigation | and | Wheth |
|  |  |  |  |  |  |  | / | address / | er the |
|  |  |  | Cost of |  |  |  | arbitratio | telephone | work |
|  |  |  | work |  |  |  | n cases | number | was |
|  | Name | Owner or | in | Date of | Stipula |  | pending / | of officer | done on |
|  | of work/ | sponsorin | Crores | commen | ted date | Actual | in | to whom | back to |
|  | project | g | of | cement | of | date of | progress | reference | back |
| Sr. | and | organizati | Rupee | as per | completi | Compl | with | may be | basis |
| No | location | on | s | contract | on | Etion | details* | made | Yes/No |
| 1 |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |

*Indicate gross amount claimed and amount awarded by the Arbitrator.

* Certified that the above information is complete and that the information furnished above is true, correct and not misleading to my/our knowledge and belief.

SIGNATURE OF BIDDER (S)

FORM D (On letterhead of respective owners of the Work) PERFORMANCE REPORT OF WORKS REFERRED TO IN ‘FORM C’

| 1 | Name of work/ Project & Location |  |
| --- | --- | --- |
| 2 | Agreement No. |  |
| 3 | Estimated Cost |  |
| 4 | Tendered Cost |  |
| 5 | Date of Start |  |
| 6 | Date of completion |  |
|  | (i) Stipulated Date of Completion |  |
|  | - Actual Date of Completion |  |
| 7 | - Whether case of levy of compensation for delay has been decided or not. - If decide, amount of compensation levied for delayed completion, if any | Yes/No |
| 8 | Performance Report |  |
|  | 1) Quality of Work | Outstanding/Very Good/Good/Satisfactory/Poor |
|  | 2) Financial Soundness | Outstanding/Very Good/Good/Satisfactory/Poor |
|  | 3) Technical Proficiency | Outstanding/Very Good/Good/Satisfactory/Poor |
|  | 4) Resourcefulness | Outstanding/Very Good/Good/Satisfactory/Poor |

|  | 5) General Behaviour | Outstanding/Very Good/Good/Satisfactory/Poor |
| --- | --- | --- |
| Dated: Stamp: |  |  |

Certified that the above information is complete and that the information furnished above is true, correct and not misleading to my/our knowledge and belief.

(To be certified by an officer not below the rank of Executive Engineer or equivalent of the client department / institution)

### FORM E

STRUCTURE & ORGANIZATION

| 1 | Name & Address of the bidder |  |  |
| --- | --- | --- | --- |
| 2 | Telephone No./Telex No./Fax No./e-mail address |  |  |
| 3 | Legal status of the bidder (attach copies of original document defining the legal status). |  |  |
|  | a) | An Individual |  |
|  | b) | A Proprietary Firm |  |
|  | b) | A firm in partnership |  |
|  | c) | A limited company or Corporation |  |
| 4 | Particulars of registration with various Government bodies (attach attested photo-copy). (Company Registration, PAN no., CST and VAT Registration, Service Tax Registration, PF Registration, ESI Registration, Registration as a Contractor with CPWD/MES/State PWD etc. ant information about others) |  |  |
|  | ORGANIZATION/PLACE OF REGISTRATION |  | REGISTRATION No. |
|  | 1 |  |  |
|  | 2 |  |  |
| 5 | Names and Titles of Directors& Officers with designation to be deputed for the Works |  |  |
| 6 | Designation of individuals authorized to act for the organization. |  |  |

| 7 | Has the bidder or any constituent partner in case of partnership firm/Limited Company/Joint Venture, ever been convicted by a court of law? If so, give details. |  |
| --- | --- | --- |
| 8 | In which field of Electrical Engineering the bidder has specialization and interest? |  |
| 9 | Any other information considered necessary but not included above. |  |

Certified that the information mentioned above is complete and no information has been left undisclosed and that the information furnished above is true, correct and not misleading to my/our knowledge and belief.

SIGNATURE OF BIDDER (S)

FORM F

ELIGIBILITY	CRITERION	FOR	TECHNICAL	EVALUATION OF EPC CONTRACTORS

| Sl. No. | Attributes | Maximum Marks | Remarks |
| --- | --- | --- | --- |
| A | Financial Strength (a) Average annual turnover (a) Solvency Certificate | 15 marks (10 marks) (5 Marks) | - 60% marks for minimum eligibility criteria - 100% marks for twice the minimum eligibility criteria or more - In between (i) & (ii) on pro-rata basis |
| B | Experience in similar class of works | 15 Marks | - 60% marks for minimum eligibility criteria - 100% marks for twice the minimum eligibility criteria or more - In between (i) & (ii) on pro-rata basis |
| C | Presentation of the Bidder – | 30 Marks | Shall be evaluated based on – |

|  | (i) |  | Evaluation Parameters - - Overall Aesthetic Beauty of the Nalanda Library interior design in coherence with the legacy of ancient Nalanda. - More preference shall be given to proposals which are in harmony with the remaining campus architecture - Sustainability features - inclusion of technology and design elements to create the buildings with a lower carbon footprint conforming to GRIHA 5- Star rating features - Comfort level - with the right proportions for different spaces and the amount of people occupying it. The building (s) should not feel cramped, awkward or too large for its primary function - Overall Space Management – to be dynamic & flexible in order to deliver a building that will function for decades to come. A tentative details of floor wise plan for Nalanda University library is enclosed with this tender (separate document). - Material Selection – Based on climate, durability, availability, sustainability, maintenance, aesthetic appeal and performance and GRIHA requirement along with the specific requirements of the Owner as stipulated in this EPC NIT. |
| --- | --- | --- | --- |
|  | Proposed |  |  |
|  | Conceptual |  |  |
|  | Plan, | (20 Marks) |  |
|  | Specifications |  |  |
|  | & 3D views |  |  |
|  | (ii)Innovative Design/ Usage of eco-friendly construction material | (10 Marks) | Any value addition from the bidder assessed through Proposal document / methodology of construction proposed etc. |
| D | Performance on works (Time Over Run) | 15 Marks | Shall be evaluated based on – |

|  | Parameter Calculation for Marks If TOR = 1.00 2.00 3.00 >3.50 Without levy of 15 11.25 7.5 7.5 Compensation With levy of 15 3.75 0 -3.75 compensation Levy of 15 7.5 0 0 compensation not yet decided For the above calculations TOR = AT/ST Where, AT = Actual Time taken for completion of the work ST = Stipulated Time in the agreement (+) justified period of extension of time. Note: Marks for value in between the stages indicated above is to be determined by straight line variation basis. |  |  |
| --- | --- | --- | --- |
| E | Performance on works (Quality) | 25 marks | Maximum – 25 Marks |
|  |  |  | (Based on Form-D) |
|  |  |  | Outstanding Quality – 100% Marks |
|  |  |  | Very Good Quality – 80% Marks |
|  |  |  | Good/Average/Satisfactory Quality – 60% |
|  |  |  | Marks |
|  |  |  | Poor Quality - 0 Marks |
|  |  |  | The evaluation will be done by the expert |
|  |  |  | committee/jury nominated by the Nalanda |
|  |  |  | University for the purpose based on |
|  |  |  | documents submitted and/or other means as decided by the committee/jury who may include visit of the site of project referred to |
|  |  |  | by the bidder or any other project deemed |
|  |  |  | fit by Engineer In Charge. |
|  |  |  | The bidder must submit precise certificates |
|  |  |  | issued by the Owner/owner signed by an |
|  |  |  | officer not below the rank of Executive |
|  |  |  | Engineer or Project Manager or equivalent |
|  |  |  | with completed photographs of the project |
|  |  |  | (s). |

#### Note:

- To become eligible for short-listing, the bidder shall secure at least 50 % marks in each section A, B, C, D & E and 70% marks in aggregate. However, the qualifying 50% marks must be on both the attributes of section A separately. In case of eligible similar works being more than one, average value of performance of works for time overrun and quality shall be taken on the basis of performance report of these works.

- The bidder meeting the minimum eligibility criteria shall be allotted 60% marks and meeting twice the eligibility criteria shall be given 100% marks in all sections. In between shall be calculated on pro-rata basis.

### FORM G

GENERAL INSTRUCTIONS FOR THE AFFIDAVIT

The affidavit shall be executed on appropriate non-judicial stamp paper of minimum value as applicable in the State of Bihar and notarized by a Notary Public;

Scanned copy of the affidavit shall be uploaded at the time of submission of the tender in soft copy.

###### AFFIDAVIT FOR SITE VISIT

I, 	, aged     years, son/daughter of	, presently residing at

and authorized by 	 (name of bidder) (“Bidder”) to solemn this affidavit on behalf of the Bidder, solemnly affirm on oath as hereunder:

The Bidder confirms that the Bidder has duly undertaken the visit of the proposed project site of Nalanda University, located at Rajgir, Bihar.

The Bidder has inspected and examined its surroundings and has satisfied itself about the site conditions and site logistics. The Bidder confirms that it is aware of the ground conditions and nature of the site, means of access to the site and the accommodation area required for establishing the labour camp. The Bidder agrees and confirms it shall be solely responsible for arranging and maintaining the afore- mentioned at its own cost including all materials, tools & plants, water, electricity, access, facilities for workers and all other services required for executing the Work unless otherwise specifically provided for in the contract documents.

The Bidder confirms and agrees that the submission of the tender implies that the requisite site visit has already been undertaken and that the Bidder has acquainted itself with the local conditions and other factors having a bearing on the execution of the Work.

DEPONENT VERIFICATION

I, 	, aged 	 years, son/daughter of	, presently residing at

and authorized by Bidder verify that the information mentioned above is true and correct to the best of my knowledge and belief.

DEPONENT

### FORM H

GENERAL INSTRUCTIONS FOR THE AFFIDAVIT:

The affidavit shall be executed on appropriate non-judicial stamp paper of minimum value as applicable in the State of Bihar and notarized by a Notary Public.

Scanned copy of the affidavit shall be uploaded at the time of submission of the tender in soft copy.

###### AFFIDAVIT FOR NO BLACKLISTING/DEBARRMENT

- I, the undersigned, do hereby certify that all the statements made in the required attachments are true and correct.

- The  undersigned  also  hereby  certifies  that  neither  our  firm M/s	 	 has been blacklisted/debarred nor has abandoned any work in any government department, India nor any contract awarded to us for such works have been rescinded, during last five years prior to the date of this bid.

- The undersigned hereby authorizes and request(s) any bank, person, firm or corporation to furnish pertinent information deemed necessary and requested by the Department to verify this statement or regarding my (our) competence and general reputation.

- The undersigned understand and agrees that further qualifying information may be requested and agrees to furnish any such information at the request of the Department Project implementing agency.

(Signed by an Authorized Officer of the Firm)

Title of Officer

Name of Firm

#### Requirement of technical Staff Technical Staff

The bidder shall deploy sufficient number of technical and administrative employees for smooth execution of the Work. However Minimum Technical Staff to be deployed at site at all times during the execution of the work will be as per following table. The bidder shall submit a list of employees stating clearly the responsibility to be assigned to each of such employees. Further, the bidder shall have the capability and resources to arrange at least 15±10% numbers of skilled and 25±10% numbers of unskilled labourers per day continuously for various stages of the Work. The bidder shall have to demonstrate the capability to arrange such workers and additional numbers as per the requirement assessed by the Engineer-in-Charge for timely completion of various stages of the Work. The capability as claimed by the bidder shall be supported by the past performance and an undertaking.

| Technical Staff | Requirement of Technical staff |  | Minimum experience of Execution in work (Years) |
| --- | --- | --- | --- |
|  | Qualification | Number |  |
| Senior Interior | Senior Architect | 1 | 10 |
| Designer |  |  |  |
|  | Graduate |  |  |
| Project Manager | Engineer/Architect | 1 | 10 |
| Site Engineer | Graduate Engineer |  | 5 |
|  | Or |  |  |
|  | Diploma Engineer | 1 | 10 |
| Planning Engineer | Graduate Engineer |  | 2 |
| / Billing Engineer | Or |  |  |
|  | Diploma Engineer |  | 5 |
|  |  | 1 |  |
| Store In-Charge | Experienced Store |  | 5 |
|  | Keeper |  |  |

### INTEGRITY PACT DECLARATION

In order to maintain transparency and integrity in the process of awarding contract, it is essential for NU and bidders to agree and abide by certain principles and policies. Therefore, NU and the bidder agree to the following:

NU shall remain committed to follow the principles of transparency, equity, and competitiveness in public procurement.

  - The Notice Inviting Tender (NIT) is an invitation to offer made on the condition that the bidder shall sign the Integrity Agreement, which is an integral part of tender/tender documents, failing which the bidder will stand disqualified from the tendering process and the tender of the bidder shall be liable to be rejected summarily.

  - The bidders agree and acknowledge that the NIT is an invitation to offer made on the condition that the bidders shall sign an Integrity Agreement in the format as provided for in this document. Such Integrity Agreement shall form an integral part of the tender documents. Any failure on the part of any bidder to execute the Integrity Agreement shall render such bidder disqualified from the tendering process. The bidder agree and acknowledge that bidding for the Work as envisaged in this document shall be regarded as an unconditional and absolute acceptance of the condition of executing the Integrity Agreement.

  - The bidders acknowledge, agree and confirm acceptance and compliance with the Integrity Agreement in letter and spirit and further agree that execution of the Integrity Agreement shall be separate and distinct from the main agreement, which shall come into existence once tender is finally accepted by NU. The bidders acknowledge and accept the tenure of the Integrity Agreement, which shall be in the line with Article 1 of the format of Integrity Agreement as prescribed herein.

The bidders acknowledge that in the event of failure to sign and accept the Integrity Agreement, while submitting the tender/tender documents, NU shall have unqualified, absolute and unfettered right to disqualify the bidder and reject the tender in accordance with terms and conditions of the tender. In addition to other components of tender document, the Integrity Pact shall also be signed between The Registrar, NU and successful bidder after acceptance of tender.

To,

………………………..,

………………………..,

### INTEGRITY PACT DECLARATION

###### (BY NU)

Sub: Notice Inviting Tender for Interior works including partitions, Facades, flooring, wall panelling, False Ceiling, Fixed furniture, etc. for the Administration Block at the Proposed Permanent Campus of Nalanda University, Rajgir, Bihar.

Dear Sir,

It is here by declared that the Nalanda University is committed to follow the principle of transparency, equity and competitiveness in public procurement.

The subject Notice Inviting Tender (NIT) is an invitation to offer made on the condition that the Bidder will sign the integrity Agreement, which is an integral part of tender/tender documents, failing which the bidder will stand disqualified from the tendering process and the tender of the bidder would be summarily rejected.

This declaration shall form part and parcel of the Integrity Agreement and signing of the same shall be deemed as acceptance and signing of the Integrity Agreement on behalf of the NU.

Yours faithfully

Registrar, NU

To,

The Registrar,

### INTEGRITY PACT DECLARATION (BIDDERS)

Nalanda University, Rajgir, Bihar

Sub: Submission of Tender for 	(Name of Work)	. Dear Sir,

I/We, -----(Name of The bidder)------------------------------------------------------------

------, acknowledge that the Nalanda University is committed to follow the principles thereof as enumerated in the Integrity Agreement enclosed with the tender/tender document.

I/We agree that the Notice Inviting Tender (NIT) is an invitation to offer made on the condition that I/We will sign the enclosed integrity Agreement, which is an integral part of tender documents, failing which I/We will stand disqualified from the tendering process. I/We acknowledge that THE MAKING OF THE TENDER SHALL BE REGARDED AS AN UNCONDITIONAL AND ABSOLUTE ACCEPTANCE of this condition of the NIT.

I/We confirm acceptance and compliance with the Integrity Agreement in letter and spirit and further agree that execution of the said Integrity Agreement shall be separate and distinct from the main contract, which will come into existence when tender is finally accepted by NU. I/We acknowledge and accept the duration of the Integrity Agreement, which shall be in the line with Article 1 of the enclosed Integrity Agreement.

I/We acknowledge that in the event of my/our failure to sign and accept the Integrity Agreement, while submitting the tender/tender documents, NU shall have unqualified, absolute and unfettered right to disqualify the bidder and reject the tender in accordance with terms and conditions of the tender.

Yours faithfully

(Duly authorized signatory of the Bidder)

(To be signed by the bidder and the Registrar, NU)

### INTEGRITY AGREEMENT

THIS INTEGRITY AGREEMENT is made at ............... on this........... Day of

.	2021

BETWEEN

The Vice Chancellor, Nalanda University represented through the Registrar, Nalanda University (hereinafter referred as “NU” or “Owner” which expression shall unless repugnant to the context or meaning thereof is deemed to include its successors & permitted assigns) of the FIRST PART:

AND

.	(Name and

Address       of       the       Individual/firm/Company) through.	(Hereinafter referred to as

the (Details of duly authorized signatory) “Bidder/Contractor” and which expression shall unless repugnant to the meaning or context hereof include its successors and permitted assigns) of the SECOND PART

Each of the NU/Owner and Bidder/Contractor individually referred to as the “Party” and collectively referred to as the “Parties”.

Preamble

WHEREAS  the  Owner  floated  a  Tender  for	(Name of Work)	. (“Tender”) and intends to award, under laid down organizational procedure, contract for ………………………………………… (Name of Work) hereinafter referred to “Contract”.

AND WHEREAS the Owner values full compliance with all applicable laws of the land, rules, regulations, economic use of resources and of fairness/transparency in its relationship with its bidder(s) and contractor(s).

AND WHEREAS to meet the purpose aforesaid, the Parties have agreed to enter into this Integrity Agreement (hereinafter referred to as “Integrity Pact” or “Pact”), the terms and conditions of which shall also be read as integral part and parcel of the tender documents and Contract between the Parties.

NOW, THEREFORE, in consideration of mutual covenants contained in this Pact, the Parties hereby agree as follows and this Pact witnesses as under:

Article 1: Commitment of the Owner

- The Owner commits itself to take all measures necessary to prevent corruption and to observe the following principles:

  - No employee of the Owner, personally or through any of his/her family members, will in connection with the Tender, or the execution of the Contract , demand, take a promise for or accept, for self or third person, any material or immaterial benefit which the person is not legally entitled to.

  - The Owner will, during the tender process, treat all bidder(s) with equity and reason. The Owner will, in particular, before and during the tender process, provide to all bidder(s) the same information and will not provide to any bidder(s) confidential/additional information through which the bidder(s) could obtain an advantage in relation to the tender process or the Contract execution.

  - The Owner shall endeavour to exclude from the tender process any person, whose conduct in the past has involved any bias of any nature.

- If the Owner obtains information on the conduct of any of its employees which is a criminal offence under the Indian Penal Code (IPC) or Prevention of Corruption Act, 1988 or is in violation of the principles herein mentioned or if there be a substantive suspicion in this regard, the Owner will be at liberty to take appropriate disciplinary action or initiate disciplinary proceedings as per its internal policies and procedures.

Article 2: Commitment of the Bidder(s)/Contractor(s)

- Each Bidder/Contractor shall be required to (including their respective officers, employees and agents) adhere to the highest ethical standards and shall report to the Owner of any suspected acts of fraud or corruption or coercion or collusion of which it has knowledge or becomes aware, during the tendering process and throughout the negotiation and award of the Contract.

- The Bidder(s)/Contractor(s) commits themselves to take all measures essential to prevent any act of corruption. The Bidders/Contractors commit themselves to observe the following principles during their participation in the Tender process and during the execution of the Contract:

  - The Bidder(s)/Contractor(s) will not, directly or through any other person or firm, offer, promise or give to any of the owner’s employees involved in the Tender process or execution of the Contract, or to any third person any material or other benefit which such person is not legally entitled to, in order to obtain in exchange any advantage of any kind whatsoever during the Tender Process or during the execution of the Contract.

  - The Bidder(s)/Contractor(s) shall not enter with other Bidder(s) in to any undisclosed agreement or understanding, whether formal or informal. This applies in particular to prices, specifications, certifications, subsidiary contracts, submission or non-submission of tenders or any other actions to restrict competitiveness or to cartelize in the tendering process.

  - The Bidder(s)/Contractor(s) will not commit any offence under the relevant Indian Penal Code (IPC)/PC Act. Further the Bidder(s)/Contractor(s) will not use improperly, (for the purpose of competition or personal gain), or pass on to others, any information or documents provided by the Owner as part of the business

relationship regarding plans, technical proposals and business details, including information contained or transmitted electronically or otherwise.

  - The Bidder(s)/Contractor(s) of foreign origin shall disclose the names and addresses of agents/representatives in India, if any. Similarly Bidder(s)/Contractor(s) of Indian nationality shall disclose names and addresses of foreign agents/representatives, if any. Either the Indian agent on behalf of the foreign principal or the foreign principal directly can submit tender in the process of tender, but not both. In cases where an agent participates in the tender on behalf of one bidder, such agent shall not be permitted to quote on behalf of any another bidder along with the first manufacturer in a subsequent/parallel tender for the same item.

  - The Bidder(s)/Contractor(s) will, when presenting tender, disclose any and all payments made, is committed to make or intends to make to agents, brokers or any other intermediaries in connection with the award of the Contract.

- The Bidder(s)/Contractor(s) will not instigate any third person to commit offences mentioned above or be an accessory to such offences.

- The Bidder(s)/Contractor(s) will not, directly or through any other person or firm indulge in fraudulent practice means wilful misrepresentation or omission of facts or submission of fake/forged documents in order to induce public official to act in reliance thereof, with the purpose of obtaining unjust advantage by or causing damage to justified interest of others and/or to influence the procurement process to the detriment of the interest of the Owner.

- The Bidder(s)/Contractor(s) will not, directly or through any other person or firm use coercive practices (means the act of obtaining something, compelling an action or influencing a decision through intimidation, threat or the use of force directly or indirectly, where potential or actual injury may befall upon a person, his/ her reputation or property to influence their participation in the tendering process).

Article 3: Consequences of Breach

Without prejudice to any rights that may be available to the Owner under law or the Contract or the Contract or its established policies and laid down procedures, the Owner shall have the following rights in case of breach of this Integrity Pact by the Bidder(s)/Contractor(s) and the Bidder/ Contractor accepts and undertakes to respect and uphold the Owner’s absolute right:

- If the Bidder(s)/Contractor(s), either before award or during execution of Contract has committed a transgression through a violation of Article 2 above or in any other form, such as to put his reliability or credibility in question, the Owner after giving 14 days’ notice to the Bidder/Contractor shall have the powers to disqualify the Bidder(s)/Contractor(s) from the process of Tender or terminate/determine the Contract, if already executed or exclude the Bidder/Contractor from future contract award processes. The imposition and duration of the exclusion will be determined by the severity of transgression and

determined by the Owner. Such exclusion may be forever or for a limited period as decided by the Owner.

- Forfeiture of EMD/Performance Guarantee/Security Deposit: If the Owner has disqualified the Bidder(s) from the Tender process prior to the award of the Contract or terminated/determined the Contract or has accrued the right to terminate/determine the Contract according to Article 3(1), the Owner apart from exercising any legal rights that may have accrued to the Owner, may in its considered opinion forfeit the entire amount of Performance Guarantee and Security Deposit rendered by the Bidder/Contractor.

- Criminal Liability: If the Owner obtains knowledge of conduct of a Bidder or Contractor, or of an employee or a representative or an associate of a Bidder or Contractor which constitutes corruption within the meaning of the Indian Penal Code (IPC) Act or if the Owner has substantive suspicion in this regard, the Owner will forthwith inform the same to any law enforcing agencies for further investigation..

Article 4: Previous Transgression

- The Bidder/Contractor declares that no previous transgressions occurred in the last five (5) years with any other company in any country confirming to the anti- corruption approach or with the Central Government or State Government or any other Central/State Public Sector Enterprises in India that could justify its exclusion from the process of Tender.

- If the Bidder makes incorrect statement on this subject, he can be disqualified from the process of Tender or action can be taken for banning of business dealings/holiday listing of the Bidder/Contractor as deemed fit by the Owner.

- If the Bidder/Contractor can prove that he has resorted / recouped the damage caused by him and has installed a suitable corruption prevention system the Owner may, at its sole discretion, revoke the exclusion prematurely. .

Article 5: Equal Treatment of all Bidders/Contractors/Subcontractors

- The Bidder(s)/Contractor(s) undertake(s) to demand from all sub-contractors a commitment in conformity with this Integrity Pact. The Bidder/Contractor shall be responsible for any violation(s) of this Integrity Pact by any of its subcontractors/sub-vendors.

- The Owner will enter into such agreements or pacts on identical terms as this Integrity Pact with all Bidders and Contractors.

- The Owner will disqualify Bidders, who do not submit the duly signed Integrity Pact between the owner and the Bidder, along with the Tender or violate its provisions at any stage of the Tender process, from the Tender process.

Article 6- Duration of the Pact

This Pact begins when both parties (the Owner and the Bidder/Contractor) have legally signed it. It expires for the Contractor/Vendor	twelve (12) months after the

completion of work under the contract or till the continuation of defect liability period, whichever is more and for all other Bidders, till the Contract has been awarded

If any claim is made/lodged during the time, the same shall be binding and continue to be valid despite the lapse of this Pact as specified above, unless it is discharged/determined by the Owner.

Article 7- Other Provisions

- This Integrity Pact is subject to Indian laws and the place of performance and jurisdiction is the place of office of the Owner/NU who has floated the Tender.

- Any change, amendment, modification or supplement or addendum to the Integrity Pact can only be brought into effect by way of mutual written agreement between the parties to the Integrity Pact.

- If the Contractor is a partnership or a consortium, this Pact shall be signed by all the partners or by one or more partners holding power of attorney signed by all partners and consortium members. In case the contractor is a company, the Pact must be signed by a representative duly authorized by board resolution of such company.

- Should one or several provisions of this Integrity Pact turn out to be invalid, the remainder of the Integrity Pact remains valid. In this case, the Parties will strive to come to an agreement to their original intensions.

- It is agreed term and conditions that any dispute or difference arising between the Parties with regard to the terms of this Integrity Agreement/ Pact, any action taken by the Owner in accordance with this Integrity Agreement / Pact or interpretation thereof shall not be subject to arbitration.

Article 8- LEGAL AND PRIOR RIGHTS

All rights and remedies of the Parties hereto shall be in addition to all the other legal rights and remedies belonging to such Parties under the contract and/or law and the same shall be deemed to be cumulative and not alternative to such legal rights and remedies aforesaid. For the sake of brevity, both the Parties agree that this Integrity Pact shall have precedence over the Tender/Contact documents with regard to any of the provisions covered under this Integrity Pact.

IN WITNESS WHEREOF the Parties have signed and executed this Integrity Pact at the place and date first above mentioned in the presence of following witnesses

:...............................................................

(For and on behalf of Owner)

.................................................................

(For and on behalf of Bidder/Contractor)

WITNESSES (Signature, name and address)

- ..............................................

- ...............................................

Place:

Dated:

(To be executed by and between the Owner and the successful bidder)

### FORMAT OF AGREEMENT

This	agreement	is	executed	at			(place	of	execution)	on	the

day of 	, 20	 BETWEEN

, which expression shall mean and include its successors and assigns (name and address of the Owner) (“Owner”) of the FIRST PART

AND

,

(name and address of the successful bidder) (“Contractor”) of the SECOND PART

The Owner and the Contractor shall be individually referred to as the “Party” and collectively referred to as the “Parties”

Whereas  the  Owner  is  desirous  of  “	(Name of Work)	” and had invited tenders by issuing Notice Inviting Tender (“Tender”) for selection of a contractor for constructing the said campus. The Contractor has submitted its tender pursuant to the issuing of the Tender by the Owner. WHEREAS the details of the work proposed to be executed by the Contractor

is	more	particularly		specified	in	the	“	(Name	of	 Work)		”, name and identification number of Contract) ( “Works”) and the Owner has accepted the Tender submitted by the Contractor for the execution and completion of the Works and the remedying of any defects therein, at a contract price										of

Rs....................................................................................................................

.	(Rupees 	(in words))

WHEREAS the Owner is now desirous of laying down the terms and conditions governing the execution of the Works and has therefore, requested the Contractor to execute the present Agreement.

NOW THIS AGREEMENT WITNESSETH as under:

- In this Agreement, words and expressions shall carry the same meanings as are ascribed to them in the Conditions of Contract as more particularly mentioned in the Tender. The Parties agree that the Tender shall form an integral part of this Agreement and shall be read and construed accordingly.

- In consideration of the payments to be made by the Owner to the Contractor as the consideration for execution of the Works (“Consideration”), the Contractor hereby covenants with the Owner to execute and complete the Works and remedy the defects therein in conformity in all aspects with the provisions of the Tender and this Agreement.

- The Owner hereby covenants to pay the Contractor in consideration of the execution and completion of the Works and in the remedying the defects wherein the Contract Price or such other sum as may become payable under the provisions of the Tender and this Agreement at the times and in the manner prescribed under the Tender.

- The following documents shall be deemed to form and be read and construed as part of this Agreement:

- Letter of Acceptance;

- Notice to proceed with the Works;

- Contractor’s Tender;

- Contract Data;

- Conditions of Contract (including Special Conditions of Contract);

- Specifications;

- Drawings;

- Bill of Quantities; and

- Any other documents listed in the Contract Data as forming part of the Contract.

In witness whereof the Parties have caused this Agreement to be executed on the day and year first written above.

The	Common	Seal	of

was hereunto affixed in the presence of:

Signed	Sealed	and	Delivered	by	the	said

Binding	Signature	of	Owner

Binding	Signature	of	Contractor

in	the	presence	of

SECTION 2

### PROFORMA OF SCHEDULES: A TO F

### SCHEDULE A

### SCHEDULE OF QUANTITIES

| Sl. No. | Category | Item | Unit | Qty. |
| --- | --- | --- | --- | --- |
| 1 | Movable Furniture | Double-sided steel book rack bay (6 shelves/side, 900mm) as per technical specs mentioned in this NIT | Nos | 800 |
| 2 | Movable Furniture | Reading chair (ergonomic) as per technical specs mentioned in this NIT | Nos | 3000 |
| 3 | Movable Furniture | Reading table (4-seater, wire-managed) as per technical specs mentioned in this NIT | Nos | 650 |
| 4 | Movable Furniture | Reading table (6-seater, wire-managed) as per technical specs mentioned in this NIT | Nos | 300 |
| 5 | Movable Furniture | Office tables (work tables) as per technical specs mentioned in this NIT | Nos | 100 |
| 6 | Movable Furniture | Executive Tables (Executive Desks) as per technical specs mentioned in this NIT | Nos | 50 |
| 5 | Movable Furniture | Research cubicle (acoustic paneled carrel) as per technical specification | Nos | 300 |
| 6 | Movable Furniture | Lounge/comfortable chair (Sofas & soft seating) as per technical specification | Nos | 100 |
| 7 | Movable Furniture | Book trolley/cart (heavy duty) as per technical specification | Nos | 50 |
| 8 | Movable Furniture | Modular Office Workstations as per technical specifications mentioned in this NIT | Nos | 100 |
| 9 | Movable Furniture | Storage Units (Cabinets, Shelves, and Filing) as per technical specifications mentioned in this NIT | Nos | 300 |
| 10 | Movable Furniture | Mobile Compactors (High-Density Storage System) as per technical specifications mentioned in this NIT | Nos | 16 |
| 11 | Movable Furniture | Classroom furniture (Training room set up) as per technical specifications mentioned in this NIT | Nos | 100 |
| 12 | Movable Furniture | Museum-Grade Showcases as per technical specifications mentioned in this NIT | Nos | 50 |
| 13 | Movable Furniture | Manuscript Display and Storage Units as per technical specifications mentioned in this NIT | Nos | 80 |
| 14 | Movable Furniture | Microfilm Storage Cabinets as per technical specifications mentioned in this NIT | Nos | 80 |
| 15 | Movable Furniture | Retractable Awnings (Motorized Cassette-Type) as per technical specifications mentioned in this NIT | Sqm | 4500 |
| 16 | Movable Furniture | Magazine display rack (5-shelf, single-sided) as per technical specification | Nos | 50 |
| 17 | Movable Furniture | Newspaper stand (6-rod) as per technical specification | Nos | 50 |
| 18 | Movable Furniture | Step stool (library-grade) as per technical specification | Nos | 30 |
| 19 | Movable Furniture | Signage & wayfinding (movable) as per technical specification | LS | 1 |
| 20 | Movable Furniture | Spare chairs (for peak events) - 10% of total seats as per technical specification | Nos | 300 |
| 21 | Movable Furniture | Digital Library Workstations - modular, wire-managed as per technical specification | Nos | 300 |
| 22 | Civil Interior Works | Interior works in partitions with gypsum/wood/engineered wood, wall paneling, false ceiling, veneering with laminate/wood veneer, wooden flooring, carpeting, glass work, DGU paneling, acoustic treatment, blinds, service bands for electrical, CCTV, Data, DALI etc., signage, POP works, stonework, tack board, glazing work, aluminum works, fabrication work, customized conference tables, etc. all kinds of interior works solutions as per technical specification. | Lump sum per Sqm | 16844 |

### SCHEDULE B

Schedule of materials to be issued to the Contractor

| Sr. No. | Description of item | Quantity | Rates in figures & words at which the material will be charged to the contractor | Place of issue |
| --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 |
|  | NIL | NIL | NIL | NIL |

### SCHEDULE C

Tools and plants to be made available to the Contractor:

| Sr. No. | Description of item | Quantity |
| --- | --- | --- |
| 1 | 2 | 3 |
|  | NIL | NIL |

### SCHEDULE D

Extra schedule for specific requirements / documents for the work if any.

The bidder may give additional information other then sought for in the preceding paragraph.

### SCHEDULE E

Reference to General Conditions of Contract. CPWD 2024 corrected up to last date of receipt of bid.

Name of Work: Engineering, Procurement and Construction (EPC) of Interior Works for the Nalanda Library Building at the Main Campus of Nalanda University at Rajgir, Bihar including wooden partitions, cabinets for FCUs, service bands for electrical, HVAC, CCTV, Data, DALI etc., fixed cupboards, storages (normal storage & compactors etc.), wooden flooring (wherever applicable), carpets, acoustic works, wall paneling, false ceiling, blinds, white boards, tack boards, skylights, glazing work, aluminum work, fabrication work, work stations, office tables, customized conference tables, signage & wayfinding, complete reception arrangements, supply and installation of double sided steel book racks, ergonomic reading chair, reading tables, research cubicles, lounge furniture with comfortable seating, heavy duty book trolley / cart, Magazine display racks, Newspaper stands, library grade step-stools, digital library workstations (wire- managed), and any other fixed furniture/interior items as per requirement.

Estimated cost of work: Rs. 31.04 Crores Earnest Money: INR 41,04,238/-

- Performance Guarantee: 5% (Five percent) of tendered value

- Security Deposit : 2.5% (Two Point Five Percent) of tendered value

### SCHEDULE F (GENERAL RULES & DIRECTIONS)

Authority Inviting Tender. - Registrar, Nalanda University, Rajgir, Bihar.

Maximum percentage for quantity of items of work to be executed beyond which rates are to be determined in accordance with Clauses “Deviation, Extra items and pricing & Deviation submitted items and pricing” Please refer below

Definitions:

| Sr. No | DESCRIPTION | DETAILS |
| --- | --- | --- |
| 1 | Owner | Nalanda University, Rajgir, Bihar |
| 2 | Tender Accepting Authority | Registrar, Nalanda University, Rajgir, Bihar. |
| 3 | Engineer In charge | Authorized Engineer Officer appointed by Nalanda University Rajgir, Bihar |
| 4 | Project Manager | Nominated Person/PMC by NU |
| 5 | Architect Consultants | Consultants, appointed by NU as Architect Consultants. |
| 6 | Percentage on cost of materials and Labour to cover all overheads and profits for Extra Items | 15% For Civil and Interior works. |

#### CLAUSE 1: Performance Guarantee

| 1 | Time allowed for submission of Performance Guarantee Programme Chart (Time and Progress) and applicable labour Licenses, Registration with EPFO, ESIC, and BOCW Welfare board or proof of applying therefrom the date of issue of letter of acceptance | 15 (Fifteen) Days |
| --- | --- | --- |
| 2 | Maximum allowable extension beyond the period provided in 1 above | 7 (Seven) Days with late fee @ 0.1% per day of the Performance Guarantee amount. |

#### CLAUSE 2: Compensation for Delay

| 1 | Authority for fixing compensation under its clause | Vice Chancellor, Nalanda University, Rajgir, Bihar |
| --- | --- | --- |

#### CLAUSE 5: Time and Extension for Delay

| 1 |  | Number of days from the | 15 (Fifteen) days from date of |
| --- | --- | --- | --- |
|  |  | date of issue of letter of | issue of acceptance of award |
|  |  | Acceptance for reckoning date of start. | letter or handing over of site to the agency, whichever is later. |

Milestone(s) as per table given below:-

Though the overall time of completion is 6 months the contractor shall complete the works in time so that the building can be occupied as per the university’s schedule.  Any delay in achieving specified timeline given in the Milestones (as mentioned below) of these building shall invite withholding of payments as indicated against each and every Milestone:

| Project Milestone Schedule: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sr. No. | Description of Milestone (Physical) | Time in (from start) | allowed Months date of | Amount to be with- held in case of non- achievement of milestone |  |  |
| Milestone of Works (Time duration 6 Months) |  |  |  |  |  |  |
| 1 | Submission of interior drawings, floor plans, elevations, 2D & 3D models, walkthrough in different options for approval of the Competent Authority | 15 Days |  | 0.5% Value. | of | tendered |
| 2 | After approval submission of GFC drawings and submission of all samples for each item to be executed under this tender along with all technical data sheet and test certificates | 30 days |  | 0.5% value | of | tendered |
| 3 | Completion of all partition and glazing works | 60 Days |  | 1% value | of | tendered |
| 3 | Completion of all wall paneling, acoustic works service bays, false ceiling, workstations for digital library, research cubical, exhibition hall displays, magazine stands, newspaper stands, etc. | 90 Days |  | 1% value | of | tendered |
| 4 | Completion of all other items including windows, doors, flooring, carpeting, tack board, white board, signage and wayfinding, etc. for all areas, Decorative & task lighting, wiring, controls, etc. | 100 Days |  | 0.75% value | of | tendered |
| 5 | Completion of supply and installation of all movable furniture including book racks, reading chairs, reading tables, customized tables, conference tables, classroom furniture, book trolleys, lounge seating, etc. all complete. | 160 Days |  | 0.75% of the tendered value |  |  |

| 6 | Final Handing over of all areas including all documentation all complete | 180 Days | 0.5 % of the tendered value |
| --- | --- | --- | --- |

Note: Withheld amount shall be released if and when subsequent milestone is achieved.

| 1 | Total Time allowed for execution of work | 6 (Six) Months |
| --- | --- | --- |
| 2 | Authority to decide Extension of Time | Vice Chancellor, Nalanda University, Rajgir, Bihar |
| 3 | Rescheduling of Milestones | Engineer In Charge, and the Rescheduling is to be approved by Vice Chancellor, Nalanda University (NU) |

Clause 5

| Part | Portion of site | Description | Time Period for handing over reckoned from date of issue of letter of intent. |
| --- | --- | --- | --- |
| 1 | Building | Handing over of the building for installation work | Ready to be handed over in all respect |

Schedule of handing over of site

CLAUSE 7 : Payment on Intermediate Certificate to be regarded as Advances

As per Detailed Clause

#### Clause 7A

| 1 | Whether Clause 7 A shall be applicable | Yes |
| --- | --- | --- |

List of Testing Equipment.

| 1 | List of Testing equipment to be provided by the contractor at site laboratory. | As per (Table-1) of Annexure-1 attached. |
| --- | --- | --- |

#### CLAUSE 10B (i) & 10B (ii) : Secured Advance & Mobilization Advance.

| 1 | Secured advance on Non- perishable materials & Mobilization Advance. | Applicable. |
| --- | --- | --- |

#### CLAUSE 11 : Work to be executed in accordance with Specifications, Drawings, and Orders etc.

| 1 | Specifications | Detailed Specifications, CPWD specifications and related IS/IEC and other mentioned codes For Interior works |
| --- | --- | --- |

#### CLAUSE 12 : Deviations/Variations/ Extent and Pricing

| 1 | Deviation limit beyond clause 12 shall apply for all building work (Above plinth level). | 30 % (Thirty per cent). |
| --- | --- | --- |
| 2 | Deviation Limit beyond clauses 12 for work up to plinth level | 100% (One Hundred per cent) |

#### CLAUSE 16 : Action in casework not done as per Specifications

| 1 | Competent Authority for deciding reduced rates. | Engineer in Charge |
| --- | --- | --- |

#### CLAUSE.17: Contractor Liable for Damages/ Defects during defects liability period

| 1 | Defects Liability Period | 24 months from completion of the project as a whole. |
| --- | --- | --- |

CLAUSE 38: Settlement of Disputes

| 1 | Settlement of Disputes | Committee appointed by Vice Chancellor of NU. |
| --- | --- | --- |

Employment of Technical Staff and employees

Requirement of Technical Representative(s) in progressive manner in satisfaction of Project Manager & Engineer-In-Charge with its recovery Rate.

| S. No. | Minimum Qualification of Technical Representative | Discipline | Minimum Experience | Number | Rate at which recovery shall be made from the contractor in the event of not fulfilling provision of clause Figures |
| --- | --- | --- | --- | --- | --- |
| 1. | Senior Interior Designer | Architect | 1 | 10 | Rs. 30,000/- |
| 2. | Project Manager Graduate Engineer/Archi tect | Civil Engineer/ Architect | Ten Years | One | Rs. 30,000/ p.m. |
| 3. | Graduate Engineer Or Diploma Engineer | Civil Engineer | Five Years Ten Years | One | Rs. 25,000/ p.m. Rs. 25,000/ p.m. |
| 4. | Graduate Engineer Or Diploma Engineer | Planning or Billing Engineer | Two Years Five Years | One | Rs. 15,000/-p.m. Rs. 15,000/-p.m. |
| 5. | Experience Storekeeper | Store- In- charge | Five years | One | Rs. 15,000/-p.m. |

SECTION 3

GENERAL RULES & DIRECTIONS

  - General Rules & Directions:

- The Tender shall state the work to be carried out, the date for submitting and opening tenders and the time allowed for executing the Work, the amount of earnest money to be deposited with the tender, the amount of the security deposit and performance guarantee to be deposited by the successful bidder and the percentage if any, to be deducted from the security deposit. Copies of the specifications, designs and drawings and any other documents required in connection with the Work signed for the purpose of identification by the officer inviting tender shall also be open for inspection by the bidder at the office of NU during office hours.

- The work must be executed by valid and relevant electrical license holder. The contractor has to submit copy of the electrical license within 7 days from date of Letter of Intent.

- In the event of the tender being submitted by a firm, it must be signed separately by each partner thereof or in the event of the absence of any partner, it must be signed on behalf of such partner by a person holding a valid power of attorney from such absentee partner specifically for the purposes of the tender. Such power of attorney shall be produced along with the tender. Any firm submitting its bid pursuant to the tender shall be duly registered under the Indian Partnership Act, 1952.

- Receipts for payment made on account of completed Work, when executed by a firm, must also be signed by all the partners, except where contractors are described in their tender as a firm, in which case the receipts must be signed in the name of the firm by one of the partners, or by some other person having due authority to give effectual receipts for the firm.

- This tender being a Percentage Rate Tender, contractor shall fill up the usual printed form, stating at what percentage below/above (in figures as well as in words) the total estimated cost given in Schedule of Quantities at Schedule-A, he will be willing to execute the work. The tender submitted shall be treated as invalid if:-

  - The contractor does not quote percentage above/below on the total amount of tender or any section/sub head of the tender.

  - The percentage above/below is not quoted in figures & words both on the total amount of tender or any section/sub head of the tender.

  - The percentage quoted above/below is different in figures & words on the total amount of tender or any section/sub head of the tender.

- Tenders, which propose any alteration in the work specified in the said form of invitation to tender, or in the time allowed for carrying out the work, or which contain any other conditions of any sort including conditional rebates, will be summarily rejected.

- In case the lowest tendered amount (estimated cost +/- amount worked on the basis of percentage above/below) of two or more contractors is same, such lowest contractors will be asked to submit sealed revised offer in the form of letter mentioning percentage above/ below on estimated cost of tender including all sub sections/sub heads as the case may be, but the revised percentage quoted above/below on tendered cost or on each sub section/ sub head should not be higher than the percentage quoted at the time of submission of tender. The lowest tender shall be decided on the basis of revised offers. In case any of such contractor refuses to submit revised offer, then it shall be treated as withdrawal of his tender before acceptance and suitable penal actions shall be taken as per Bid Security Declaration.

- If the revised tendered amount of two more contractors received in revised offer is again found to be equal, the lowest tender, among such contractors, shall be decided by draw of lots in the presence of the Tender Evaluation Committee and any other member nominated by the Competent Authority at NU& the lowest contractors those have quoted equal amount of their tenders.

- In case all the lowest contractors those have quoted same tendered amount, refuse to submit revised offers, then tenders are to be recalled after taking suitable action as per Bid Security Declaration form of each contractor.

- Contractor(s), action has been taken as per Bid Security declaration because of non-submission of revised offer, shall not be allowed to participate in the re- tendering process of the work.

- The NU will open tenders in the presence of any intending contractors who may be present at the time, and will enter the amounts of the several tenders in a comparative statement in a suitable form. In the event of a tender being accepted, the contractor shall thereupon for the purpose of identification sign copies of the specifications and other documents mentioned above. In the event of a tender being rejected, the Bid Security Declaration shall thereupon be returned to the respective bidders.

- The NU shall have the right of rejecting all or any of the tenders and shall not be bound to accept the lowest or any other tender.

- The receipt of an accountant or clerk for any money paid by the bidder shall not be considered as an acknowledgment for payment to the NU and the bidder shall be responsible for ensuring that a receipt signed by the NU or a duly authorized cashier is obtained for monies paid.

- The bidders shall sign a declaration under the Officials Secret Act, 1923, for maintaining secrecy of the tender documents, drawings or other records connected with the Work awarded to the bidder. The unsuccessful bidders shall return all the drawings shared during the course of the tender process.

- Use of correcting fluid, anywhere in tender document shall not be permitted. Such tender shall be liable to be rejected.

- This tender being a Percentage Rate Tender, only percentage quoted shall be considered. Any tender containing item rates is liable to be rejected. Percentage quoted by the contractor in percentage rate tender shall be accurately filled in figures and words, so that there is no discrepancy.

- This tender being a Percentage Rate Tender, the bidder shall quote percentage below/above (in figures as well as in words) at which he will be willing to execute the work. He shall also work out the total amount of his offer and the same should be written in figures as well as in words in such a way that no interpolation is possible. In case of figures, the word ‘Rs.’ should be written before the figure of rupees and word ‘P’ after the decimal figures, e.g. ‘Rs. 2.15P and in case of words, the word ‘Rupees’ should precede and the word ‘Paisa’ should be written at the end.

- The Contractor whose tender is accepted, shall be required to furnish performance guarantee of 3% (Three per cent) of the tendered amount within the period specified herein. Deposit at Call receipt of any scheduled bank/Banker's cheque of any scheduled bank/Demand Draft of any scheduled bank or Government Securities or Fixed Deposit Receipts or Guarantee Bonds of any Scheduled Bank or the State Bank of India in accordance with the prescribed form.

- The bidder whose tender is accepted shall be required to furnish by way of Security Deposit for the fulfilment of the Contract, an amount equal to 2.5% of the tendered value of the work. The Security Deposit shall be collected by deductions from the running bills as well as final bill of the Contractor at the rates mentioned above. The Security Deposit shall also be accepted in cash or in the shape of Government securities, Fixed deposit receipt of a scheduled bank or State Bank of India will also be accepted for this purpose provided conformity advice is enclosed.

- On acceptance of the tender, the name of the accredited representative(s) of the bidder, responsible for taking instructions from the Engineer-in-Charge, shall be communicated in writing to the Engineer-in-Charge/Architect Consultants with a copy to the Owner.

- GST or any other tax applicable in respect of the Contract shall be payable by the Contractor and NU shall not entertain any claim whatsoever in respect of the same. However, Component of GST at time of Supply of Service (as provided in CGST Act 2017) provided by the contract shall be varied if different from that applicable on the last date of receipt of the tender including extension if any.

- The contractor shall give a list of both Nalanda University officials/employees and/or any other official/employees of the Consultants appointed by NU in regard to this Project related to him.

- The tender for the work shall not be witnessed by a contractor or contractors who himself/ themselves has/have tendered or who may and has/have tendered for the same work. Failure to observe this condition would render, tenders of the

Contractors tendering, as well as witnessing the tender, liable to summary rejection.

- The ESI and EPF contribution on the part of employer in respect of this contract shall be paid by the contractor. These contributions on the part of the employer paid by the contractor shall be reimbursed by the Engineer-in-Charge to the contractor on actual basis. The applicable and eligible amount of EPF & ESI shall be reimbursed preferably within 7 days but not later than 30 days of submission of documentary proof of payment provided same are in order.

- The Contractor shall comply with the provisions of the Apprentices Act, 1961, and the rules and orders issued there under from time to time. Any failure to so shall amount to a breach of the Contract and the Owner may in its discretion, without prejudice to any other right or remedy available in law, cancel the Contract. The Contractor shall also be liable for any pecuniary liability arising on account of any violation under the provisions of the Apprentices Act, 1961.

SECTION 4

Scope of Work, User Requirements & Technical Specifications

The Interior Works’ preliminary concept drawings has been provided as a separate document with this tender. This document is the base document /primary requirements of the user and shall be construed as the part of the Contract Agreement.

## Minimum Acceptable Specifications for interior works –

| Sl. No. | Item | Specification |
| --- | --- | --- |
|  |  | Wooden Slats Wall Paneling |
|  |  | Supply & installation of wall paneling made |
|  |  | of pinewood E1 grade fiberboard, |
|  |  | melamine/veneer laminated finish, groove |
|  |  | perforated L32-2 - (2mm grooves @ 32mm |
|  |  | c/c), back lined with Sound texblack |
|  |  | acoustical fleece, tongue-groove edge for a |
|  |  | seamless look, 5-test fire retardant grade/ |
|  | Wall Paneling for | Non-FR, Acoustics NRC 0.65- 0.75 (For |
|  | Conference Rooms or | E300* Mounting), size 128x2440x16mm, |
| 1 | other areas wherever | volume density of base board 800 - 830 |
|  | required as per | Kg/m3, 10-10.5Kgs/m2 (L32), installed by |
|  | approved design | using Strut framework system. Slats to be |
|  |  | backed with Synthetic PF 10x25 adhered to |
|  |  | wall with stick. The Paneling shall be |
|  |  | finished as per the drawings and to the |
|  |  | satisfaction of the Engineer in Charge. All |
|  |  | the support systems shall be as per the |
|  |  | Approved make list and the suspension |
|  |  | system shall be got approved before |
|  |  | starting installation at site. |

| 2 | False Ceiling Works | Providing & fixing false ceiling at all height including providing & fixing of framework made of special section, power pressed from M.S. sheets and galvanized with zinc coating of 120 gms/ sqm (both side inclusive) as per IS : 277 and consisting of angle cleat of size 25mm wide x 1.6mm thick with flanges of 27mm and 37mm, at 1200mm c/c, one flange fixed to the ceiling with dash fastener 12.5mm dia x 50mm long with 6mm dia bolts, other flange of cleat fixed to the angle hangers of 25 x10 x0.50mm of required length with nuts & bolts of required size and other end of angle hanger fixed with intermediate G.I channels 45 x15 x 0.90mm running at the spacing of 1200 mm c/c, to which the ceiling section 0.5mm thick bottom wedge of 80mm with tapered flanges of 26 mm each having lips of 10.5mm, at 450mm c/c, shall be fixed in a direction perpendicular to G.I intermediate channel with connecting clip made out of 2.64mm dia x 230mm long G.I wire at every junction, including fixing perimeter channels 0.50mm thick 27mm high having flanges of 20mm and 30mm long, the perimeter of ceiling fixed to wall/ partitions with the help of Rawl plugs at 450mm center, with 25mm long dry wall screws @ 230mm interval, including fixing of Calcium Silicate Board to ceiling section and perimeter channels with the help of dry wall screws of size 3.5 x25mm at 230mm c/c, including jointing & finishing to a flush finish of tapered and square edges of the board with recommended jointing compounds, jointing tapes, finishing with jointing compounds in three layers covering up to 150mm on both sides of joints and two coats of primer suitable for boards, all as per manufacture's specification and also including the cost of making opening for light fittings, grills, diffusers, cut outs made with frame of perimeter channels suitably fixed, all complete as per drawings, specification and direction of the Engineer in charge but excluding the cost of painting with 12.5 mm thick tapered edge gypsum moisture resistant board |
| --- | --- | --- |

| 3 | Writing Boards | Providing and supplying Steel Writing Board for writing purpose using marker pen. Medium Density Fiber (MDF) Board to be covered with electrogalvanized steel sheet conforming to IS: 277-2003(Reaffirmed 2007), Amdts.1&2, of 0.3 to 0.4 mm thickness on the front and with sheet of 0.25 mm to 0.03 mm thickness on the back side of MDF Board. Writing top surface of White Boards to receive e3 vitreous Enamelled coating of 0.11 mm min thickness on top. Galvanizing on All other surfaces of Sheet steel to be of 0.03 mm min. thickness. The top surface of Writing board shall be free from waviness and shall show no scratches when HB to 3H pencils are used for writing. The surface shall show excellent erase-ability when the specified writing medium is used. It should be possible to fully erase the marking of permanent marker pens using methanol, without adversely affecting the e3 vitreous coating in any manner. The core material shall be 9 mm thick MDF board having Bulk Density of 750 kg per cubic meter and Grade-1 as per IS: 12406- 2003 Edition 2.2. Both the top and the backing sheet shall be properly fixed with the MDF board using suitable adhesive with mechanical press to avoid any moisture absorption. The Writing board will be fixed to walls with the help of suitable size Stainless steel screws and wooden/PVC rawl plugs with teak wood Framing of 65 mm x 50 mm size on the periphery and 30 mm x 50 mm wide stiffeners on the backside. |
| --- | --- | --- |
| 4 | Tack Boards Acoustical Panels | Supply and Installation of Tack Board acoustical panels hardened square edge, FR grade NRC fabric (colour choice), wrapped on encapsulated glass fiber core panels made from size 600x1200x25mm, volume density 100-120Kgs/m3, weight 3.0kg/m2 installed by using Strut Impale Clip. Panels should have Acoustics – NRC 0.9 (For D50 Mounting), Fire Resistance Class 1(UK) as per manufacturer specifications. The Paneling shall be finished as per the drawings and to the satisfaction of the Engineer in Charge. All the support systems shall be as per the Approved make list shall be got approved before starting installation at site. |

| 5 | Tack Board Panels | Supplying and fixing Tack board in line and level as per architectural drawing and as directed by engineer in charge. Tack board made of Fabric: Fine Jute Fabric of Colour as approved during sampling on the front side. Thickness: of Jute Fabric 1±0.1 mm. The top surface should be smooth, finish, without any wrinkles, fungus proof, terminate proof & eye catching. The colour should not fade even if the Tack board is fixed in Direct sunlight position. Soft Board: Thickness: 12.00 mm. thick density of soft board should be 2.5 kg/m2. Linear expansion/ contraction in range 33% to 90% RH @ 200 C: % max. 0.5. The core material is soft board. The board should be soft, resilient, light coloured sheet material of approved make. Soft board with 12 mm thick beading on each side should be fixed to backside plywood with adhesive. Back Side plywood: The Backside of the Tack board to be supported with 12.00 mm thick marine plywood confirming to IS 710 of approved make. The plywood will be fixed to walls with the help of suitable size Stainless steel screws and wooden/PVC rowl plugs with teak wood support for levelling. Tack board with plywood to be fixed to wall/partition with suitable size SS Screws or equivalent Two-way adhesive tape or with adhesive in such a way that screws are not visible from the front side of Tack board as per drawing and as directed by engineer in charge. |
| --- | --- | --- |

|  |  | Providing & fixing premium quality Manually operated Roller blinds with Prestressed Micro-Aerated Solar protection fabric of high tenacity Polyester Yarn with PVC Coating & Fungistatic Treatment and should block the light flux & offer elimination of glare and sunlight. |
| --- | --- | --- |
|  |  | Fabric for Blinds: Composition: 40% Polyester / 60% PVC, Weight:400 g/m2 (+/-5%), Thickness:0.45 mm (+/-5%), Fabric Openness Factor: 3% to 4%, Tensile Strength (Warp/Weft): 220/220 daN / 5cm, Tearing Strength (Warp/Weft): 30/25 daN, Fire Retardancy:M2 (NFP92-507)- |
| 6 | Manually Operated Roller Blinds | Roller Tube shall be of extruded Aluminium alloy 38mm O.D (or as per system dimension) with a minimum wall thickness of 1.0mm duly anodized for long life. Clutch shall be wrap spring design with high strength fiberglass reinforced polyester assembly and high carbon steel springs to transmit motion from driving to driven members of clutch mechanism. Clutch shall operate by directionally with the use of an endless beaded chain. Clutch mechanism shall be crash proof, prevent slippage, and shall raise and lower smoothly to any desired height. Clutch shall never need adjustment. Idler shall be of high strength fiberglass reinforced polyester, consisting of an outside sleeve and center shaft. Sleeve shall provide bearing surface for roller tube and rotate freely on center shaft, providing smooth, quiet, and long wearing operation. Brackets shall be of atomized steel powder coated to give superior finish. Bracket shall accommodate overhead, side or face mounting with clutch assembly on either end of the roller. Bottom of the blind shall be provided with aluminium tube powder coated in a colour matching to the fabric. The fabric shall be enclosed in the suitably created pocket along with the tube. The tube shall be closed from sides with end caps to give a neat look. |

|  |  | Providing and fixing Motor for Motor |
| --- | --- | --- |
|  |  | operated blinds with remote (including |
|  |  | Remote) over manually operated blinds of |
|  |  | above items. |
|  |  | Motor for the motorized Operation of the |
|  |  | blinds to confirm the following |
|  |  | requirements: Rated Torque 6NM, RPM 28, |
|  |  | Protection IP 44 and above, Rated Voltage |
|  |  | 230 mm with suitable Remote to operate up |
|  |  | to 4 nos. Motorized blind in a single room |
|  |  | The clutch is of wrap spring design. High |
|  |  | carbon steel springs are provided to |
|  |  | transmit motion from driving to driven |
|  |  | members of clutch mechanism. Clutch is |
|  |  | operated directionally by the use of Remote |
|  |  | Transmission Service Type Motor with |
|  |  | Remote control. Clutch never needs any |
|  |  | adjustment. The Idler is of high strength |
|  |  | reinforced plastic, consisting of a center |
|  |  | shaft. The idler is mounted on the heavy |
|  |  | quality bracket (to Take the load of Blinds, |
|  |  | Idler and the Motor Mechanism) by using a |
|  |  | plastic lock. The metal sleeve inside the |
|  |  | plastic lock provides bearing surface for the |
| 7 | Motor Operated Roller Blinds | center shaft, which rotates freely, providing smooth, quiet and long wearing operation in |
|  |  | a roller tube of 32mm (OD) (depending |
|  |  | upon the size) aluminium extruded grooved |
|  |  | tube made of alloy T6063 weighing |
|  |  | 0.27kg/m. Control unit and Idler are fixed |
|  |  | to the either ends of the tube comprising |
|  |  | Roller Headrail which is made of aluminium |
|  |  | extruded rail Power coated in white. Control |
|  |  | unit and Idler are mounted on the Head rail |
|  |  | with powder coated M.S. brackets The fabric |
|  |  | of the blind is fitted to the tube with heavy |
|  |  | duty adhesive tape. Bottom rail is an |
|  |  | aluminium extruded rail with a groove for |
|  |  | fixing the fabric and powder coated in white. |
|  |  | It is fixed to the bottom of the fabric and |
|  |  | the purpose of it is, to keep the fabric in |
|  |  | tension and straight. The fabric is fitted to |
|  |  | the tube using plastic insert. The end caps |
|  |  | of the tube are ABS. The size of rail shall be |
|  |  | 21mm (OD) made of Alloy HE 9 WP |
|  |  | weighing 0.17kg/m. The fabric shall be |
|  |  | attached to the roller tube with high quality |
|  |  | self-adhesive tape. Operational action shall |
|  |  | be smooth and up to the satisfaction and as |
|  |  | per direction of EIC. |

| 8 | Steel Work | Steel work welded in built up sections/ framed work, including cutting, hoisting, fixing in position and applying a priming coat of approved steel primer using structural steel etc. as required. In gratings, frames, guard bar, ladder, railings, brackets, gates, and similar works. |
| --- | --- | --- |
|  |  | Providing and fixing aluminium work for |
|  |  | doors, windows, Shutters, ventilators, and |
|  |  | partitions with extruded built up standard |
|  |  | tubular sections/ appropriate Z sections and |
|  |  | other sections of approved make |
|  |  | conforming to IS: 733 and IS: 1285, fixing |
|  |  | with dash fasteners of required dia and size, |
| 9 | Aluminium Works for shutters of doors, windows, Furniture & ventilators including providing and fixing hinges/ pivots and making provision for fixing of fittings wherever required including EPDM rubber / neoprene gasket as per requirement | including necessary filling up the gaps at junctions, i.e., at top, bottom and sides with required EPDM rubber/ neoprene gasket etc. Aluminium sections shall be smooth, rust free, straight, mitered and jointed mechanically wherever required including cleat angle, Aluminium snap beading for glazing / paneling, C.P. brass / stainless steel screws, all complete as per approved architectural drawings and the directions of Engineer-in-charge. (Glazing and paneling paid separately). Powder coated aluminium (minimum thickness of powder coating 50 |
|  |  | micron) |
|  |  | [For shutters of doors, windows, Furniture & |
|  |  | ventilators including providing and fixing |
|  |  | hinges/ pivots and making provision for |
|  |  | fixing of fittings wherever required including |
|  |  | the cost of EPDM rubber / neoprene gasket |
|  |  | required.] |

| 10 | Aluminium Works for Internal Partition Walls | Providing and fixing aluminium work for doors, windows, Shutters, ventilators and partitions with extruded built up standard tubular sections/ appropriate Z sections and other sections of approved make conforming to IS: 733 and IS: 1285, fixing with dash fasteners of required dia and size, including necessary filling up the gaps at junctions, i.e. at top, bottom and sides with required EPDM rubber/ neoprene gasket etc. Aluminium sections shall be smooth, rust free, straight, mitered and jointed mechanically wherever required including cleat angle, Aluminium snap beading for glazing / paneling, C.P. brass / stainless steel screws, all complete as per architectural drawings and the directions of Engineer-in-charge. (Glazing and paneling paid separately). Powder coated aluminium (minimum thickness of powder coating 50 micron) |
| --- | --- | --- |
| 11 | Woodwork in frames of Doors & Windows | Providing and fixing of woodwork in frames of doors, windows, clerestory windows, partitions, and other frames, wrought framed and fixed in position with hold fast lugs or with dash fasteners of required dia. & length in Second class teak wood. |
| 12 | Woodwork in frames of Partitions with Glass/Wooden panels/Plywood Panels/gypsum board. | Providing woodwork in frames Wooden members are to be fixed to cover the M.S./Aluminium sections of the main partition frames from all sides as per the detailed drawings including fixing the wooden members with necessary screws, nuts, bolts etc. Wooden members of Shutters frame if any and wooden beadings for fixing Glass panels, wooden panels, plywood panels of the partition with wooden members of the frames will also be paid under this same item. The work to be carried out as per the detailed drawings and as directed by Engineer In charge in Second class teak wood. |
| 13 | Wooden Doors/ Window shutters | Providing and fixing paneled or paneled and glazed shutters for doors, windows and clerestory windows fixing with butt hinges of required size with necessary screws, excluding paneling which will be paid for separately, all complete as per direction of Engineer-in-charge. (Note: - Butt hinges and necessary screws shall be paid separately) – in Second class teak wood 35 mm thick shutters. |

| 14 | Glazing of Doors / Window Panels | Providing and fixing paneling or paneling and glazing in paneled or paneled and glazed shutters for doors, windows, and clerestory windows (Area of opening for panel inserts excluding portion inside grooves or rebates to be measured). Paneling for paneled or paneled and glazed shutters 25 mm to 40 mm thick: - in Second class teak wood. - Fly proof stainless steel grade 304 wire gauge with 0.5 mm dia. wire and 1.4 mm wide aperture with matching wood beading |
| --- | --- | --- |
| 15 | Flush Door Shutters | Providing and fixing ISI marked flush door shutters conforming to IS : 2202 (Part I) non-decorative type, core of block board construction with frame of 1st class hard wood and well matched commercial 3 ply veneering with vertical grains or cross bands and face veneers on both faces of shutters, lipping with 2nd class teak wood/saguwan wood battens 25 mm minimum depth on all edges of flush door shutters : 35 mm thick excluding ISI marked Stainless Steel butt hinges with necessary screws. |
| 16 | Plywood – 25mm Thick | Providing and fixing 25 mm thick calibrated BWP plywood confirming to IS 710 with necessary fixing arrangement including adhesive, nails, screws etc. all complete, as per approved drawings and as per direction of engineer in charge. For wooden panelling, partitions, wardrobes, cabinets, band, patta, benches, Tables, modesty panels etc. |
| 17 | Plywood – 18mm Thick | Providing and fixing 18 mm thick calibrated BWP plywood confirming to IS 710 with necessary fixing arrangement including adhesive, nails, screws etc. all complete, as per approved drawings and as per direction of engineer in charge. For wooden panelling, partitions, wardrobes, cabinets, band, patta, benches, Tables, modesty panels etc. |

| 18 | Plywood – 12mm Thick | Providing and fixing 12 mm thick calibrated BWP plywood confirming to IS 710 with necessary fixing arrangement including adhesive, nails, screws etc. all complete, as per approved drawings and as per direction of engineer in charge. For wooden panelling, partitions, wardrobes, cabinets, band, patta, benches, Tables, modesty panels etc. |
| --- | --- | --- |
| 19 | Plywood – 8mm Thick | Providing and fixing 8 mm thick exterior grade non asbestos heavy duty fiber cement board with necessary fixing arrangement and screws etc. all complete, as per approved drawings and as per direction of engineer in charge. |
| 20 | Plywood – 6mm Thick | Providing and fixing 6 mm thick exterior grade non asbestos heavy duty fiber cement board with necessary fixing arrangement and screws etc. all complete, as per approved drawings and as per direction of engineer in charge. |
| 21 | PVC edge beading | Providing and fixing PVC edge beading 2 mm thick / second class teakwood beading 12 mm thick with necessary fixing arrangement and screws etc. all complete, as per approved drawings and as per direction of engineer in charge for various thickness of plywood for wardrobes, cabinets, partitions etc.. |
| 22 | Wall bands for services with plywood & veneer | Providing & fixing of wall band up to 300 mm wide made from 12 mm thick marine plywood conforming to IS: 710 finishing with 4 mm veneer and second-class teak wood beading, back side supporting second class teakwood Patti 48 X 48 mm top and bottom with necessary fixing arrangement and screws etc. all complete, as per drawing and as directed by EIC. |
| 23 | Wall bands for services with MDF Board | Providing & fixing to wall band up to 300 mm wide made from exterior Grade-I MDF Board 18 mm thick confirming to IS:12406 with second class teak wood beading and back side supporting second class teakwood patti 48x48 mm top and bottom with necessary fixing arrangement and screws etc. all complete as per drawing and as directed by EIC. |

| 24 | Pelmets | Providing & fixing C shaped pelmet up to 200 mm made wide from 18 mm thick marine plywood conforming to IS: 710 finishing front side with 4 mm thick veneer and second class teak wood beading, back side supporting to wall with necessary fixing arrangement and screws etc. all complete, as per drawing and as directed by engineer- in- charge. |
| --- | --- | --- |
| 25 | High pressure laminate wherever required as per approved drawings/design or as per the directions of the Engineer-In-Charge | Providing & fixing decorative high pressure laminated sheet of plain / wood grain in gloss / matt / suede finish with high density protective surface layer and reverse side of adhesive bonding quality conforming to IS: 2046 Type S, including cost of adhesive of approved quality. a) 1.0 mm thick on one side only |
| 26 | Veneer wherever applicable | Providing and fixing 4 mm thick divine teak quarter cut Veneer conforming to IS: 1328 (type-1) with second class teak wood beading of approved colour, texture sample as approved and as per direction of Engineer-In-Charge (EIC). |
| 27 | Wooden grill for FCU Cabinets/ HVAC Ducts | Providing and fixing Wooden Grill louvers made from second class teak wooden Patti 40 x12 mm at center-to-center distance of 40 mm fixed to 50x20 mm wooden frame with necessary fixing arrangement and screws etc. all complete, at required location in frames/shutters of door/windows as per drawing and as directed by engineer in charge. |
| 28 | Glazing in Doors/Windows/Intern al Partitions | Providing and fixing glazing in door, window, ventilator shutters and partitions etc. with EPDM rubber / neoprene gasket /teakwood beading etc. complete as per the architectural drawings and the directions of EIC. - With toughened glass panes of 8.0 mm thickness - With toughened glass panes of 5.0 to 6.0 mm thickness - With Toughened glass panes of 12.0 mm thickness |
| 29 | PVC Acoustic & Smoke Seal | Providing and fixing PVC Acoustic & smoke seal to protect rebated timber frame 12 mm profile RP 120 Raven make or equivalent as per drawing and as directed by engineer in charge. |

| 30 | Self-adhesive for gaps | Providing and fixing EPDM perimeter Bulb type of seal self -adhesive for Gaps as per drawing and as directed by EIC. |
| --- | --- | --- |
| 31 | Elite brush seal sweeper gasket | Providing and fixing expose type Elite brush seal sweeper gasket Enviro make or equivalent as per drawing and as directed by EIC. |
| 32 | Automatic door bottom seal | Providing and fixing acoustical Automatic door bottom seal RP8si seal of raven make or equivalent as per drawing and as directed by engineer in charge. |
| 33 | Mirrors | Providing and fixing 5 mm thick mirror of superior glass (of approved quality) and of required shape and size fixed with second class teakwood beading or with adhesive or with two-way tape (3M or equivalent Quality) on to with 6 mm thick hard board ground fixed to wooden cleats with C.P. brass screws and washers complete as per the architectural drawings and the directions of Engineer-in-charge. |
| 34 | Door Closer | Providing and fixing Door Closer made of High Quality aluminium extruded section body tubular type universal hydraulic door closer (having brand logo with ISI, IS: 3564, embossed on the body, door weight unto 80 kg , with double speed adjustment, Rack and pinion should be manufactured from steel alloy duly machined and heat- treated, suitable for right-hand and left hand door application, with necessary accessories and screws etc. complete as per drawing and as directed by EIC. |
| 35 | Ball bearing Hinges | Providing & Fixing S.S. 316 Grade Fire resistance BSEN 1634:1:2000 Certified & Mechanically tested BSEN 1935:2002 Satin Finish S.S. 316 Grade 4 ball bearings Hinges approximate size 102mm X 76mm X 3 mm minimum Weight 260 grm of approved make with AISI 316 Grade ball Bearing S.S. Pin, Cap, and with necessary S.S. Self Tapping Phillips Cross Head Screws etc. complete as per approved drawing and as directed by EIC. |

| 36 | Hinges without bearings | Providing & Fixing S.S. 316 Grade Satin Finish Fire resistance BSEN 1634:1:2000 Certified & Mechanically tested BSEN 1935:2002 Hinges without bearings approximate size 76mm X 65mm X 2 mm minimum weight 100 grm of approved make with AISI 316 Grade S.S. Pin, Cap, and with necessary S.S. Self Tapping Phillips Cross Head Screws etc. complete as specified as per drawing and as directed by engineer-in -charge. |
| --- | --- | --- |
| 37 | Tower Bolts 300mm | Providing & Fixing S.S. 304 Grade Stainless Steel Satin Finish Round Tower bolt of overall length (excluding Bracket) 300 mm and inner bolt of dia meter 10mm and outer barrel dia meter 15 mm, minimum weight 430 grm. of approved make with necessary Nickel-Plated Screws complete as per specified as per drawing and directed by Engineer in charge. |
| 38 | Mortise Handle | Providing & Fixing Fire resistance BSEN 1634:1:2000 Certified & mechanically tested BSEN 1906:2010 Pair of AISI S.S. 316 Grade Satin Finish Hollow Pipe or solid design Mortise Handle with S.S. 316 Grade euro profile escutcheons keyhole for Mortise Pin Cylinder, high grade brass bushing for extra fixing strength for intensive use of door with back-to-back fixing screws system. of approved make with both side active mortise handle and spindle, High- Quality Stainless-Steel Wood Screws (8 PCS.) for minimum door thickness 30 mm as per drawing and as directed by engineer in charge. The Inner and Outer Rose of Mortise handle and Escutcheons must be of AISI 316 grade only. |
| 39 | Double Door Lock Body | Providing & Fixing Satin Finish Double Door Lock body, 26 mm brass latch. 52 mm lock with Back Set center of approximate size 85 X 45 mm of approved make, minimum Weight 0.880 grm suitable for minimum 30 mm thick Double Door Shutter with necessary fixing screw as specified as per drawing and as per directed by EIC. |

| 40 | Single Door Lock Body | Providing & Fixing Fire resistance BSEN 1634:1:2000 Certified Satin Finish Single Door Mortise Lock body approximate size of back set 50 mm X 85 mm, with 52 mm Lock and 26 mm brass latch, Stainless steel main & Strike plate & including back to back fixing feature suitable for minimum door thickness 30 mm Single door shutter with necessary fixing screw as per specified as per directed by EIC. |
| --- | --- | --- |
| 41 | Mortise Pin Cylinder | Providing & Fixing Satin Finish 5 pin Mechanism, high Quality brass body Mortise Pin Cylinder with 5 high accuracy Computerized Dotted keys of approved make one side key & one side knob suitable for minimum door thickness 30 to 40 mm with necessary Fixing Screw as specified and as per drawing and as per directed by engineer in charge. |
| 42 | Mortise Pin Cylinder with both side key | Providing & Fixing Fire resistance BSEN 1634:1:2000 Certified Satin Finish Secure Standard 5 pin Mechanism Mortise Pin Cylinder with both Side Keys, with 5 nos high accuracy Brass keys of approved make suitable for minimum door thickness 30 to 40 mm with necessary fixing screw as per specified and as per drawing and as per directed by engineer in charge. |
| 43 | Floor/Wall mounted magnetic door closer | Providing & Fixing Satin Finish Magnetic Door Catcher mounted on floor or wall to stay the door of approved make, minimum Weight 150 grm with necessary nickel- plated screw complete, as specified, as per drawing and as directed by engineer in charge. |
| 44 | Door Stop | Providing & Fixing high Quality Zinc Material wall mounted Door Stop length of 75 to 100 mm including Rubber of approved make minimum Weight 260 grm with necessary Screws etc. complete as specified, as per drawing as directed by EIC. |
| 45 | C-Shaped Handles | Providing & Fixing Stainless Steel S.S. 316 Grade C Shaped Handles in Satin Finish, 10 mm dia and center to center minimum 96mm size, minimum weight 94 grm with necessary screws etc. complete of approved make as per drawing and as directed by EIC. |

| 46 | Round Tower Bolt 100mm | Providing & Fixing S.S. 304 Grade Stainless Steel Satin Finish Round Tower bolt of overall length (excluding Bracket) 100 mm and inner bolt of dia meter 10mm and outer barrel dia meter 15 mm, minimum weight 150 grm of approved make with necessary nickel-Plated Screws complete as per specified as per drawing and directed by EIC. |
| --- | --- | --- |
| 47 | Window Stays | Providing and fixing stainless steel (SS 304 grade) adjustable friction windows stays of approved quality with necessary stainless- steel screws etc. to the side hung windows as per direction of Engineerin- charge complete. Size: 255 X 19 mm |
| 48 | Soft closing clip on hinges | Providing and fixing Satin Finish soft closing Clip on hinges of M.S. materials, Opening Angle should be 110 Degree or as required of Approved makes for cupboard shutters, kitchen cabinets etc. with necessary concealed SS Screws complete, as specified in drawing and as approved by Engineer in charge. |
| 49 | Multipurpose Cabinet /Wardrobe Lock | Providing and Fixing Stainless Steel S.S. 304 Grade multipurpose Cabinet / Wardrobe Lock heavy duty for minimum 25 mm Door thickness with keys of approved make with necessary screws etc. complete as approved drawing and as per direction of Engineer in charge. |
| 50 | Multipurpose Drawer Locks | Providing and Fixing Stainless Steel Multipurpose Drawer Lock heavy duty with key of approved make with necessary screws etc. complete as approved drawing and as per direction of Engineer in charge. |
| 51 | Drawer Channel | Providing and fixing Corrosion Resistance Premium Quality Zinc coated Drawer Channel pair 500mm long, heavy duty Metal Body & Precision Ball Bearings for higher Durability with necessary screws etc. complete as per directions of Engineer-in- charge. |
| 52 | Grip profile handle | Providing and fixing aluminum Grip profile handle with necessary screws, adhesive, etc. complete as approved drawing and as per direction of Engineer in charge. |

| 53 | D Shape pull handle | Providing & Fixing Fire Resistance BSEN 1634:1:2000 Certified S.S. 316 Grade Satin Finish 1.5 mm thick Hollow Pipe 'D' Shape Pull Handle with dia. 32 mm and centre to centre distance 600mm high Grade Brass Bushing should be for Extra Fixing strength including necessary fixing Accessories of Approved make complete as specified as per drawing and as per directed by engineer in charge. |
| --- | --- | --- |
| 54 | Magnetic catcher for wardrobe | Providing and fixing Magnet Catcher for wardrobe/ cabinet shutters of approved make with necessary screws etc. complete as approved drawing and as per direction of Engineer in charge. |
| 55 | Sliding Top & Bottom Track set | Providing and fixing sliding Top & Bottom Track set of approved make suitable for wooden sliding door of total weight up to 100 kg. with all hardware as per architectural drawing and as directed by engineer in charge. ( Including Top Roller ‐,Bottom Roller ‐,Heavy duty aluminium top track profile of suitable size for the door up to Rigid aluminium flush bottom track profile 25x19x3;conceled bottom guide 100x32x3;concealed intermediate hanger 100x32x3 with roller made of wood pile or Nylon; end bottom guide 100x32x3;aluminium drop bolt 200x36x2 spender dia 10 mm; end carrier set 100x32x3; etc.as per manufacturers specifications and as per approval of Engineer-In-Charge- One Complete set for one door of Minimum 1.5 meter width. Number of rollers and rail length to be provided accordingly. |
| 56 | Patch Lock | Providing & Fixing Satin Finish Patch Lock of High-Quality Aluminium body including Euro Profile Pin Cylinder of Brass, Spring-Loaded Push Pins for easy installation & Firm Fittings with S.S. 304 Grade Cover, Suitable for Glass Door, Maximum Door Weight 120 Kg. & Door Width up to 1200 mm of Approved make with necessary fixing Accessories complete as per specified and as per drawing and directed by engineer in charge. |

| 57 | Satin Finish Dead Bolt | Providing & Fixing Fire resistance BSEN 1634:1:2000 Certified Satin Finish Dead Bolt with back set approximate size of 50 mm and Bolt part should be high Quality Zinc with size 52 mmX 10 mm, minimum Weight should be approximate 441 grm. including 2.5 mm thick Stainless Steel main & 1.2 mm thick Strike Plate of Approved make suitable for minimum 30 mm thick Single door Shutter with necessary fixing screws as specified as per drawing and as directed by engineer in charge. |
| --- | --- | --- |
| 58 | Dash fastener | Providing and fixing carbon steel galvanized (minimum coating 5 micron) dash fastener of 10 mm dia double threaded 6.8 grade (yield strength 480 N/mm²), counter sunk head, comprising of 10 m dia polyamide PA 6 grade sleeve, including drilling of hole in frame, concrete/ masonry, etc. as per direction of Engineer-in-charge. Size: 10 x 80 mm |
| 59 | Silicon sealant filling | Filling the gap in between frame & adjacent RCC/ Brick/ Stonework by providing weather silicon sealant over backer rod of approved quality as per architectural drawings and direction of Engineer-in- charge complete. Up to 5mm depth and 5 mm width |
| 60 | White cement-based putty | Providing and applying white cement-based putty of average thickness 1 mm, of approved brand and manufacturer, over the plastered/Gypsum or any other wall/ ceiling surface as required to prepare the surface even and smooth complete. |
| 61 | Primer on woodwork | Applying priming coats with primer of approved brand and manufacture, having low VOC (Volatile Organic Compound) content with ready mixed pink or grey primer on woodwork (hard and soft wood) having VOC content less than 50 grams/ liter. |
| 62 | Cement primer on wall | Applying priming coats with primer of approved brand and manufacture, having low VOC (Volatile Organic Compound) content with water thinnable cement primer on wall, ceiling and partitions surface having VOC content less than 50 grams/liter |

| 63 | Premium acrylic emulsion paint of interior grade | Painting with premium acrylic emulsion paint of interior grade, having VOC (Volatile Organic Compound) content less than 50 grams/ liter of approved brand and manufacture, including applying additional coats wherever required to achieve even shade and colour in Two coats. |
| --- | --- | --- |
| 64 | Epoxy paint on Steelwork | Finishing with Epoxy paint (two or more coats) at all locations prepared and applied as per manufacturer's specifications including appropriate priming coat, preparation of surface, etc. complete on steel work. |
| 65 | Fire-retardant paint on cleaned wood / ply / steel surface | Providing and applying two coats of fire- retardant paint on cleaned wood / ply / steel surface @ 3.5 sqm per liter per coat including preparation of base surface as per recommendations of manufacturer to make the surface fire retardant. |
| 66 | Melamine polishing on wood/veneer | Providing and applying Melamine polishing of approved makes on wood & veneer works (two or more coats) including finishing grooves with paint/polish, preparation of surface and staining to the approved colour and shade as per the manufacturer’s specifications including scaffolding, curing, cleaning the surfaces and other incidental work to be done etc. complete at all floors for any height as directed by EIC. |
| 67 | Varnishing | Varnishing with varnish of approved brand and manufacture: Two or more coats of glue sizing with copal varnish over an under coat of flatting varnish |

|  |  | Providing and Fixing of Flocked Carpet rolls |
| --- | --- | --- |
|  |  | flooring having a density of 70 million fibers |
|  |  | of nylons 6.6 per sqm firmly anchored into a |
|  |  | waterproof backing and having an average |
|  |  | recycled content of 20%. The carpet must |
|  |  | inhibit the growth of Bacteria and Fungi. |
|  |  | The Carpet must be Zero Emission carpet |
|  |  | (Emission below detection limit after 28 |
|  |  | days in accordance with ISO 16000-9 |
|  |  | requirements). Fire Test EN-13501, |
|  |  | Appearance Retention Hexapod ISO 140-8, |
|  |  | Friction Slip resistance Test EN 14041 Class |
|  |  | DS, Sanitized anti- microbial treatment with |
|  |  | resilient waterproof backing. The carpet |
| 68 | Flocked Carpet rolls flooring | should be anti-static and thickness shall be 4.0 to 5.0 mm with approximate weight of 1.8 kg/ sqm. Dimensional stability as per |
|  |  | ISO 2551: <0.2%. Slip Resistance as per |
|  |  | DIN 51097: >0.7 Dry and Wet. Light |
|  |  | fastness as per EN ISO 105 B02: ≥ 6. |
|  |  | Commercial heavy as per EN 685: Class 33. |
|  |  | Acoustical Impact Sound Absorption EN-ISO |
|  |  | 717-2: 20 dB. Reaction to fire as per EN |
|  |  | 13501-1: Bfl-s1. |
|  |  | The rate shall be inclusive of fixing at site |
|  |  | with, if necessary, border and fixing |
|  |  | arrangement as per the drawings and |
|  |  | direction of Engineer in charge complete in |
|  |  | all respect. Flooring should be done as per |
|  |  | manufacturers specifications and as directed |
|  |  | by engineer in charge. |
|  |  | Providing and fixing Engineered wooden |
|  |  | flooring 15 mm up to 3 mm top layer, core |
|  |  | layer pine wood oak London of Mikasa or |
|  |  | equivalent with necessary T- profile and |
| 69 | Engineered wood flooring | reducer fixing with under layer 2 mm thick High Density polyurethane foam having |
|  |  | density of approx. 90 kg/cum and 2 mm |
|  |  | thick rubber sheet as per manufacturer’s |
|  |  | specifications and as directed by engineer in |
|  |  | charge. |
|  |  | Providing and fixing Engineered wooden |
|  |  | skirting 2400x80x20 mm up to 3 mm top |
|  |  | layer, core layer pine wood oak London of |
| 70 | Engineered Wooden Skirting | Mikasa or equivalent with necessary T- profile and reducer fixing with underlayer 2 mm thick High Density polyurethane foam |
|  |  | and 2 mm thick rubber sheet as per |
|  |  | manufacturer’s specifications and as |
|  |  | directed by engineer in charge. |

| 71 | Kota Stone slab flooring | 25 mm thick Kota stone slab flooring over 20 mm (average) thick base laid over and jointed with grey cement slurry mixed with pigment to match the shade of the slab, including rubbing and polishing complete with base of cement mortar 1: 4 (1 cement: 4 coarse sand) |
| --- | --- | --- |
| 72 | Vitrified tiles flooring | Providing and laying vitrified floor tiles in different sizes (thickness to be specified by the manufacturer) with water absorption less than 0.08% and conforming to IS: 15622, of approved make, in all colours and shades, laid on 20mm thick cement mortar 1:4 (1 cement : 4 coarse sand), including grouting the joints with white cement and matching pigments etc., complete. Size of Tile 600x600 mm |
| 73 | Signage plate for cabin/classroom/meeti ng rooms | Supply and installation of Signage plate for Cabin/Classroom/Meeting Rooms Names of Size 300 mm x 120mm x 1.5 mm in SS 304 Matt finish having Letters engraved in the SS plate in Black vinyl steam pressed to get a clean finish and having font size of name at least 20 mm and font size of designation 16 mm fixed on to the wall or Door surface with high quality adhesive tape of 3M or equivalent complete to the satisfaction of Engineer In Charge. |
| 74 | Signage plate for toilets | Supply and installation of Signage plate for Toilets (Male/Female/Divyang/Pantry) of Size 120 mm x 120mm x 1.5 mm in SS 304 Matt finish having Letters engraved in the SS plate in Black vinyl steam pressed to get a clean finish and having font size of name at least 30 mm fixed on to the wall or Door surface with high quality adhesive tape of 3M or equivalent complete to the satisfaction of Engineer In Charge. |
| 75 | FIRE EXIT SIGNAGE | Supply and installation of Signage plate for FIRE EXIT SIGNAGE of Size 300 mm x 120mm x 1.5 mm in SS 304 Matt finish having Letters engraved in the SS plate in Black vinyl steam pressed to get a clean finish and having font size of name at least 40 mm fixed on to the wall or Door surface with high quality adhesive tape of 3M or equivalent complete to the satisfaction of Engineer In Charge. |

| 76 | Directional Signage Plates | Supply and installation of Directional Signage plate (Arrows) of Size 120 mm x 120mm x 1.5 mm in SS 304 Matt finish having Letters engraved in the SS plate in Black vinyl steam pressed to get a clean finish and having Arrow Size 100 mm x 75 mm fixed on to the wall or Door surface with high quality adhesive tape of 3M or equivalent complete to the satisfaction of Engineer In Charge. |
| --- | --- | --- |
| 77 | Aluminium profile shutters for overhead cabinets | Supplying and Fixing aluminium profile shutters for overhead cabinets including finishing of the shutters frame with powder coated finish of minimum 50 micron of colour as per the Architects selections. The Shutter profile shall have in built handle provision for the bottom shutter profile as per Architect selection, design, and drawing. The rate shall be inclusive of all the cleats, screws, EPDM gaskets, Silicone Sealants, fabrication as per drawing, fixing at site etc. required for the shutter. (Soft Close hinges and Glass infill shall be paid separately). |
| 78 | 12mm back painted glass | Suppling and fixing of approved quality back painted glass with 12 mm thick glass for horizontal and vertical surfaces from Approved make list painted with high quality low VOC paint to provide a uniform and glossy finish to the painted surface when seen through the glass. The finish and the paint colour shall be as per selection of Architect. The back painted glass shall be prepared using approved quality paint and shall be sprayed on to the glass and baked as per manufacturers specifications to get proper finish and quality of the back painted glass. Rate shall be inclusive of fixing of the glass on the wall panelling or on to the shutters as per the design and sizes mentioned in the drawing. |

| 79 | 6mm back painted glass | Suppling and fixing of Approved quality back painted glass with Glass 6 mm thick for cabinet shutters from Approved make list painted with high quality low VOC paint to provide a uniform and glossy finish to the painted surface when seen through the glass. The finish and the paint colour shall be as per selection of Architect. The back painted glass shall be prepared using approved quality paint and shall be sprayed on to the glass and baked as per manufacturers specifications to get proper finish and quality of the back painted glass. Rate shall be inclusive of fixing of the glass on the wall panelling or on to the shutters as per the design and sizes mentioned in the drawing. |
| --- | --- | --- |
| 80 | Corian Slabs for tabletops | Supplying and fixing of 12 mm thick Corian slab in single piece on tabletop including fixing the same on the table using approved quality adhesive including chamfering all the edges of the tabletop to half round or tapered edge as per the Architects directions and to the satisfaction of the Architect/ Engineer IN Charge. The rate shall be inclusive of all the tools and tackles and other accessories required for the same. |
| 81 | Aluminium tubular handlebar | Providing and fixing aluminium tubular handlebar 32 mm outer dia, 3.0 mm thick & 2100 mm long with SS screws etc. complete as per direction of Engineer-in-Charge. Powder coating minimum thickness 50- micron aluminium tubular handlebar. |
| 82 | Acoustic movable partition with fabric finish | Providing and Fixing of Acoustic Movable partition with Fabric Finish of approved make having thickness of 100 mm with minimum STC value of 44 or above having internal built consisting of Retractable Mechanism to move Top and Bottom Seal Manually Operated by a Handle inserted in the Panel Stile, Rotating the handle clockwise / anticlockwise shall simultaneously Extend or Retract the Top and Bottom Seals through gear Mechanism which shall firmly hold the Seals in the Retracted or Extended mode. The panel Trims will be standard series with exposed Aluminium trims of 15mm thickness finished in 6063 - T5 Anodized Aluminium shall be used. Each panel will have top and bottom Rubber Retractable seal with Concave / Convex Vertical Interlocking profiles. Vinyl |

|  |  | Seal of Butane Rubber will be used at top and bottom and as well as Closure Seals for the Operable wall system. Retractable seal drop will be 20 mm. Each Panel shall be suspended on Extruded 6063 - T5 Anodized Aluminium Track, Steel Cold Ball Bearing Rollers / Trolley of material nylon wheel made with the base Material of POM 45 tempered steel, maintenance free with multiple Ball bearing. The individual element is adjustable in height without opening of the ceiling. 100 mm thick Panels will be made from 12mm MDF / Clipboard Panels hung in Acoustic Isolation from the frame and allowed to vibrate independently of the frame. The Panels must be filled with Acoustic material Rockwool of 64Kg/m3 density & Sound Damping layer of 2 mm thick high-density material to achieve Sound STC of 44 dbA as per site condition. The width of panel will be from 800 mm to 1200 mm maximum, as per site condition. The panel faces shall be finished with selected material as per University’s selection and colour. Partitions shall be top supported, manually operated panels stacked in an area nominated in the plans. When closed, the operable wall shall provide a complete sound retardant barrier. No floor track shall be used. The panels will be stacked as per the agreed method of One Side Stacking OR two Side Stacking in accordance with the approved architectural plan. The Required MS Framing work for the movable partition is included in the cost. The Contractor shall submit shop drawings for the movable partition for approval before commencement of work at site. |
| --- | --- | --- |
| 83 | Open cell GI lay-in ceiling tiles | Providing and fixing horizontal level Armstrong Cellio or equivalent Open cell GI lay-in ceiling tiles with bevelled tegular edges with border panels forming board edge of size 600x600x38mm (Nominal) having cell size of 150x150mm made of 120 gsm galvanized steel as per IS 277 (2003) suitable f or architectural finishes in 0.35mm thickness as per in IS:513 (1994) in Global white colour. The tile shall be electrostatically poly ester powder coated using RoHS compliant powder having coating thickness ranging from 60-80 microns. The powder coating shall pass Salt Spray Test f or 750 hours as per ASTM B117, QUV –A Test f or 1000 hours as per |

|  |  | ASTM G-154 – (Cycle – 1) and Humidity test f or 1000 hours as per DIN 50017. The powder coating shall have an adhesion of GT0 as per EN ISO 2409. The shade variation across batches shall be delta E<1 using Excite/BYK Gardner instrument and gloss level of 25±5% at 60° as per ASTM D523. Tile shall have a Fire Performance class A1 as per EN- 13964. SUSPENSION: The tile shall be laid on approved suspension system with 15 mm wide T - section flanges colour global white (matching to tile colour) having double stroke rotary stitching on all T sections i.e., the Main Runner, 1200 mm & 600 mm Cross Tees with a web height of 38mm (all sections). The def lection loading of the system shall be 12.5 kg/m2 (as per standard installation lay out mentioned below) and main beam tested as per ASTM C635 (Def lection limit less than L/360) with a def lection loading of 15kg/m. The end details of the cross tee shall be made of pre hardened steel clip fixed to the ends of the cross tee to provide double locking between “cross tee to cross tee” and “cross tee to main beam”. All main beam to main beam and cross tee to cross tee connection shall have a pull-out strength of more than 100kg. The T Sections shall be made of hot dipped galvanized steel 90 gsm and pre- painted steel with baked poly ester paint of 0T bending capability. The Tile & Grid system used together shall carry a 10 year warranty against manufacturing defect and manufacturer shall provide a warranty certificate. The Tile & Grid shall have the manufacturers name embossed/printed on it. The supply shall be backed by a Manufacturing Test Certificate. INSTALLATION: To comprise main runner spaced at 1200mm securely fixed to the structural soffit using approved suspension system (specifications below) at 1200mm maximum. The First/Last suspension system at the end of each main runner shall not be greater than 450mm from the adjacent wall along with the first and the last main beam shall be at less than 600mm from the wall. Flush fitting 1200mm long cross tees to be interlocked between main runners at 600mm to form 1200 x 600 mm module. Cut cross tees longer than 600mm require independent support. 600 x 600mm module to be formed by fitting 600mm long f lush |
| --- | --- | --- |

|  |  | fitting cross tees centrally between the 1200 mm cross tees. Perimeter trim to be Armstrong wall angles of size 3000x19x19mm, secured to walls using screws at 450mm centre to centre maximum. All borders cut tiles shall be laid, using lay-in end cap for installation to have a Micro look effect on perimeters. Installation shall be carried out as per Armstrong recommended procedure. SUSPENSION SYSTEM accessories manufactured and supplied by Armstrong consisting of M6 Anchor Fasteners with hanger hole, pre-straightened Hanger wire of dia – 2.5 mm of 1.80 m length having a tensile strength of 344-413 MPa and a minimum loading strength of 110 kgs. Additionally, adjustable hook clips made of 0.7mm thick Grade 1 C55 galvanized spring steel of dimension 100 x 20mm and 4 mm aquiline wire to be used for level adjustment during installation purpose in main runners. The contractor shall submit shop drawings for approval before proceeding for the work. |
| --- | --- | --- |
| 84 | CURTAIN WALL FRAMING SYSTEM (CW1) | Site assembled stick system complete with prefabricated insulated glazing units held in place with aluminium extruded restraint caps and finished with snap on cover. Mullion size - 75mmX100mm. Frame Finish (External): SDF Powder coating (To Meet Requirement of AAMA 2605) (Colour-TBC) Work includes aluminium extruded framework, assembly hardware, interface details such as cladding cover, flashings, insulation, sealants, brackets and any sub- frame as required. |
| 85 | FIXED VISION GLASS | Typical insulated vision glass [OUT]6mm HS with Double Low-e coating on surface 2 + 12mm Air Space + 6mm HS clear - Saint Gobain SKN II 476 Envision Plus |
| 86 | OPERABLE WINDOW SYSTEM | Aluminium extruded side hung operable system with subframe design adapted to the interfacing stick punch window system. Frame Finish (External): SDF Powder coating (To Meet Requirement of AAMA 2605) (Colour-TBC). Works to include operable hardware, locking arrangement and opening restraints as described in the technical specifications. |

| 87 | FRAMING SYSTEM AND EXTERIOR CLADDING | Open joint, rain screen cladding using 3mm solid aluminium sheet prefabricated cladding panels on site installed sub-frame. Cladding joints complete with insert channels to conceal visible fasteners. Frame Finish (External): SDF Powder coating (To Meet Requirement of AAMA 2605) (Colour- TBC). Aluminium solid panel finish (external): Frame Finish (External): SDF Powder coating (To Meet Requirement of AAMA 2605) (Colour-TBC) Works to include backing sub-frame, structural anchors, and interface seals. |
| --- | --- | --- |
| 88 | FRAMING SYSTEM AND INTERNAL CLADDING | Providing and fixing Second Class teak wood paneling within the framing system. Paneling for paneled surface 25mm to 40mm thick. |
| 89 | FINISHING/ POLISHING OF INTERNAL CLADDING | Providing and applying Melamine polishing of approved makes on wood & veneer works (two or more coats) including finishing grooves with paint/polish, preparation of surface and staining to the approved colour and shade as per the manufacturer’s specifications including scaffolding, curing, cleaning the surfaces and other incidental work to be done etc. complete at all floors for any height as directed by engineer in charge. |
| 90 | CURTAIN WALL FRAMING SYSTEM (For double height curtain walls (CW2) | Site assembled stick system fixed on site installed steel sub-frame (100mmX200mm) and 100mm wide aluminium extruded glazing adapters and cover complete with prefabricated insulated glazing units held in place with aluminium extruded restraint caps and finished with snap on cover - 100mmX 18mm. Frame Finish (External): SDF Powder coating (To Meet Requirement of AAMA 2605) (Colour-TBC) Work includes steel sub-frame, aluminium extruded adaptive framework assembly hardware, interface details such as cladding cover, flashings, insulation, sealants, brackets and any sub-frame as required. |
| 91 | FIXED VISION GLASS | Typical insulated vision glass [OUT]6mm HS with Double Low-e coating on surface 2 + 12mm Air Space + 6mm HS clear - Saint Gobain SKN II 476 Envision Plus. |
| 92 | OPERABLE SYSTEMS - DOORS | 12mm thick tempered clear glass on a frameless system fixed on stainless steel patch fittings with integrated floor springs. MATCHING FOR CW2 |

| 93 | 700 MM WIDE LINEAR SKYLIGHT AT ROOF LEVEL (SK1) | FRAMING FOR SKYLIGHT - Site assembled stick system fixed on site installed steel sub-frame (100mmX50mm) and aluminium extruded glazing adapters - 50 mm wide and cover complete with prefabricated insulated glazing units held in place with aluminium extruded restraint caps 50mm x18mm and finished with snap on cover - 50 mm wide. Frame Finish (External): SDF Powder coating (To Meet Requirement of AAMA 2605) (Colour-TBC). Work includes steel sub-frame, aluminium extruded adaptive framework assembly hardware, interface details such as cladding cover, flashings, insulation, sealants, brackets, and any sub-frame as required. |
| --- | --- | --- |
| 94 | GLAZED INFILL | 6mm HS + 1.52 PVB + 6mm HS clear laminated clear glass. |
| 95 | Interior partitions and door Framing system - interior solid partitions | 25mmX100mm Aluminium Extruded Frame anodized to AAMA 607.1. in approved finish with etched surface having 25 microns minimum of material removed. Works to include anchors, interface seals, brackets etc. |
| 96 | Solid infill - interior solid partitions | Providing and fixing Second Class teak wood paneling within the framing system. Paneling for paneled surface 25mm to 40mm thick. |
| 97 | Finishing/ polishing of internal solid infill | Providing and applying Melamine polishing of approved makes on wood & veneer works (two or more coats) including finishing grooves with paint/polish, preparation of surface and staining to the approved colour and shade as per the manufacturer’s specifications including scaffolding, curing, cleaning the surfaces and other incidental work to be done etc. complete at all floors for any height as directed by engineer in charge. |
| 98 | Framing system - interior glass partitions | 25mmX100mm Aluminium Extruded Frame anodized to AAMA 607.1. in approved finish with etched surface having 25 microns minimum of material removed. Works to include anchors, interface seals, brackets etc. |
| 99 | Vision infill - interior glass partitions | Providing and fixing 10mm fully tempered clear glass + system airgap + 10mm fully tempered clear glass for infill within the framing system. |

| 100 | Internal doors | 12mm thick tempered clear glass on a frameless system fixed on stainless steel patch fittings with integrated floor springs, Patch Fittings, pivots, with SS 316 600 mm handles as per approval from the Architects and integrated access control system- Matching to Internal partitions. |
| --- | --- | --- |
| 101 | FIXED VISION GLASS - LEANING INWARD | FIXED VISION GLASS - SLOPING INWARD - FRAMING SYSTEM OVER STEEL STRUCTURE IN FORM OF CASSETTE FRAMES - Providing and fixing aluminium work for doors, windows, ventilators and partitions with extruded built up standard tubular sections/ appropriate Z sections and other sections of approved make conforming to IS: 733 and IS: 1285, fixing with dash fasteners of required dia and size, including necessary filling up the gaps at junctions, i.e. at top, bottom and sides with required EPDM rubber/ neoprene gasket etc. Aluminium sections shall be smooth, rust free, straight, mitred and jointed mechanically wherever required including cleat angle, Aluminium snap beading for glazing / paneling, C.P. brass / stainless steel screws, all complete as per architectural drawings and the directions of Engineer-in-charge. (Glazing, paneling and dash fasteners to be paid for separately) |
|  |  | Polyester powder coated aluminium (minimum thickness of polyester powder coating 50 micron) |
| 102 | LAMINATED IGU - VISION GLASS SLOPING INWARD | Providing and fixing double glazed hermetically sealed glazing in aluminium windows, ventilators and partition etc. with Laminated glass - 6 mm thick SG SKN II 476 ENVISION PLUS glass+1.52mm PVB+6MM thick HS clear glass on outside and 6mm thick clear glass on the inside, having 12 mm air gap, including providing EPDM gasket, perforated aluminium spacers, desiccants, sealant (Both primary and secondary sealant) etc. as per specifications, drawings and direction of Engineer-in-charge complete. |
|  |  | Laminated insulated vision glass [OUT]6mm HS with Double Low-e coating on surface 2+1.52mm PVB+6mm clear HS glass + 12mm Air Space + 6mm HS clear - Saint Gobain SKN II 476 Envision Plus |

| 103 | TYPICAL FIXED VISION GLASS | FRAMING SYSTEM FOR FIXED VISION GLASS BASED ON ARCHITECTURAL MODULATION- Providing and fixing aluminium work for doors, windows, ventilators and partitions with extruded built up standard tubular sections/ appropriate Z sections and other sections of approved make conforming to IS: 733 and IS: 1285, fixing with dash fasteners of required dia and size, including necessary filling up the gaps at junctions, i.e., at top, bottom and sides with required EPDM rubber/ neoprene gasket etc. Aluminium sections shall be smooth, rust free, straight, mitred and jointed mechanically wherever required including cleat angle, Aluminium snap beading for glazing / panelling, C.P. brass / stainless steel screws, all complete as per architectural drawings and the directions of Engineer-in-charge. (Glazing, panelling and dash fasteners to be paid for separately) |
| --- | --- | --- |
|  |  | Polyester powder coated aluminium (minimum thickness of polyester powder coating 50 micron). |
| 104 | TYPICAL VISION GLASS | Providing and fixing double glazed hermetically sealed glazing in aluminium windows, ventilators and partition etc. with 6 mm thick SG SKN II 476 ENVISION PLUS glass on outside and 6mm thick clear glass on the inside, having 12 mm air gap, including providing EPDM gasket, perforated aluminium spacers, desiccants, sealant (Both primary and secondary sealant) etc. as per specifications, drawings and direction of Engineer-in-charge complete. |
|  |  | Typical insulated vision glass [OUT]6mm HS with Double Low-e coating on surface 2 + 12mm Air Space + 6mm HS clear - Saint Gobain SKN II 476 Envision Plus. |

| 105 | OPERABLE DOORS | SYSTEM | - | Providing and fixing aluminium work for doors, windows, ventilators and partitions with extruded built up standard tubular sections/ appropriate Z sections and other sections of approved make conforming to IS: 733 and IS: 1285, fixing with dash fasteners of required dia and size, including necessary filling up the gaps at junctions, i.e. at top, bottom and sides with required EPDM rubber/ neoprene gasket etc. Aluminium sections shall be smooth, rust free, straight, mitred and jointed mechanically wherever required including cleat angle, Aluminium snap beading for glazing / paneling, C.P. brass / stainless steel screws, all complete as per architectural drawings and the directions of Engineer-in-charge. (Glazing, paneling and dash fasteners to be paid for separately) : For shutters of doors, windows & ventilators including providing and fixing hinges/ pivots and making provision for fixing of fittings wherever required including the cost of EPDM rubber / neoprene gasket required (Fittings shall be paid for separately) : Polyester powder coated aluminium (minimum thickness of polyester powder coating 50 micron) |
| --- | --- | --- | --- | --- |
|  |  |  |  | Handles, Locks, Door Closers etc. to be calculated separately |

| 106 | SKYLIGHTS | FRAMING SYSTEM OVER STEEL STRUCTURE IN FORM OF CASSETTE FRAMES - Providing and fixing aluminium work for doors, windows, ventilators and partitions with extruded built up standard tubular sections/ appropriate Z sections and other sections of approved make conforming to IS: 733 and IS: 1285, fixing with dash fasteners of required dia and size, including necessary filling up the gaps at junctions, i.e. at top, bottom and sides with required EPDM rubber/ neoprene gasket etc. Aluminium sections shall be smooth, rust free, straight, mitred and jointed mechanically wherever required including cleat angle, Aluminium snap beading for glazing / paneling, C.P. brass / stainless steel screws, all complete as per architectural drawings and the directions of Engineer-in-charge. (Glazing, paneling and dash fasteners to be paid for separately) Polyester powder coated aluminium (minimum thickness of polyester powder coating 50 micron) |
| --- | --- | --- |
| 107 | SKYLIGHT LAMINATED GLASS | Providing and fixing laminated glass with 6 mm thick clear glass on outside and 6mm thick clear glass on the inside, having 1.52mm PVB Layer, including providing EPDM gasket, perforated aluminium spacers, desiccants, sealant (Both primary and secondary sealant) etc. as per specifications, approved drawings, and direction of Engineer-in-charge complete. |

| 108 | ALUMINIUM CLADDING AT PLINTH LEVEL | EXTERIOR CLADDING - Providing and fixing machine moulded aluminium covering of approved pattern & design, made out of machine cut aluminium sheet and machine holed for receiving dash fastener, over expansion joints on vertical surfaces/ceiling floors, the fixing on plate in one row on one side of joint only shall be done with stainless steel dash fasteners of 8 mm dia and 75 mm long bolt including providing aluminium washers 2 mm thick & 15 mm dia , at a staggered pitch of 200mm centre to centre including drilling holes in the receiving surface and providing expandable plastic sleeves in holes etc. complete as per direction of Engineer-in-charge. Powder coated aluminium sheet 2.5mm thick (minimum thickness of powder coating 50 micron) |
| --- | --- | --- |
| 109 | FRAMING SYSTEM | CASSETTE FRAME SYSTEM AS PER ARCHITECTURAL INTENT TO REVEAL THE COVER CAPS EMPHASIZING MODULATION OF PANELS ON THE EXTERIOR - Providing and fixing aluminium work for doors, windows, ventilators and partitions with extruded built up standard tubular sections/ appropriate Z sections and other sections of approved make conforming to IS: 733 and IS: 1285, fixing with dash fasteners of required dia and size, including necessary filling up the gaps at junctions, i.e. at top, bottom and sides with required EPDM rubber/ neoprene gasket etc. Aluminium sections shall be smooth, rust free, straight, mitred and jointed mechanically wherever required including cleat angle, Aluminium snap beading for glazing / paneling, C.P. brass / stainless steel screws, all complete as per architectural drawings and the directions of Engineer-in-charge. (Glazing, paneling and dash fasteners to be paid for separately) Polyester powder coated aluminium (minimum thickness of polyester powder coating 50 micron). |

| 110 | ALUMINIUM CLADDING ABOVE PLINTH LEVEL (SLOPING INWARD) | EXTERIOR CLADDING - Providing and fixing machine moulded aluminium covering of approved pattern & design, made out of machine cut aluminium sheet and machine holed for receiving dash fastener, over expansion joints on vertical surfaces/ceiling floors, the fixing on plate in one row on one side of joint only shall be done with stainless steel dash fasteners of 8 mm dia and 75 mm long bolt including providing aluminium washers 2 mm thick & 15 mm dia , at a staggered pitch of 200mm centre to centre including drilling holes in the receiving surface and providing expandable plastic sleeves in holes etc. complete as per direction of Engineer-in-charge. Powder coated aluminium sheet 2.5mm thick (minimum thickness of powder coating 50 micron) |
| --- | --- | --- |
| 111 | FRAMING SYSTEM | CASSETTE FRAME SYSTEM AS PER ARCHITECTURAL INTENT TO REVEAL THE COVER CAPS EMPHASIZING MODULATION OF PANELS ON THE EXTERIOR AS WELL AS INTERIOR - Providing and fixing aluminium work for doors, windows, ventilators and partitions with extruded built up standard tubular sections/ appropriate Z sections and other sections of approved make conforming to IS: 733 and IS: 1285, fixing with dash fasteners of required dia and size, including necessary filling up the gaps at junctions, i.e. at top, bottom and sides with required EPDM rubber/ neoprene gasket etc. Aluminium sections shall be smooth, rust free, straight, mitred and jointed mechanically wherever required including cleat angle, Aluminium snap beading for glazing / paneling, C.P. brass / stainless steel screws, all complete as per architectural drawings and the directions of Engineer-in-charge. (Glazing, paneling and dash fasteners to be paid for separately) Polyester powder coated aluminium (minimum thickness of polyester powder coating 50 micron) |

| 112 | INTERNAL CLADDING FOR SOLID CLAD PART OF FAÇADE | Providing and fixing 12 mm thick prelaminated particle board flat pressed three layer or graded wood particle board conforming to IS: 12823 Grade l Type ll, in paneling fixed in aluminium doors, windows shutters and partition frames with C.P. brass / stainless steel screws etc. complete as per architectural drawings and directions of Engineer-in-Charge. |
| --- | --- | --- |
| 113 | ALUMINIUM CLADDING AT ROOF | EXTERIOR CLADDING - Providing and fixing machine moulded aluminium covering of approved pattern & design, made out of machine cut aluminium sheet and machine holed for receiving dash fastener, over expansion joints on vertical surfaces/ceiling floors, the fixing on plate in one row on one side of joint only shall be done with stainless steel dash fasteners of 8 mm dia and 75 mm long bolt including providing aluminium washers 2 mm thick & 15 mm dia , at a staggered pitch of 200mm centre to centre including drilling holes in the receiving surface and providing expandable plastic sleeves in holes etc. complete as per direction of Engineer-in-charge. |
|  |  | Powder coated aluminium sheet 2.5mm thick (minimum thickness of powder coating 50 micron) |
| 114 | INTERNAL CLADDING FOR ENTRANCE AND EXIT WALLS | Providing and fixing 12 mm thick prelaminated particle board flat pressed three layer or graded wood particle board conforming to IS: 12823 Grade l Type ll, in paneling fixed in aluminium doors, windows shutters and partition frames with C.P. brass / stainless steel screws etc. complete as per architectural drawings and directions of engineer in-charge. |

| 115 | Acoustic Doors | Acoustical door shall be of thickness as mentioned in the approved drawing, having 38 dB acoustic cum fire check door shutters, having Fire rating 120 Minutes minimum, including door frame of minimum section 145mm x 75mm teak wood fixed to the door jambs with anchor fasteners as required, having infill of 47mm thick resin bonded glass wool of 24kg/cum density coated with FR Acrylic sealant sandwiched between two 9mm thick calcium silicate boards 100% without asbestos, Brucite and Meerschaum having a density not more than 1150kg/cum and thermal conductivity 0.14 W/M*K faced with 8mm thick marine IS 303 grade ply of approved with heat activated intumescent fire seal strip of size 10mm x 4mm mounted in the grooves having hardwood lipping of size 15mm x 70mm and fire retardant 1mm thick laminate finish facing on both sides in desired shade and colour. The work shall be executed as per drawings, and instructions of engineer in charge. (Note: Contractor to submit shop drawing & Test certificates from manufacturer (The Manufacturers should already have a test report with complete Raven Seals tested from CBRI Roorkee for the total design including door closers and panic bar) for approval of the Engineer in charge. The scope to include supply and installation of all required hardware as per hardware schedule. |
| --- | --- | --- |

| 116 | Fire Doors | - Two Hr metal Fire resistant door including door frame confirming to BS: 476 part 22 and IS 3614 part II all complete, design and size as per Local fire authority and NBC norms and recommendations . The fire door should not collapse during the rated period of fire under the specified fire conditions. Door to have fire rated vision panel with 6mm thick clear glass as per specifications. The door frames and shutter are finished with thermo setting acrylic paint for scratch resistance and durability. All fittings and accessories to be SS and shall also include Panic hardware and incl. 2 hr fire rated automatic dual adjustable door closer ,30-minute fire rated smoke seal (including 2 hr fire resistance stop seal for shaft as per NBC norms) shall be as per fire norms specifications and as approved by E- in-C. The Panic Hardware shall have 05 years mechanical warranty. - 50 mm thick glazed fire-resistant door shutters of 120 minutes fire rating conforming to IS:3614 (Part-II), tested and certified as per laboratory approved by Engineer-in- charge, to be done by EPC contractor as per CPWD specification and as per relevant DSR 2018 item(s) as per approval of E-in-C. The fire door should not collapse during the rated period of fire under the specified fire conditions. All fittings and accessories to be SS 304 grade and shall incl. Panic hardware and incl. 2 hr fire rated automatic dual adjustable door closer ,30 minute fire rated smoke seal (incl. 2 hr fire seal in shafts as per NBC norms) shall be as per fire norms specifications and as approved by E-in-C. |
| --- | --- | --- |

## Minimum Acceptable Specifications for Supply & Installation of Movable Furniture –

    - General Quality Standards and Certifications

- Compliance with Standards: All furniture shall comply with relevant Indian Standards (BIS) for design, materials, and safety. For library furniture, refer to IS 1829 (Part 1:1978 & Part 2:1993), IS 1883:1983 (metal shelving racks, adjustable type), and IS 4116:1988 (wooden shelving cabinets, adjustable type). Where an Indian Standard is not available, internationally recognized standards (ANSI/BIFMA, ISO) shall be followed for performance and safety.

- BIFMA Certification: Furniture should meet or exceed BIFMA standards for strength and durability in each category – e.g. BIFMA X5.1 for office seating, BIFMA X5.4 for lounge seating, BIFMA X5.5 for desk and table products, BIFMA X5.6 for panel-based workstations, and BIFMA X5.9 for storage units. Independent test certification to these standards is preferred to ensure high quality.

- Materials and Finish Quality: Use premium-grade materials – high-density fiberboard (HDF) and seasoned solid hardwood for wood components, and prime quality CRCA/HR steel for structural parts. All wood shall be properly seasoned and treated for durability (termite-proof, minimal warping), and all steel parts shall be rust-protected and powder coated or enamel finished. Powder coating shall be per IS 13871 with at least 40–50 µm thickness for a smooth, hard finish. Finishes should be non-toxic and long-lasting (scratch and stain resistant).

- Environmental and IGBC Criteria: Furniture should contribute to green building goals – use low-VOC paints, adhesives, and finishes (e.g. GREENGUARD certified for indoor air quality) so that products meet the BIFMA X7.1 standard for low emissions. Wherever possible, use sustainably sourced wood (FSC certified) and recyclable or eco-friendly materials. Preference will be given to products with IGBC/GreenPro or equivalent green certifications, and those offering BIFMA “Level” sustainability certification for multi-attribute environmental performance.

    - Library Book Racks (Fixed Shelving)

- Construction: Provide heavy-duty library shelving racks designed for a ~3,00,000 book collection. Racks shall be of fixed type (anchored to floor or wall for stability) with modular unit bays that can be ganged together. Use robust steel sections for uprights (angles/channels conforming to IS 808 and IS 1977) and adjustable slotted posts at 50 mm pitch for shelf height adjustment. Welding and bolting shall be done neatly with smooth grinding at joints; no sharp edges or burrs (all edges to be rounded or deburred for safety).

- Dimensions & Layout: Standardize rack dimensions as per BIS guidelines for convenient access and load distribution. Height of each rack unit shall be about 2100–2200 mm (approx. 2175 mm as per IS 1829), allowing 6–7 adjustable shelves plus a base. Depth for double-faced racks ~450 mm (for back-to-back shelving) and for single-faced racks (against walls) ~230 mm. Typical bay width ~900–1000 mm (adjustable shelving length) for ease of handling books. Each double-faced bay (~900 mm on each side) holds roughly 150 books (estimate ~40 books per meter of shelf). Adequate aisle spacing shall be provided between rack rows as per library standards (at least 1.0–1.2 m clear aisle).

- Load Capacity: Shelves shall be engineered for heavy loads – at least 60 kg per shelf (per bay) without permanent deformation. The bottom-most shelf in double-faced units should support ~80 kg (since it carries books from both sides). Shelves must not sag under full load of books; use minimum 0.8–1.0 mm thick steel or 25 mm thick engineered wood panels for shelves, with reinforcements (bending/boxing or undercarriage stiffeners) as needed.

- Design Features: Adjustable shelves on clips or brackets to accommodate various book heights (standard shelf spacing ~350 mm center-to-center, adjustable in 50 mm increments). Provide book stopper partitions (bay dividers) for double-faced racks to prevent books from pushing through to the opposite side. Each shelf front edge shall have an integrated label holder groove or metal label frame for easy identification of sections. End-of-aisle panels or rack end boards should be provided (can be decorative wooden end panels for aesthetics, fixed onto steel structure). All racks must be plumb and stable – base plates or plinths should allow anchoring to floor; if free-standing, provide cross-bracing or wall fixings as required to prevent tipping.

- Materials & Finish: The primary structure (uprights, shelves) shall be of mild steel (CRCA) confirming to IS 513 or IS 1079, Grade O, with high-quality powder coated finish in approved color. The powder coat should be oven-baked, smooth and uniform, with a hard adherent film free of runs or scratches. Surface treatment before coating must include degreasing, pickling, and phosphating as per IS 3618 to ensure corrosion resistance. Minimum dry film thickness of 40 µm is required. If wooden shelving is used in select areas (for a heritage or premium look), components shall be solid teakwood or high-grade HDF board with laminate finish – but such shelves must still meet the same load criteria. Any wood parts should be finished with melamine polish or laminated surface for durability. All hardware (nuts, bolts) to be zinc plated or galvanized to prevent rust.

- Standards and Testing: The shelving system shall conform to IS 1829 (Part 2): 1993 specification for library steel furniture and IS 1883:1983 for adjustable metal racks. Fabrication tolerances, shelf deflection limits, etc., should follow these standards. In absence of a specific BIS test, shelves may be tested per ANSI/BIFMA shelf loading standards. Finished racks should also meet fire safety norms (materials with at least Class 1 surface spread of flame if applicable for large installations). A certificate of compliance to these standards and load tests should be provided.

    - Comfortable Seating (Reading Area Chairs)

- Type and Design: Provide comfortable armchairs or lounge chairs for reading areas and informal library seating. These chairs are typically stationary (non-wheeled), with a focus on support and comfort for extended sitting. The design shall include a high supportive backrest (ergonomically contoured) and comfortable seat with ample cushioning. Backrests should be slightly inclined and curved to match the lumbar curve, without overly restricting movement. Seat dimensions should be generous: approximately 450 mm seat height, 450–500 mm seat width and depth to accommodate users of all sizes. Armrests, if provided, should allow easy ingress/egress and support reading posture.

- Construction & Materials: The frame should be heavy-duty hardwood (such as teak/beech) or metal (steel tubular frame) for strength and longevity. Joints in wooden frames must be double-dowelled or mortise-tenon glued and screwed; in metal frames, use continuous welds ground smooth. Cushioning: Seats and backs to have high-density polyurethane foam (min. 40 kg/m³ density) or equivalent, with a softer top layer for comfort. The cushioning should retain shape over time without sagging. Where applicable, use webbing or spring support under the cushions for added comfort. The upholstery material shall be a premium fabric or eco-leather (leatherette) that is wear-resistant (minimum 30,000 rubs Martindale), fade-resistant, and easily cleanable. Fabric upholstery is preferred to be breathable; if leatherette is used, provision for ventilation (perforation or breathable design) is recommended to avoid sweating.

- Ergonomics and Comfort: Chairs shall incorporate ergonomic features suitable for reading posture – a slightly reclined back (≈100° angle), padded armrests at a comfortable height, and seat edge rounded (waterfall front) to prevent pressure on thighs. The design should allow slight rocking or flex if possible (for example, a spring or flex-back) to enhance comfort during long reading sessions, but without compromising stability. If upholstery is not heavily padded, a woven cane or mesh support can be used for the seat/back to provide ventilation and slight yield as recommended in library standards. Noise-dampening pads or glides must be fitted on chair legs to avoid noise on floor movement.

- Finish & Aesthetics: All wood surfaces to be smoothly sanded and finished with PU lacquer or melamine polish in approved shade, showing the natural grain if solid wood is used. Metal frames, if any, to have powder coat finish in a color complementing the decor. Upholstery fabric color/pattern will be as per architect’s approval, with fire-retardant treatment if required by safety codes. The finished look should reflect premium quality and blend with the library interior design.

- Standards Compliance: Chairs shall conform to IS 3499 (Part 1):1985 for non-revolving chairs (covering materials, construction, and finish) as applicable. Key dimensions should follow ergonomic guidelines (e.g. ISO 9241-5 or BIFMA G1) for seat height, depth, and back support. Durability should be verified per BIFMA X5.4 (Lounge and Public Seating) tests which include drop tests, leg strength, and seat load tests. The chairs must be able to support at least a 120 kg static load without structural damage (safety factor included). All foam and fabric should meet flammability standards (BS 5852 or CA 117) for fire safety in public spaces.

    - Ergonomic Chairs (Adjustable Work/Study Chairs)

- Type and Usage: Provide high-quality ergonomic swivel chairs for workstations, study carrels, and staff offices. These chairs shall be revolving, tilting, and height-adjustable to accommodate various user preferences. The design must promote correct posture for computer use or desk work over long periods.

- Key Ergonomic Features: Chairs shall have a gas-lift height adjustment mechanism allowing seat height roughly from 400 mm to 500 mm from the floor (covering 5th–95th percentile users). The seat pan should be at least 450 mm wide and 400 mm deep with a rounded front edge. A five-star base (preferably of polished aluminum or fiber-reinforced nylon) with five rolling casters is required for stability and smooth movement (casters to be twin-wheel and suitable for carpet/tile floors). The chair should provide tilt mechanism with lock and tension control, enabling the backrest to recline ~15° and lock upright. Armrests shall be provided – ideally 2D or 3D adjustable (height adjustable to fit under tables, width or pivot adjustable for comfort). A lumbar support (adjustable height/depth) must be integrated into the backrest to support the lower back. Backrest should be high enough to support mid-back or shoulders (overall chair height not less than ~800 mm), and preferably with a mesh back or breathable upholstery for ventilation.

- Materials & Construction: The chair frame (seat and back support) should use structural grade plastic or metal – e.g. glass-filled nylon back frame or steel under-seat plate. The seat should have a firm inner plywood or injection-molded shell padded with molded foam (density ~45 kg/m³) for lasting comfort. The backrest may be foam-upholstered or mesh. All fabric or mesh used must be durable (min. 50,000 rubs) and preferably replaceable. The base’s five legs to be of die-cast aluminum alloy (ADC12 grade or equivalent) or fiber-reinforced polymer, with a load capacity of at least 150 kg. The class-4 gas spring and tilt mechanism components shall be of steel with proper surface treatment. Fasteners must be high-tensile steel. The overall construction should be wobble-free and sturdy – e.g., the chair must safely support 150 kg load on the seat without any instability or structural damage.

- Adjustability & Comfort: Chairs should support dynamic seating – swivel 360°, smooth height adjustment, and reclining. Tilt tension should be adjustable to user weight. If possible, include a synchro-tilt mechanism (backrest reclines at slightly higher ratio than seat) for better ergonomics. Armrests if adjustable should range roughly 180 mm to 250 mm above seat and be comfortably padded. Edges and corners of all components are to be rounded. Provide pivoting backrest or flex-back that follows user’s back. Seat and back cushions (if upholstered) should be covered in breathable mesh/fabric that meets fire safety norms.

- Finish & Appearance: All visible plastic parts shall have uniform color and texture (color as per user selection, typically black or grey for base/arms). Metal parts (like the base or support brackets) can be chrome plated or powder-coated. The finish must be free of sharp mold lines or burrs. Logos of manufacturer (if any) should be discreet. Aesthetic design should be contemporary and professional, suitable for an executive library environment.

- Standards and Certification: Chairs must conform to IS 3499 (Part 2):1985 – specification for revolving and tilting chairs, meeting all prescribed dimensional and safety requirements. Performance to be validated against ANSI/BIFMA X5.1 standards for office seating (including back durability test, seat drop test, caster durability, swivel cycle test, etc.). The manufacturer should ideally have BIFMA membership or certification. Additionally, the chairs should comply with BIFMA X7.1 (low VOC emissions) and be GREENGUARD certified if possible, as these will be used in air-conditioned indoor spaces.

    - Study Tables (Reading Tables)

- Design and Sizes: Study tables for the reading areas shall be of sturdy construction with dimensions as per library standards. A typical single-reader study table shall be about 900 x 600 mm in table top size, allowing ample space for books and writing. Larger tables for group study can be provided – e.g. a 3-person reading table ~2400 x 600 mm (which can seat three readers on one side), or a four-person table ~1200 x 1200 mm square, etc., as per layout requirements. Height of the table top shall be around 750 mm above floor, which is the standard desk height for writing comfort. For improved ergonomics during long study hours, each table should have a modest footrest bar around 150 mm above the floor for readers to rest their feet. Table corners should be rounded or chamfered for safety.

- Construction & Materials: The table top board may be constructed from High Density Fiberboard (HDF) or plywood of at least 18 mm thickness, or blackboard, with a high-pressure laminate (HPL) surface on both sides. The laminate should be 1 mm thick decorative laminate (matt finish, scratch-resistant). Edges of the tabletop to be either post-formed or finished with hardwood lipping (minimum 20 mm thick) with rounded profile, securely glued and pin-nailed. The understructure/frame shall be either solid wood (teak/maple) or steel. If wood, legs should be 50 mm × 50 mm section timber or equivalent, with apron rails of ~25 mm thickness; all joints mortise-tenon or equivalent with reinforcement by corner blocks. If metal, legs and frame should be MS square tubes (≥ 50 × 50 × 2 mm) or fabricated steel sections, with proper gussets/bracing to prevent wobble. Steel components to be powder coated in an appropriate color. Each table should be robust enough to not deflect or wobble under concentrated load of at least 50 kg on the top.

- Features: Drawer Units: Some study tables (especially single-reader study carrels) should have an under-desk pedestal or drawer unit on one side for storage of personal items/books. This pedestal can have 1–2 drawers (with locks) and possibly an open shelf for bags. Drawers should run on smooth channels (telescopic slides) and be of metal or wood construction. Wire Management: If tables will be used with laptops or study lamps, provide cable grommets on the tabletop and a modesty panel or cable tray under the top for concealing wires. Modesty Panel: Larger tables should have a modesty panel (wood or metal) in the front for privacy and to improve rigidity, especially if placed in open areas. Levelers: Adjustable leveling glides to be provided on table legs (especially for metal frames) to account for floor unevenness.

- Finish: Wooden surfaces (if any) to be polished or laminated. For polished solid wood, use polyurethane (PU) polish or melamine polish in satin finish – it should be smooth, without roughness, and sealed against moisture. All laminate surfaces must be of uniform color/pattern as selected (with edge banding matching or in contrast as per design). Steel frames to have powder coated finish (min. 50 µm) in standard colors like black, grey, or as specified. The finish should pass scratch resistance and corrosion tests (500-hour salt spray with no rust). Visible hardware like screws should be stainless or coated, and preferably concealed with caps.

- Standards Compliance: Design follows IS 1829 recommendations for library reading tables (size and height). The furniture should also align with IS 5968:1981 (dimensions for office tables) if applicable, ensuring ergonomic height. Load-bearing capacity and durability should be tested per BIFMA X5.5 (Desk/Table standard) – including deflection under load, stability (tip) test, etc.. All wood products (HDF/ply) must comply with IS 12406 (for MDF/HDF) or IS 303/710 (for plywood) regarding emission (E1 grade formaldehyde) and mechanical performance.

    - Office Tables (Work Tables)

- Description: Standard office tables are required for library staff offices or general use (e.g. librarians’ desks, cataloging work tables). These non-modular desks are typically for one user, providing a work surface and some storage (drawers). Typical sizes may range from 1200 x 600 mm for a compact desk to 1500/1800 x 750 mm for a larger table, depending on staff requirements. Height shall remain ~750 mm as standard.

- Construction: Top surfaces to be made of high-quality particle board, MDF, or plywood (18 mm or thicker) laminated with scratch-proof laminate (preferably post-formed front edge for durability). Underframes can be panel-end type (i.e. gable ends of boards) or with separate legs. Panel-end construction should use 25 mm thick pre-laminated boards for sides and modesty panel of 18 mm board. If using separate legs/frame, steel rectangular tube legs (e.g. 50 × 25 mm, 2 mm thick) or solid wood legs are acceptable, with cross bracing to ensure stability. Fasteners and inserts (like knock-down fittings) used in board constructions must be of high quality (minifix, etc.) to allow assembly/disassembly without loss of strength. Provide CPU hanger/stand and keyboard tray for computer workstations as needed. Each table should include a drawer unit: typically a box drawer and a file drawer on ball-bearing slides, built of 0.8 mm steel sheet or 18 mm board, lockable.

- Ergonomics & Features: Modesty panel in front (for privacy and to hide cables) made of matching board or perforated metal. Cable management: desks should have at least two 50 mm diameter cable grommet holes on the top, and provision below the desk (cable trays or clips) to route power/data cords neatly (especially for computer usage). Edges and corners must be smooth – all board edges finished with PVC edge band (2 mm thick on impact edges). If table is against a wall, allow for a gap or cut-out for running cables down. Weight capacity: the desk should support equipment like computers and books – at least 50 kg uniformly distributed on top without bowing.

- Finish: All laminate surfaces in the color/texture as per design (for instance, grey or beech woodgrain laminates). Exposed edges banded in matching PVC. Steel parts (legs, drawer fronts if metal) powder coated. If solid wood is used for any part, it shall be well seasoned and finished in PU polish. The overall finish should be professional and consistent with corporate office furniture quality.

- Standards Compliance: Office tables to adhere to BIS guidance for office furniture dimensions and safety. Where applicable, refer to IS 5277 for storage units (if file drawer included) and IS 1829/IS 1883 for shelving aspects. Desks should satisfy BIFMA X5.5 performance tests (including stability under vertical and horizontal loads, strength of desk drawers, cycle testing for drawers, etc.). All particle boards or MDF used must be E1 low-formaldehyde as per IS 3087 / IS 12406. Ensure that no harmful substances (like lead in paints or VOCs beyond limits) are present, in line with IGBC healthy indoor environment criteria.

    - Executive Tables (Executive Desks)

- Design and Size: Executive tables are high-end desks for senior personnel or the chief librarian’s office, reflecting premium quality. These are typically larger (e.g. 1800–2100 mm length × 900 mm depth for the main desk) with additional side credenza or return units as required. Height is standard ~750 mm. The design should be elegant with clean lines – often a freestanding desk with thick top and full panels, possibly with modest overhangs. Include a modesty panel and possibly an extended conference overhang on the visitor side for small meetings.

- Materials: Use top-grade materials for an executive feel. The tabletop core may be plywood or MDF (25 mm or thicker) with natural wood veneer finish (e.g. oak, walnut, teak veneer) polished to a smooth sheen, or a high-end laminate (textured or high-gloss as specified). Solid wood lipping of wood like teak,
beech or oak (at least 5 mm thick) should run all around the top edges for impact resistance and a rich look. The under-structure (panels or legs) can be made of MDF/plywood with veneer, or metal with cladding – for example, a steel frame providing support with wooden panels outside. Use of leatherette writing pads inlaid on the tabletop can be considered for a luxurious finish.

- Features: Provide storage components integrated into the desk: typically a pedestal of drawers (one shallow drawer for stationery and one or two deeper drawers for files) on one or both sides. Drawers to have soft-close channels and central locking mechanism. A wire management system must be built-in – concealed cable raceways for running power/LAN to the tabletop for computer, with flip-open cover outlets on the desk surface for power/USB. The desk should also include provisions for wire routing between the main desk and any return/side units (like an underside channel). Drawer Construction: Drawer fronts in matching veneer/finish; inner box in plywood or metal, mounted on telescopic slides. Handles: Use premium handles or push-open hardware for a sleek look. All locks should be of high quality (brass or equivalent) with a master key option if needed.

- Structural Integrity: Even with a stylish design, the desk must be structurally sound. If it has long spans (e.g. 2 m length) without intermediate support, ensure the top doesn’t sag – use metal stiffeners or thicker core. The desk should support heavy loads (e.g. stacks of books or equipment) – at least 100 kg distributed load capacity. Stability is crucial: if the design is ‘floating top’ style or has a single pedestal support, verify that it cannot tip with reasonable force. Leveling adjustors to be provided on base for leveling on uneven floors.

- Finish: Veneer surfaces shall be finished with polyurethane (PU) polish in open grain or closed grain finish as specified (matte or semi-gloss) – showing a uniform tone and free of blotches. Laminate surfaces (if used) should be of premium grade (e.g. high gloss or textured laminate, as per selection). All visible edges of boards to have matching veneer or solid wood trim. Metal components (like support frames or cable grommets) can be chrome plated or powder coated with a metallic finish. The overall appearance should exude executive quality, with meticulous attention to detail at joints and interfaces.

- Standards Compliance: The executive desk shall meet the same functional performance tests as other tables (BIFMA X5.5 for structural performance). Additionally, any glass components (if included for tops or returns) should be tempered safety glass (as per IS 2553). Adhesives and finishes must comply with IS 4200 series (for furniture finishing) ensuring no toxic emissions. Given the bespoke nature of executive furniture, manufacturer should provide a warranty for structural soundness and finish (e.g. polish not fading or peeling) and preferably have ISO 9001 quality certification.

    - Workstations (Modular Office Workstations)

- Description: Provide modular workstations for library staff or patron computer areas. These typically consist of work surfaces with integrated partition panels (half-height panels for privacy and acoustics). Workstation units could be linear, cluster (e.g. 4-pack), or cubicle style depending on layout. Each workstation should accommodate a computer, writing space, and basic storage (like under-desk pedestal or shelves).

- Panel System: Partition panels should be tackable and sound-absorbing, about 1200–1500 mm in height from floor (sufficient to define space while allowing light and oversight). Panels to have an aluminum anodized (or powder coated) frame structure and finished with fabric upholstery or laminate/acrylic as per design. Use fire-retardant foam/fiber board inside panels for noise reduction. The panels must be sturdy and have provision to attach work surfaces, shelves, or accessories. Panel junctions (90°, 120° or straight connectors) should be structurally secure, with leveling feet for stability. The system should be modular to allow reconfiguration if needed.

- Work Surface: The desk portion of the workstation shall use particle board or MDF of 25 mm thickness with laminate finish for durability. Typical size per person could be ~1200 mm width by 600 mm depth (for a single-sided workstation) or as required by design (e.g. corner L-shapes). Front edge to be post-formed or have PVC edge banding. Provide cut-outs or grommets on the worktop for cable management. Support the worktops on either panel hangers (cantilever brackets attached to panels) or leg supports. If using cantilevers, they should be heavy gauge steel brackets tested for load. If using legs, match them to panel system design (metal legs with levelers).

- Storage and Accessories: Each workstation should be equipped with at least one mobile pedestal unit (2-3 drawers, on wheels) that fits under the desk, constructed of laminated board or steel (with central locking). Additionally, desktop shelves or overhead cabinets may be mounted on the panel for extra storage (with clear height above desk of ~400 mm). Include accessory options like pin-up boards, whiteboard tiles on panels, paper trays or CPU holder mounted below desk, etc., as per user needs. All accessories should be compatible with the panel system (tool-free installation preferred).

- Electrical Provisions: The workstations must have integrated electrical raceways along the panels for safe routing of power and data cables. Panels should have a removable raceway cover at desk level or skirting level housing plug sockets, with capacity for wiring as per project requirements (e.g. 3 nos. 6/16A sockets and 2 data outlets per seat). All electrical components must be ISI marked and the system should allow daisy-chaining as per electrical design. There should be no exposed wires on the work surface – use cable trays and grommets effectively.

- Materials Finish: Panel fabrics to be polyester or polypropylene blend, inherently fire-retardant or treated to meet ASTM E84 Class A or equivalent. Color as per scheme (usually soothing, light colors for a library environment). Laminate for worktops should be of standard office grade (with resistance to wear, cigarette burn, etc.). Metal parts (panel frames, legs) powder coated (minimum 40 µm) in neutral colors (grey, white, etc.). All components should have a clean finish with no sharp edges (top of panels should have a protective cap).

- Standards Compliance: The modular workstation system shall meet ANSI/BIFMA X5.6 standards for panel systems – this covers stability of panels (not toppling when force is applied), strength of attachments (like work surfaces on panels), etc. Work surfaces should also comply with BIFMA X5.5 for load and deflection. The foam and fabric in panels should meet IS 110 (1983) or relevant standards for smolder resistance. Electrical fittings must comply with IS 732 (wiring code) and be certified. The entire system should carry a GreenGuard or similar certification for low emissions due to the extensive use of composite wood and fabric (to align with IGBC requirements).

    - Storage Units (Cabinets, Shelves, and Filing)

- Book Cases and Cabinets: Provide storage cabinets for library use, including book cases, file cabinets, and glass-front display cabinets. Steel book cases (with hinged glass doors) for reference materials shall conform to IS 7761:1983 – generally a cabinet ~1.8 m height with four adjustable shelves, made of steel sheet (0.8–1.0 mm thick) and lockable glazed doors. Glass-front cabinets (for rare books or archives) shall conform to IS 7760:1985, having framed glass doors with locking and dust-proof sealing. For aesthetic areas, wooden book cabinets can be provided – constructed from plywood/board with veneer or laminate finish, toughened glass doors, and adjustable shelves on pins. All cabinets must have leveling feet and wall anchoring provision for safety.

- Filing and Multi-Purpose Storage: Include standard steel filing cabinets (4-drawer or 2-drawer units for A4 or FC size files) made of CRCA steel, 0.7 mm body and 0.8 mm drawer fronts. Drawers to slide on heavy-duty ball-bearing runners and have anti-tip mechanism (such that only one drawer can open at a time to prevent tipping). Each drawer with a handle and label holder, central locking for all drawers. Additionally, provide wooden storage cupboards for staff use (ex: 2 m high cupboards with double doors) using 18 mm pre-laminated board for body and doors, edge-banded, with 3-4 shelves inside, lockable handles. Hinges should be soft-closing Euro hinges or heavy piano hinges in steel almirahs.

- Compartmental Shelving & Specialty Storage: For periodicals and journals, provide tilted display racks with storage. For newspapers, wooden newspaper stands with rods. Map cabinets or plan file cabinets (if needed) should be flat file drawers (steel or wood) with appropriate size (A0/A1) and roller slides. Lockers: If patrons need lockers, provide units of lockers (each compartment ~300×450×500 mm) powder-coated steel with ventilation slots and locks. All such specialty items should be of equal quality standards.

- Materials and Finish: Steel storage units must be made from prime quality CRCA steel (confirming to IS 513). All welds to be spot or seam welded smoothly. After fabrication, all steel items shall undergo antirust surface treatment and be powder coated with epoxy-polyester powder, minimum 50 µm thickness, in approved shade (typically grey or beige). The finish should pass hardness and adhesion tests (no peeling on cross-hatch test, pencil hardness ≥ HB). Wooden cabinets if provided should use laminate finishes for heavy use areas; if veneer/polish is used, it should match other furniture and be properly sealed. Glass for doors must be 4 mm (for small doors) to 6 mm (for large doors) toughened safety glass. Locks should be brass or steel with nickel finish, all using a master key system if required.

- Standards Compliance: All storage furniture must adhere to relevant BIS standards: e.g., IS 3312:2021 for steel shelving cabinets (adjustable shelves), IS 7761 for book cases, IS 5244 for sliding drawer cabinets, etc. They should also meet BIFMA X5.9 standards for storage units – including drawer cycle testing, door hinge tests, unit stability (tip test with open drawers), and shelf load tests. Weight capacities: each shelf in cabinets should hold ~40 kg, file drawers should hold ~30 kg of files, without deformation. If Eco-mark/GreenPro criteria are specified, materials should have certified eco-friendly coatings and recyclable content.

    - Mobile Compactors (High-Density Storage System)

- General Description: Mobile compactor units (also known as mobile shelving systems) shall be provided to maximize archival storage in limited space (ideal for less-used or archival books in a 3 lakh capacity library). The system comprises movable shelving racks mounted on rails, with one static bay at one end for stability. All compactor units shall be mechanical drive type (chain-and-sprocket drive with hand wheel) for ease of movement of heavy loaded racks.

- Configuration & Dimensions: The compactor system will consist of single-faced and double-faced shelving bays mounted on mobile bases. Height: Each bay approximately 1980 mm body height, plus base frame and rail (total ~2080 mm from floor to top). Bay Size: Standard bay width ~915 mm (three feet) and depth 457 mm for single-faced, 914 mm for double-faced (two back-to-back units). Multiple bays can be conjoined to form a movable unit. For example, a typical configuration might be 3 bays per mobile unit, each bay ~915 mm wide, yielding a 2745 mm long double-faced mobile unit. Shelves: Each bay to have 5 compartments (4 adjustable shelves + 1 fixed bottom) by default, with shelf pitch adjustable in 25 mm increments. The aisle space (when opened) between two mobile units should be roughly 750–800 mm for user access.

- Load Capacity: The compactor shelving must be heavy duty – each shelf shall carry minimum 70 kg load uniformly. The entire mobile unit (multi-bay) base frame and wheels must support the total loaded weight without deformation. The design load per bay is high to accommodate dense storage (books, files) – e.g., a 4-bay double-faced unit ~3.6 m long should hold ~2800 kg in total (70 kg × 40 shelves), so factor of safety should be applied accordingly. The tracking and drive mechanism should allow a single operator to move a fully loaded unit with minimal effort (approximately <10 kg force).

- Construction: Bodies/Carriage: The shelving racks themselves to be constructed similar to fixed shelving (0.8–1.0 mm steel shelves, adjustable, with back/top cover panels). These rack bodies are bolted or welded onto a rigid base frame made of rolled steel sections (IS 2062 steel, 3.15 mm thick channel/frame). Base frames shall have guide wheel assemblies – at least 4 wheels per base frame (for a 2-bay long unit) or more for longer units, made of cast iron or steel with nylon tread. Wheels mounted on sealed ball bearings and fixed to axles on the base frame. Rail System: Rails made of bright steel square bars (~25 mm) housed in a C-channel (2 mm thick) grouted to the floor. Rails must have end stoppers and an anti-derailment design. The rails should be perfectly leveled and aligned to prevent binding. Drive Mechanism: A chain-and-sprocket drive connects the hand wheel to the wheel axles of the unit. Turning the hand wheel translates into linear motion of the shelving unit. The chain drive shall be concealed inside a cover and include an idle locking knob to secure the unit when not in motion. Hand wheels should be robust (approx. 300 mm diameter, three-spoke with a polymer grip) for easy operation. Each mobile unit should also have a manual locking pin or knob that can lock it in a closed position for safety, and a central locking bar mechanism on the last unit to lock all units together when compacted.

- Safety Features: The system must include anti-toppling features – e.g. an anti-tip wheel or rail guiding mechanism that prevents the unit from tipping when heavily loaded and moved. Safety locks should be provided so that units do not accidentally roll (each unit with a brake or lock). Optionally, push handles or knob-type handles on the movable units’ sides can be given for manual positioning. When all units are closed together, a central lock (lever or lock bar) shall enable locking the entire block to prevent unauthorized access. Also include aisle safety reflectors or indicators if needed (to show if an aisle is occupied, for personnel safety in large installations).

- Finish: All steel components of the compactor (shelves, panels, frames, rails) are to be pre-treated and powder coated. The steel should be of prime quality (preferably pre-galvanized or pre-coated sheets) of thickness not less than 0.8 mm for shelves and 1.0 mm for load-bearing members. The powder coating (or epoxy paint) should be of 40–50 microns and of baked enamel quality, in a smooth semi-gloss finish (standard grey or as specified). The finish must withstand heavy usage – no peeling or chipping with repetitive movement. Apply rubber or PVC buffers between units to avoid metal contact and noise when closing. All nuts, bolts, and moving parts should be zinc plated or otherwise corrosion protected.

- Standards Compliance: The compactor system should generally comply with IS 1553 (Design of Library Shelving) recommendations for space utilization, and the fabrication with relevant BIS for steel shelving. While there is no dedicated BIS standard for mobile compactors, the system’s performance should be tested to relevant international benchmarks. For example, DIN EN 14073/14074 or similar standards for office storage regarding stability and strength may be referenced. The vendor should provide a test report or usage reference that the system can handle the stated loads. Tracks and mechanisms should conform to material standards (EN 31 for rails or equivalent). Furthermore, all aspects of the compactor should satisfy fire safety and building codes – e.g., if required, the compactor paint should be flame retardant, and the design should allow sprinkler penetration (typically open shelf design does). Warranty of at least 2 years on moving parts and an assurance of after-sales service must be provided.

    - Lounge Furniture (Sofas and Soft Seating)

- Items and Use: Lounge furniture will be used in library lounge areas or waiting zones, comprising sofas (1-seater, 2-seater, 3-seater), low coffee tables, and possibly ottomans. These should be of premium quality and comfort, inviting users to relax and read. The style should be modern yet durable, given the high usage in a public setting.

- Sofa Construction: Internal frame of sofas shall be made of seasoned solid wood (such as pine/salwood for internal structure) and/or a combination of wood and steel reinforcement. Joints in the frame must be strong (glued, screwed and corner-blocked). Seat support can be by steel sinuous springs or webbing straps with flame-retardant foam topping. Cushioning to be multi-layered foam: e.g., 12 cm thick seat cushions with high-density core (≥ 35–40 kg/m³) and softer top layer for comfort, wrapped in polyester fiber. Back cushions can be softer (density ~28 kg/m³) but should retain shape. Ensure that the seating has adequate depth (~500 mm) and backrest height to support users comfortably. If loose cushions are used, they should have removable covers for cleaning.

- Upholstery: Use high-quality upholstery fabric or eco-leather. Fabric should be heavy-duty commercial grade (≥ 30,000 double rubs Wyzenbeek or Martindale) and stain-resistant (preferably with a stain guard treatment). All upholstery should be lock-stitched and reinforced; stress points to have piping or top-stitching. If leatherette (artificial leather) is used, choose grades that are UV-resistant (to avoid cracking/peeling) and soft to touch, with a breathable layer if possible. Color and texture per design scheme (darker shades or patterned weaves may be preferred for hiding wear). Fire safety: upholstery should comply with BS 7176 Medium Hazard or equivalent (cigarette and match test).

- Coffee Tables and Ottomans: Coffee tables should have hardwood or MDF tops with veneer/laminate or glass top, and solid wood or metal legs. Edges of glass, if used, must be polished and glass should be tempered. Designs should avoid sharp corners (use chamfered or rounded corners for tables in lounge). Ottomans or poufs, if provided, should have the same build quality as sofas (wood frame, foam, upholstered) and ideally on glides or casters (locking) for mobility.

- Feet and Details: All sofas and chairs to have solid wood or metal feet, 50–100 mm high, with felt or rubber pads to protect flooring. Feet finished in polish or powder coat to match design. Bottom of sofas should allow clearance for cleaning underneath. Any decorative elements (buttons, tufting) should be secure and non-removable by users.

- Finish & Appearance: The lounge furniture pieces should present a consistent aesthetic. Wood parts (exposed legs or trim) to be polished in matching tone (e.g., walnut or wenge). Visible metal accents to be stainless steel or powder-coated as per design. The finishing of upholstery must be taut, with proper alignment of patterns if any, and no wrinkles. Cushions should recover shape after use (use foam with good resiliency or add siliconized fiberfill). Detailing such as tufted backs or piping is optional per style but must be executed to high standards. The overall impression should be that of comfort and luxury, aligning with a high-end library ambiance.

- Standards and Durability: Lounge seating should be built to commercial durability standards – capable of heavy daily use. It should conform to ANSI/BIFMA X5.4 for lounge seating, which includes tests for leg strength, seat impact (drop test with 102 kg bag), and cyclic loading. Frames should last 10+ years; fabric and foam should maintain integrity for at least 5 years under normal use. Any foam should be CertiPUR-US certified or equivalent (low VOC, no harmful chemicals) to align with green requirements. Coffee tables to satisfy safety (if glass, to IS 2553 tempered glass standard) and stability (no tipping with a 15° load). All lounge furniture to come with at least 1-year comprehensive warranty.

    - Classroom Furniture (Training Room Setup)

- Overview: If the library includes a multi-purpose room or classroom for lectures, provide classroom-style furniture that is flexible and durable. This typically includes student chairs (with or without writing tablets) and training tables that can be rearranged. The furniture should support quick reconfiguration (stackable or folding where possible) and withstand frequent handling.

- Student Chairs: For adult users (college level), use stackable chairs with option of a folding writing tablet. Chairs should have a one-piece molded seat/back shell of high-impact polypropylene or a plywood shell with foam upholstery, mounted on a sturdy metal frame. The standard seat height ~450 mm, seat width ~400 mm. The frame to be 1.2 mm thick MS elliptical or round tube, epoxy powder coated. If writing tablets are provided, they should be made of MDF or polypropylene, anti-slip surface, and securely foldable to the side or rear. Tablet hinges must be robust (metal-on-metal joint) to survive heavy use. Weight capacity of chairs at least 120 kg. Alternatively, folding tablet chairs (metal frame, fabric padded seat) could be offered, but ensure they lock safely when opened. Provide ganging hooks if chairs need to be aligned in rows.

- Training Tables: Use lightweight folding tables or flip-top modular tables for classroom use. Typical size: 1200 x 450 mm or 1500 x 500 mm per table, seating 2 participants. Tables should have folding legs or a flip-top mechanism so they can be stored compactly. Frame in MS steel tube (1–1.2 mm thick) with lockable folding joints. The tabletop of 18 mm particle board or plywood with laminate surface (prefer post-formed edges or PVC edge). Tables should be sturdy when deployed – locking devices to prevent accidental folding, and leg stabilizers to avoid wobble. Provide modesty panels if required for a more formal arrangement (could be removable).

- Special Seating (if needed): In case the library holds seminars, a few ergonomic task chairs could also be used in a classroom for instructors (same specs as ergonomic chairs above). But generally, classroom chairs should be simpler for bulk use.

- Finishes: Chairs: plastic shells in a color that complements the library (blue/grey etc.), UV resistant so they won’t discolor. Metal frames powder coated black or grey. Tablet surfaces in black ABS or laminated board. Tables: laminate in light colors, frame coated in neutral shade. All finishes should be scratch resistant and easy to clean (important for high turnover use).

- Standards Compliance: Classroom chairs and tables should conform to BIS standards for educational furniture where available (e.g., IS 4837 for school chairs, IS 7139 for lecture halls – if applicable). They should also meet BIFMA/ANSI standards for safety: non-upholstered chairs can be tested per BIFMA X5.1 (structural adequacy for general seating). Drop tests, fatigue tests for tablet arms, etc., should be conducted. If stacking chairs are used, stability of stack (at least 10 high without tipping) must be ensured and chair design per ISO 7173 tests. All furniture must be fire-safe (plastic materials self-extinguishing UL94 HB or better, foam/fire retardant fabric if upholstered). Edges of tablets and tables to be smooth per IS 1003 (safety requirements for school furniture). The vendor should supply a sample for evaluation to confirm ergonomics and sturdiness before bulk supply.

    - Museum-Grade Showcases

- Design & Aesthetics: Freestanding horizontal (table-style) and vertical (upright) vitrines designed for public display and archival preservation. Aesthetics shall reflect the Nalanda legacy with classical wood, bronze, and glass tones in the exterior finish. The cases will feature a clean, frameless glass presentation for maximum visibility, supported by a discreet structural frame. Wooden or bronze-finished base cabinets/pedestals may be used to harmonize with heritage interiors, provided they are sealed and inert. Overall form and detailing should be elegant and unobtrusive to highlight the displayed artifacts.

- Materials & Construction: Each showcase shall have a rigid metal framework (welded steel or aluminum alloy) for structural stability, which may be cladded with seasoned hardwood or bronze/brass trims for aesthetic appeal. All construction and trim materials must be chemically inert, suitable for long-term museum use – no off-gassing or acidic content that could harm artifacts. Interior linings, adhesives, and paints must be low-VOC and non-toxic. Use of powder-coated metal, glass, and other inherently non-emitting materials is required to meet green building and conservation standards. Wood components (if any) shall be formaldehyde-free (E1 or better) and finished in water-based low-VOC sealants. Joints should be tight and dust-proof, with silicone or EPDM gaskets (non-outgassing) to seal all openings. Construction must comply with archival storage standards (ISO 11799 for document repositories) to ensure materials and design do not compromise a stable internal environment for artifacts.

- Glazing (Glass Panels): Use laminated safety glass for all viewing panels (minimum two-layer laminated, each layer tempered). Glass should be ultra-clear, low-iron for true color rendering, and incorporate a UV-filter interlayer blocking ≥97% of UV radiation. This protects sensitive items from light damage. Laminated safety glazing also provides high security – even if shattered, it remains in place, keeping the case sealed. Glass thickness shall be sufficient for large spans (e.g. 6+6 mm lamination or higher as needed to meet safety norms). All glazing must meet IS 2553 (Indian safety glass standard) and/or EN 12150/EN 12543 for tempered/laminated glass. Exposed glass edges to be polished and set in silicone gaskets or bronze cames for a dust-tight fit. Where appropriate, apply an anti-reflective coating or glass treatment to minimize glare without affecting UV blockage.

- Environmental Control: The showcases must function as microclimate enclosures. They shall be tightly sealed with dust gaskets and minimal air exchange (target <0.1 air changes per day), achieved through continuous compression seals around doors/lids. Each case will include provisions for relative humidity (RH) buffering, such as a silica gel compartment or passive humidity control cassettes accessible to conservators. The design should maintain internal RH within a stable range (e.g. 45–55%) and temperature close to ambient, in line with PAS 198:2012 for environmental conditions. Materials inside (fabric linings, supports) must absorb no moisture or pollutants – use inert, Oddy-tested materials for all internal surfaces. LED lighting shall be integrated internally (if required) in a manner that isolates heat and UV from the objects. LEDs must emit no UV/IR and produce minimal heat; lighting levels should be adjustable and meet conservation guidelines (e.g. ≤50 lux for sensitive manuscripts). Any light fixtures ideally mounted outside the sealed volume or behind UV-filtered glass. Environmental monitoring ports (for data loggers or sensors) may be provided to track temperature/RH inside the case without opening.

- Security & Locking: Provide secure locking mechanisms to prevent unauthorized access or tampering. Cases should have flush, tamper-resistant locks (museum-quality barrel or pin tumbler locks) with key control restricted to authorized personnel. Locking may be multi-point for larger cases to ensure even gasket compression. The access doors/lids (whether top-opening for horizontal cases or side-opening for vertical cases) shall open smoothly – use gas spring assists or torque hinges for controlled motion so that heavy glass panels can be opened by one person safely. When closed and locked, there should be no pry points; the showcase should meet international museum security standards (e.g. EN 1627 class for resistance to forced entry, if applicable). All hardware inside the case (locks, hinges) should be hidden or minimally visible from the exterior to maintain a clean display appearance.

- Lighting & Electrical: Internal lighting (where included) will use conservation-grade LEDs with high color rendering (CRI ≥90) to faithfully illuminate exhibits. LED strips or spots should be recessed in the case hood or base, with diffusers to prevent glare on glass. Wiring must be concealed, and heat from lights must be vented away or isolated (for example, fiber-optic lighting can be considered for zero heat inside case). No UV-emitting lights are allowed. There should be an option to regulate lighting intensity and to turn off lights when the room is closed to minimize light exposure (since light damage to artifacts is cumulative). Electrical components should comply with IS 302 and IEC safety standards; a master cutoff switch for each case’s lighting is recommended. If needed, provide cable management and the ability to route power inconspicuously (e.g., from floor conduits).

- Dimensions & Configuration: A variety of sizes are anticipated to suit different exhibit items. Horizontal table cases might be roughly 1200–1500 mm (L) × 800 mm (W) for the display deck, with a height of 1000–1100 mm (for comfortable viewing from above). Vertical showcase vitrines might be ~2000 mm tall, with footprints around 1000×1000 mm or as appropriate for objects. Both forms shall be built with modularity in mind – for example, standardized base cabinets and interchangeable glass hoods where possible – to ensure consistency. Each showcase will include leveling feet (to accommodate floor unevenness) and must allow discreet anchoring to the floor or wall for seismic/wind stability if required. Construction tolerances are tight: doors should fit with ≤1 mm gaps when closed to ensure effective sealing. Despite their large size, units should be movable in sections (base, glass hood, etc.) to facilitate installation through standard doorways and then assembled on-site.

- Standards & Conservation Compliance: ISO 11799 (Information and documentation – Document storage requirements) and PAS 198:2012 guidelines must be met or exceeded. This means the cases, when closed, create an environment that protects against light, dust, pollutants, and excessive fluctuation of climate. All interior materials (paints, adhesives, textiles, cushioning foams) must pass the Oddy Test and/or Photographic Activity Test (PAT, ISO 18916) confirming they won’t harm paper, photographs, or artifacts over time. The showcases should be manufactured to professional museum case standards (as used by national archives or museums) – e.g., low air exchange rate, and provision for microclimate control. Safety glass certification is mandatory (e.g., UL 972 or EU equivalent for burglary resistance if displays include high-value items). In terms of structural performance, the cases must be sturdy and level, with no risk of toppling or collapse; if they are large freestanding units, they should comply with furniture stability standards (ANSI/BIFMA or EN 16121 for public space furniture). All finishes shall be IGBC compliant for low emissions – powder-coated metals, glass, and properly sealed woods are inherently low-VOC choices. The finished product should thus contribute to a safe environment both for the artifacts and for people (no toxic fumes, and sustainable materials where possible).

    - Newspaper and Magazine Display Units

- Function & Layout: These units will serve as high-visibility display racks for current newspapers and magazines, with integrated under-storage for back issues. Each unit shall be a modular bay that can stand alone or be ganged in a series. The design features sloped display shelves (angled for easy reading of titles) that hinge up or open to reveal storage compartments beneath. This “flip-up” periodical shelving system allows the latest issues to be on display while older issues are neatly stored yet easily accessible to readers. Units should encourage browsing: the displayed publications are presented face-out at a readable angle, and retrieval of back issues is convenient via the hinged shelf mechanism. The overall appearance should be neat and inviting, consistent with a modern library periodicals area, with no sharp edges or protrusions.

- Dimensions (Typical): A single-sided magazine/newspaper display unit is approximately 900 mm wide × 350 mm deep (about 36″ × 14″) per bay, and about 1900 mm high for a unit with 5 display tiers (or ~2200 mm for 6 tiers). Each display shelf is roughly 330–380 mm high to accommodate large format newspapers when closed. Depth of the under-shelf storage should be sufficient to hold stacked newspapers flat (≥300 mm clear internal depth). If double-sided units are used (for placement in open areas, accessible from both sides), the depth roughly doubles (~600–700 mm) while width and height remain similars. The system shall be modular, meaning additional bays can be attached side by side with shared vertical panels; this allows future expansion or reconfiguration. Adjustable leveling glides must be included at the base of each unit for stability on uneven floors.

- Construction & Materials: Hybrid wood and metal construction is required to balance durability with aesthetic warmth. The structural frame (uprights, shelf brackets, and internal supports) shall be made of heavy-gauge steel – e.g., 18-gauge for double-sided units, 22-gauge for single-sided shelves as a guideline – providing robust support for heavy stacks of paper. Vertical end panels and the hinged display shelves can be of engineered wood or plywood with veneer finish, or wood laminate on an MDF core, for a furniture-quality appearance. All wood products must be low-emitting (meeting formaldehyde E1 or better) and low-VOC in finishes. Metal components: steel parts are to have a powder-coated finish (electrostatically applied and baked enamel) in a color that complements the wood (for instance, matte black, grey or bronze tone powder-coat along with woodgrain laminate). Powder coating ensures a smooth, inert surface with no VOC emissions and high durability against scratches. The steel frame should include double-wall uprights and closed-back panels for rigidity and to protect stored materials. Fasteners should be hidden where possible (use of cam locks or concealed screws) to prevent snagging and present a clean look. All exposed edges of wood must be rounded or covered with impact-resistant PVC/T-molding for safety and longevity.

- Display Shelf Mechanism: Each display shelf shall be a hinged, flip-up panel that can be lifted by users or staff to access the storage below. The hinge mechanism must be sturdy – preferably a continuous piano hinge or multiple heavy-duty hinges along the shelf width – to withstand frequent use. When opened, the shelf should stay up (either via a detent or friction stay) so a user can retrieve materials with both hands free. Optionally, incorporate a soft-close feature or damping mechanism to prevent the shelf from slamming shut. The angle of the display shelves should be roughly 10° from horizontal; this incline displays magazine covers or newspaper fronts clearly while keeping them in place. A small lip (20–25 mm) at the bottom of each shelf will ensure that individual issues do not slide off. The storage compartment beneath each shelf should have a fixed flat shelf to hold back issues. These compartments must accommodate the largest publication size in the collection – for example, broadsheet newspapers stored flat, which may require an internal width ~500 mm or more. Interiors of storage cubbies should be smooth powder-coated steel or laminated board for easy cleaning and to avoid staining the paper.

- User Accessibility: The units are intended for public use, so ergonomic placement of materials is important. The top display shelf should be no higher than ~1700 mm from the floor, to be reachable by most users. The bottom display tier should be elevated (with a closed base plinth of ~100–150 mm) to keep materials from being too low and to allow toe space. Lifting of the display shelves should require minimal effort; hinges and any assist mechanism should make them light to open. Signage or label holders shall be provided on either the front of each shelf or on the end panels to identify the titles on display. For example, an acrylic label strip or metal card holder could be integrated. The design should also facilitate refilling by staff: shelves that lift and slide back (retract) into the unit are one acceptable approach, or those that simply hinge upward past 90°. All movements should be safe and intuitive.

- Finish & Environmental Considerations: Surface finishes must be durable and low-emission. Powder-coated metal (which is inherently non-emitting and contains no solvents) will be used on all steel parts. Wood surfaces will be finished with high-quality lacquer or polyurethane that is low in VOC (meeting IGBC green interior requirements for minimal off-gassing). The finish should be wear-resistant to withstand daily handling of periodicals. Colors/finishes will be selected in keeping with the library’s design scheme – likely warm wood tones for the wood components and neutral or dark tone for metal. All materials should be colorfast under ambient lighting (no fading or off-gassing over time). From a conservation standpoint, while patrons will handle current issues, the storage of back issues inside the unit means materials should not introduce acidity or pollutants; thus, interior paint/laminate should be archival-quality or at least neutral pH. If any fabric or rubber parts are used (for example, a cushioned pad to hold up the display shelf when closed), they must be non-staining and stable. Regular cleaning of these units should be possible with mild cleaners, so finishes should be moisture-resistant and not prone to delamination.

- Structural Performance & Safety: The display units must be structurally sound and safe in a public environment. They should comply with relevant furniture strength and stability standards (such as ANSI/BIFMA X5.9-2019 for storage units, which evaluates durability and tip-over safety). Each unit, when loaded with printed materials, should remain stable – if free-standing double-sided units are used, they may need a wider base or floor anchoring to prevent tipping. An anti-tip kit or bracket that secures single-sided units to walls is recommended for seismic safety and to prevent any pull-over by climbing. The hinged shelves, when flipped open, should not cause the unit to become front-heavy; internal counterweights or wall-fixing might be used if necessary. Load-bearing capacity: each display shelf should support at least 5–10 kg (a stack of periodicals) without deformation, and each storage compartment shelf should hold at least 20–30 kg of paper. The steel shelving and frame will easily meet this if of proper gauge, but the design must ensure no excessive shelf deflection (prefer <3 mm deflection under full load). All fasteners and connectors should be steel; critical load joints might use bolts and metal inserts rather than relying solely on wood screws. Safety features include smoothly finished edges, no pinch points in the hinge mechanism, and ideally a soft-close so that a dropping shelf won’t injure fingers. The base of units should be enclosed to avoid litter accumulation and to give a built-in look. In summary, the units should be library-quality furniture: robust, stable, and built to last many years of daily usage while preserving the condition of stored newspapers and magazines.

    - Manuscript Display and Storage Units

- Flat Display Cases for Manuscripts: These are table-top style manuscript display showcases for exhibiting rare books, manuscripts, or documents under controlled conditions. Each case shall be a low, horizontal vitrine with a lockable glass lid. Construction will parallel that of museum-grade cases, with an inert powder-coated metal or wood frame and UV-filtering safety glass top. The glass viewing panel (typically the entire top surface) must be laminated UV-filter glass to protect delicate manuscripts from light damage. The case interior (deck) will be lined with archival fabric or board – e.g., a neutral cotton fabric or suede over acid-free board – to provide an elegant backdrop and cushion for displayed items. Materials inside must all be PAT-certified (Photographic Activity Test passed) for safe contact with paper and photographs. The display area should accommodate an opened manuscript at a comfortable angle: include internal adjustable book cradles or wedges (made of Ethafoam or similar inert foam, with fabric covering) so that books can be propped open without strain on the binding. The lid should open on concealed hinges or assisted lifts; once closed, it should seal dust-tight with a rubber gasket. Locks are required on each case (small cam locks or flush barrel locks) to prevent public access. Internally, provide a small compartment or tray for silica gel or other humidity buffer, or design the case volume to work with the building’s HVAC to maintain roughly 50% RH. In essence, these display cases act as microclimate enclosures for single or small groups of manuscripts. Dimensions might be on the order of 1000 mm (L) × 750 mm (W) for the display area, with an overall height of ~1100 mm when on its supporting stand (for viewing). The base or stand for each case should be sturdy and match the case’s finish (wooden pedestal or powder-coated steel legs, etc.). All finishes on the interior must be matte to avoid glare and not attract attention away from the exhibit item. Lighting for these cases is generally external (room lighting); if internal lights are used, they must be low-intensity LEDs with strict controls. In sum, the manuscript display cases shall offer museum-level preservation: UV protection, inert environment, secure locking, and climate buffering, all while presenting the document in an accessible manner. They must meet the same standards as the museum showcases described above (ISO 11799, PAS 198 compliance, Oddy/PAT tested materials, etc.), scaled to manuscript-sized objects.

- Archival Flat-File Storage Cabinets: In addition to display, the library requires archival storage units with drawers for manuscripts and other flat items (such as maps, prints, oversized documents). These shall be flat file cabinets (plan file type) of the highest conservation quality. Construction: use solid, heavy-gauge steel throughout, with fully welded frame and reinforced corners for longevity. The steel shall be treated to be non-corroding – either galvannealed or given a protective powder coat – ensuring no rust or metal migration onto stored materials. Drawers must slide on ball-bearing telescopic slides or stainless steel tracks with ball rollers, providing smooth, easy operation even when fully loaded. Each drawer will have a safety stop to prevent it from pulling out entirely and dumping contents. Drawers should also have cushioned bumpers to dampen impact when closed. The interior of each drawer and cabinet must be entirely inert: the powder-coat paint applied should be baked and fully cured, with no residual solvent (the coating shall be solvent-free, non-reactive epoxy/polyester, giving a smooth, hard finish). Inside surfaces can be lined with removable acid-free liner paper or board, but this is optional if the paint finish itself has passed Oddy/PAT tests (many high-quality flat files are used unlined). The design should prevent dust ingress – typically, a flanged back and tight tolerances around drawers; some models include a thin felt or rubber seal at drawer fronts. The cabinet base shall either be an enclosed steel plinth (raising the bottom drawer off the floor to protect from any water on floor and to put it at usable height) or a leg frame; stacking of modules is possible (e.g., two 5-drawer units stacked for 10 drawers total) and if so, provide interlocks to secure stacked units. Dimensions & Capacity: The drawers should be sized to store large manuscript folios flat. A common standard is to accommodate A0 paper (841 × 1189 mm) internally. For instance, a 5-drawer cabinet might have internal drawer tray dimensions ~1300 × 900 mm to allow for oversized archives. Drawer height around 50 mm (useful height) is typical for documents – enough to store a boxed manuscript or a small stack of papers. The cabinet should support heavy loads; each drawer may be rated for ~30-40 kg. Handles on drawers shall be sturdy aluminum or steel pulls, preferably recessed to avoid snagging. A labeling holder on each drawer front is required for cataloging contents.

- Environmental Safeguards: Manuscripts in storage are highly sensitive, so the cabinets must not contribute any pollutants. All materials used should pass the PAT (ISO 18916) indicating safe contact with photographic emulsions and papers. This includes paints, gasketing materials, adhesives, etc. If drawer liners or pads are provided, they must be acid-free (pH neutral or slightly alkaline buffer for paper items) and lignin-free. No felt or foam that off-gasses should be in direct contact – if cushioning is needed, use polyethylene foam that has been tested for archival use. Ventilation for the cabinet is optional; generally, flat files are not tightly sealed, which allows the ambient climate (provided by the air-conditioned, humidity-controlled library) to percolate. The storage room conditions should meet ISO 11799 recommendations (around 18-20°C, 40-50% RH). However, to ensure stable micro-environment, the cabinet could accommodate silica gel if needed – e.g., a small slot for desiccant canisters in a drawer (though ideally the ambient environment suffices). Fire safety: While not a requirement in the brief, it’s noted that metal flat files offer some fire resistance. If fire-rated cabinets are needed, that could be a separate specification; at minimum, the steel should be thick enough to provide a heat buffer (and obviously, absolutely no flammable finishes). The units should also prevent pest ingress – tight fits and possibly pest-resistant strips (e.g., felt seals treated with insect repellent) can be considered.

- Locking & Security: The storage cabinets shall have a central locking system. A single key turn should lock all drawers simultaneously (common bar lock mechanism). The lock should be of good quality (steel cam lock with brass keys, or even a digital code lock if specified) to protect valuable manuscripts. When locked, all drawers must be secured in the closed position. Additionally, an anti-tip interlock is critical: opening one drawer should automatically lock out the others from opening, to prevent center of gravity from shifting forward. This is standard on many flat file systems and must be included for safety, especially if the cabinet is not anchored. The cabinets can be designed to be bolted together side by side (if multiple units in a row) and/or bolted to the wall or floor for earthquake safety – anchor points or brackets should be provided.

- Standards & Durability: These units should meet or exceed ANSI/BIFMA X5.9 standards for storage furniture, ensuring they have been tested for safety, strength, and longevity. This includes cycle testing of drawers (tens of thousands of open/close cycles with rated loads) and stability tests. The heavy-gauge steel, welded construction, and quality hardware specified are aimed at delivering decades of trouble-free use. All drawers should operate quietly and smoothly even when fully loaded. Paint finish shall be high-quality – resistant to chipping and corrosion (e.g., 50 µm powder coat thickness, ASTM B117 salt spray tested). Compliance with ISO 11799 means the storage meets archival standards for protecting documents – the cabinets contribute to that by providing physical protection (no bending or light exposure) and using materials that do not degrade the items. Also, per ISO 18911 (which covers photographic film storage but by extension relevant to paper), storage enclosures and cabinets should be made of non-corroding metal or inert plastic, a criterion fully met by these steel powder-coated cabinets. In summary, the manuscript storage cabinets are to be of museum-grade quality, providing safe, accessible, and long-lasting flat storage. They come complete with all necessary features (locks, labels, etc.) and carry a substantial warranty (the expectation is at least 5 years manufacturer’s warranty). Installation will include placing them on proper level bases and verifying drawer alignment and smoothness.

    - Microfilm Storage Cabinets

- Description: Purpose-built steel cabinets shall be provided for the long-term storage of microforms (including 16mm/35mm microfilm reels and microfiche jackets). Each cabinet is typically a tall unit with multiple drawers or trays designed specifically to hold microfilm in its various formats. The cabinets will be free-standing, with a footprint that optimizes space (e.g., approximately 500 mm wide × 600 mm deep for a cabinet holding many reels, and ~1800 mm height). The total quantity and configuration will depend on capacity needs, but each unit should maximize storage density while ensuring easy retrieval and compliance with archival standards.

- Materials & Build: Construct the cabinets from high-quality cold-rolled steel, minimum 1.0–1.2 mm thickness for walls and drawers, with reinforced panels to prevent flexing. All metal components must be non-ferrous or treated to resist corrosion over time – galvanized or zinc-plated steel is preferred, followed by a durable powder-coat finish. Archival standards emphasize that enclosures for film be made of non-corroding, non-deteriorating materials, hence the exclusive use of protected metal (or approved plastics) in these cabinets. The finish will be a light-colored powder coating (to allow easy visibility of any dust and discourage pest hiding), which is inert and emits no VOCs after curing. No wood is to be used internally, and any plastic or rubber components (like drawer mats or bumpers) must be stable and non-degrading (e.g., silicone rubber, polypropylene, or polyethylene that meet archival criteria). The cabinet’s structural design should include a sturdy frame and an anti-tip base (wider kick plate or provision to bolt to floor) because microfilm reels can be weighty when many are stored together.

- Storage Format & Interiors: The cabinet must accommodate microfilm rolls and fiche in an organized manner. For rolls: drawers can have separators or slots sized for standard microfilm reel boxes. For example, a common approach is shallow drawers with metal dividers creating rows where each 100′ or 100′×35mm reel in its archival box can sit upright. Alternatively, a cabinet may employ pull-out tray drawers with grooves for plastic microfilm cartridges. For microfiche: a different section or separate drawers with taller height can be provided, with adjustable wire dividers to keep fiche envelopes upright. Flexibility is key – the shelving or divider system should be somewhat adjustable to handle various microform sizes and future changes. Each drawer should extend at least 90% to allow easy access to items at the back. Inside surfaces need to be smooth and burr-free to avoid snagging film boxes or cutting personnel – all edges should be rolled or deburred. Drawers must support the weight of a full load of film without distortion (reinforced bottom panels if wide). Capacity: As a guideline, a single cabinet with five drawers might hold on the order of 1000+ rolls of microfilm (depending on drawer size and arrangement) or tens of thousands of fiche; specific capacity to be confirmed based on collection volume.

- Ventilation & Environmental Control: Microfilm is very sensitive to heat and humidity. Cabinets should allow some air exchange with the surrounding environment to avoid microclimates that trap moisture (which could promote vinegar syndrome in acetate film). At the same time, they should limit dust ingress. The design solution is to include small ventilation openings or louvered sections – typically at the top or back of the cabinet – which permit air flow but are baffled to block dust. These openings might be fitted with fine dust filters. The overall storage environment must be maintained as per ISO 18911:2010 recommendations: for extended-term microfilm storage, a cool, dry, stable climate is required (about ≤21°C and 20–30% RH). While room HVAC provides this, the cabinet should not significantly impede the environment from reaching the film. Avoid airtight seals on drawers; a slight gap is acceptable and even beneficial for airflow (besides, truly airtight enclosures are not recommended for film due to off-gassing of acetic acid from degrading acetate film). If needed, humidity-control packets (silica gel) can be placed inside drawers, but ideally, consistent external climate obviates this. It is crucial that no felt or foam lining in the drawers will trap moisture or emit VOCs. The cabinet material itself (steel) and powder coat are neutral and will not contribute to contamination or moisture; this aligns with archival best practice that storage cabinets be inert and protective. Optionally, hygrometer strips or temperature/RH data loggers could be placed in representative drawers to monitor conditions without opening them frequently.

- Security & Accessibility: Locking mechanisms should be in place to secure the microfilm collection. A central lock (ideally with a captive key or combination lock to avoid lost keys) can secure all drawers at once. Given the high value of archival microfilm, consider a lock that’s robust (e.g., a pin tumbler lock with duplicate keys controlled by the institution). Each drawer should have a handle that allows comfortable grip; consider recessed handles to avoid catching clothing. Anti-tip safety is mandatory: typically, an interlock will allow only one drawer to open at a time. This prevents accidental tipping even if drawers are heavily loaded at full extension. Additionally, the cabinet should have provisions to be bolted to a wall or another cabinet bank for increased stability (especially if very narrow but tall). Drawers should slide quietly and have an end-of-travel dampening so they don’t slam (this also protects the film contents from sudden jarring). For quick identification, each drawer or compartment will have an indexed labeling system (either holder for paper labels or an engraved plate) so staff can organize contents (e.g., by roll range or fiche number).

- Compliance & Longevity: These microfilm cabinets must adhere to ISO 18911 standards not just for environment but for materials: the standard and related guidelines (e.g., ANSI IT9.2) specify that storage enclosures for film be made of archival-quality materials and that they ensure longevity of the media. By using powder-coated steel and acid-free boxes within, this requirement is met. In practice, polyester-base silver gelatin microfilm stored under proper conditions has a life expectancy of 500+ years, and the cabinets are part of providing those “proper conditions.” Therefore, the fabrication quality should target a similarly long service life. There should be no plasticizers, paint volatiles, or outgassing substances in the cabinet that could lead to film deterioration (like causing redox blemishes or vinegar syndrome). The cabinet should also hold up mechanically over time – guides and slides must retain alignment. Testing to a furniture standard (BIFMA or equivalent) for drawer durability and tip stability is recommended to ensure robustness. All design and production processes should follow ISO 9001 quality guidelines. The delivered cabinets should come with at least a 5-year warranty, and ideally a certification or statement that they meet archival storage requirements (some manufacturers provide test results or Oddy/PAT certificates for their powder coating and gasket materials). The cabinets will protect microforms from physical damage (no bending or crushing), light (opaque metal keeps light out), and dust, while keeping them readily accessible to authorized users. In sum, they form a secure, climate-friendly, and compliant microfilm storage solution for the library.

    - Retractable Awnings (Motorized Cassette-Type)

- General Description: Provide motorized retractable awnings for two large terrace quadrants (2nd floor level), offering approximately 200 linear meters of coverage in total. The system shall use full-cassette, folding-arm awnings, meaning when retracted the awning fabric and arms are completely enclosed in a protective cassette housing. This design choice is to ensure durability in a monsoon-prone climate – the cassette shields the fabric and mechanism from rain, wind, and UV when not in use. Multiple awning units will be installed in series to cover the total span (exact sizing of each unit to be determined by manufacturer’s standard lengths and site structural considerations). The awnings will extend out to provide shade and rain cover for the terrace, and fully retract into the cassette near the building facade when not needed. They must be architecturally integrated and minimally intrusive visually (cassette can be mounted to a soffit or wall and finished to match building colors).

- Structure & Materials: The awning frames, including the cassette housing, roller tube, arms, and support brackets, shall be made of high-strength extruded aluminum alloy (6063-T5 or equivalent) for an optimal strength-to-weight ratio. All aluminum components must be powder-coated with a durable outdoor-grade finish (polyester powder coating, UV-stabilized) to prevent corrosion, even in humid and rainy conditions. The coating color will be as per architect’s approval (likely a neutral or metallic tone that blends with the facade). Critical load-bearing pieces (e.g., arm joints, connecting pins) may be of stainless steel for added strength and corrosion resistance. The folding arms shall incorporate internal tension systems – either twin stainless steel cables or double chain linkage coupled with springs – to keep the fabric taut and to absorb wind shocks. Arm profiles should be robust (minimum 50 × 20 mm cross-section or as engineering dictates) and engineered to withstand wind loads without permanent deformation. All pivot and hinge points in the arms must use maintenance-free bushings or bearings (e.g., Teflon-lined or bronze bearings) to ensure smooth operation and long life. Mounting brackets (wall or soffit mount depending on site) should be heavy gauge steel or aluminum, with at least 3 fixation points each, designed to distribute loads into the building structure safely. The structural performance of the installed awnings must meet EN 13561 requirements for wind resistance: specifically, achieve Wind Resistance Class 2 or higher, which corresponds to sustaining roughly Beaufort force 5 winds (~30–38 km/h) without damage. Higher performance (Class 3, Beaufort 6) is desirable for extra safety in sudden gusts. Supporting calculations or certifications from the manufacturer should be provided. Additionally, the arms and fabric assembly must be able to shed rainwater effectively; a minimum pitch of about 14° from horizontal when extended is recommended so that rain does not pool. The frame should include integrated rain gutters or drip edges if water runoff could otherwise enter the building or cause concentrated streams. All fasteners (bolts, nuts) used in assembly and installation shall be stainless steel (A2/A4 grade) or hot-dip galvanized, to prevent rust staining and ensure longevity.

- Awning Fabric: The awning canopy shall be made of premium solution-dyed acrylic fabric, engineered for outdoor use. This fabric must be inherently UV-resistant, fade-resistant, water-repellent, and mildew-resistant, with the color pigments extruded into the fibers for maximum colorfastness. Weight of the fabric should be on the order of 290–320 g/m², giving a balance of strength and flexibility. It should have a high UPF (Ultraviolet Protection Factor), blocking at least 90–98% of UV rays to protect occupants and underlying materials from UV exposure. A waterproofing treatment (such as fluorocarbon coating) is required so that light rain beads off; the fabric should also resist mold/mildew growth (it being breathable helps prevent condensation and mildew). Seams in the fabric (if any, for larger widths) must be heat-sealed or double-stitched with UV-stable thread (e.g., Teflon/PTFE thread) to prevent deterioration. The front edge of the fabric will be attached to an aluminum front bar which in turn connects to the folding arms. The valance (if specified) should be removable and also made of the same solution-dyed acrylic, with a scallop or straight edge as per design; it too should be UV and weather resistant. The fabric color/pattern will be chosen by the library (solid colors likely); it should have at least Class 7-8 lightfastness on the Blue Wool scale, ensuring minimal fading over years of sun exposure. When retracted into the cassette, the fabric is protected from the elements, further extending its life. Typical manufacturer warranties on such fabrics are 5–10 years; the specification should require a durable grade (e.g., Sunbrella or equivalent) with a minimum 5-year no-fade warranty. In summary, the fabric will retain its strength and appearance for many monsoon seasons, and any soiling (dust, bird droppings) can be cleaned with mild soap without damaging the material.

- Operation & Automation: The awnings will be equipped with electric tubular motors housed inside the roller tubes. These motors should be properly sized for the width and projection of each awning (with a safety margin in lifting torque). The system will allow group and individual control of the awnings. Controls include wall-mounted switches at convenient locations and remote control transmitters. Crucially, the system shall integrate weather sensors for automatic retraction: at a minimum, wind sensors that detect excessive wind and trigger the awnings to retract to avoid damage. The wind sensor could be an anemometer type or accelerometer type mounted on the lead bar – it must be calibrated to retract at a safe threshold (e.g., when winds approach Beaufort 5, well before structural limits). Additionally, rain sensors should be installed – typically rain sensors will close the awning upon detecting sustained rainfall, to prevent water pooling and to protect the fabric. (If the terrace use requires it, a sun sensor can also be included to deploy the awning when strong sunlight is detected and retract at dusk, but this is optional.) The control system will likely be a central automation unit that takes input from sensors and overrides manual commands as needed for safety. All motors and controllers must conform to IEC/EN electrical safety standards and be rated for outdoor use (motor casings IP54 or better for dust/water resistance). A manual override (hand crank) is required for each awning motor, allowing it to be closed in case of motor or power failure. The automation should have obstacle detection – if an awning is retracting and encounters an obstruction (or is hindered), the motor should stop to prevent damage. Wiring for power and sensors will be concealed in cable ducts or within the building structure as much as possible; weatherproof junction boxes and conduits to be used for any exposed runs. The entire system should be integrable with the building management system (BMS) if present, or at least have a central disable switch (for maintenance).

- Wind Resistance & Safety Features: Because these awnings are in a location subject to high winds (including storm gusts during monsoon), safety is paramount. The specified Class 2 wind resistance (or better) per EN 13561 means the awning can remain extended in moderate winds (~Beaufort 5). In practical terms, any wind stronger than that should trigger automatic retraction via the sensors. The arms and mounting must endure not just steady wind but turbulence and sudden gusts; hence, factor of safety in the arm design and bracket anchoring should be high (wind force on a fully extended awning of large size can be significant). The supplier should provide data or simulation results for wind load capacity. If a higher class awning (Class 3) is available and within budget, it is recommended to further ensure performance in windy areas – Class 3 would correspond to ~Beaufort 6 (strong breeze, 39–49 km/h). In any case, during very high winds or storms, awnings must be retracted; the system should fail-safe (e.g., if power is lost and wind is high, the manual override can be used quickly). The structure must also account for rainwater: while not intended to hold a lot of weight, the awning fabric under a downpour will catch water until it runs off. The arms and fabric tension should support this without tearing. Awnings should never be left out in extreme weather – this will be stated in O&M manuals – but the design should cover typical monsoon rain intensity. The front bar should have integrated channels or drip edges to guide runoff to the sides rather than letting water dump in the middle. All moving parts like arm joints should incorporate locking pins or safety straps to prevent the arms from flipping upwards in case of sudden wind uplift. When retracted, the cassette should have a locking mechanism or tight closure that keeps the awning securely rolled up and arms folded, so wind cannot deploy or flutter it (many systems have a motor brake and/or a clip lock for this purpose). Every unit should have limit switches set so that it stops at full extension (to prevent over-stressing the arms) and at full retraction (so the motor stops when the awning is safely in the cassette). The installation contractor must strictly follow the manufacturer’s guidelines for substrate and fasteners – likely using expansion anchors or through-bolts into concrete beams, etc., at spacing dictated by load calculations. After installation, each awning’s operation (manual and automatic) and sensor function should be tested in various conditions.

- Standards & Compliance: Aside from EN 13561 (which governs many aspects like operating effort, wind load, and endurance), the awning system components should carry a CE mark (for those sourced from Europe) or equivalent certification demonstrating conformity. This implies compliance with the Machinery Directive, Low Voltage Directive, EMC Directive, etc., for the motor and controls. If there are any relevant Indian Standards (IS) for awnings or outdoor fabrics (e.g., IS for textiles weather resistance), they should be complied with – typically international standards are referenced. The fabric should meet fire safety standards for awnings (for instance, NFPA 701 or equivalent) if the application demands flame retardancy – not explicitly stated, but often public buildings consider this. IGBC considerations: the powder-coated aluminum structure and acrylic fabric are both low in VOC emissions (and often recyclable at end of life). By using a long-lasting, modular shading device, the design supports energy efficiency (reducing solar gain) – this can contribute to green building points. All paints and materials used should be free of heavy metals and toxic chemicals (RoHS compliant motors, lead-free paints). Documentation should be provided for the fabric’s UV resistance and mold resistance claims (often per ASTM or ISO fabric tests). The vendor should also supply an operation & maintenance manual and training, since proper use (retraction in high winds, periodic cleaning of fabric, etc.) will be essential to the longevity and safety of the system.

- Warranty & Maintenance: The awning assembly (fabric, motor, structure) shall come with robust warranties: generally, ~10 years on the aluminum hardware and 5 years on the fabric and motor is expected at this quality level. The supplier must assure availability of spare parts (fabric panels, arm components, motors) for a reasonable period (at least 10 years). Maintenance requirements are minimal but important: periodic inspection of brackets, tightening of any bolts, cleaning the fabric (per manufacturer’s guidelines, e.g., with mild soap every 6 months), and checking the sensor calibration. The motors are typically maintenance-free but might need occasional limit adjustment. The specification should call for all sensor systems to be tested at handover (e.g., simulate high wind to see auto-retraction). Because of the scale (200 m coverage), the units should have a centralized control option to retract all awnings quickly in case of an incoming storm. Safety notices (like “Awnings must be retracted in winds above X km/h”) should be provided to facility managers. In conclusion, the retractable awning system shall be a state-of-the-art installation that provides shade and rain protection while withstanding the region’s harsh weather. It will comply with international standards for performance and safety, use top-grade materials (solution-dyed acrylic and powder-coated aluminum) for longevity, include smart automation (wind/rain sensors) for protection, and be backed by warranties – ensuring a reliable, low-maintenance shading solution for the library’s terraces.

    - Heavy-Duty Library Book Trolley

## 1. Scope - Design, manufacture, supply, and installation of heavy-duty, low-noise, maneuverable book trolleys for library operations (shelving, circulation, sorting, inter-stack movement). Trolleys shall be robust for continuous daily use, ergonomically safe, and finished for long service life in an indoor, air-conditioned environment.

## 2. Capacity & Configuration

- Payload (minimum): 200 kg uniformly distributed per trolley without permanent deformation.

- Typical form factors (any or mix as scheduled):

  - A-Frame (double-sided), 2 or 3 tiers, sloped shelves.

  - U-Frame (single-sided), 2 or 3 tiers, sloped or flat shelves.

  - Tub/return trolley with removable bins for returns & re-shelving.

- Shelf geometry:

  - Usable shelf length: 800–900 mm per side.

  - Shelf depth: 250–300 mm (books), 350 mm (folios/oversize variant).

  - Shelf pitch (sloped types): 7–12° with 30–40 mm front lip (book retainer).

  - Adjustability: at least 2 positions per shelf (flat/sloped) or interchangeable flat and sloped shelves.

## 3. Ergonomics & Handling

- Overall height: 1000–1100 mm (top handle).

- Handle height: 950–1050 mm; grip Ø 30–38 mm; continuous push bar both ends (for two-way use) with non-slip sleeve.

- Trolley footprint: ≤ 950 (L) × 520 (W) mm for standard model; turning aisle ≤ 1200 mm.

- Noise: rolling and structure-borne noise < 65 dB(A) on finished floors at rated load.

- Edges: all corners radius ≥ 2.0 mm; no burrs; finger-safe gaps.

## 4. Materials & Construction

- Frame/uprights: CRCA/MS tube/formed sections ≥ 1.6 mm thick; welded or bolt-together with structural gussets.

- Shelves:

  - Steel ≥ 1.0 mm with swaged/stiffened ribs or 18 mm HDF/ply with 1.0 mm HPL both faces & 2 mm PVC edge.

  - Flat shelves to include anti-drum pads or EVA liners to cut noise & shock.

- Base: boxed steel base with integrated caster mounting plates (no single-point screw studs).

- Fasteners: zinc-plated 8.8 grade or SS; nyloc nuts where accessible.

- Bump protection: full-perimeter replaceable thermoplastic bumper (≥ 10 mm thick) at skirting height + corner guards.

- Book stops/dividers: removable steel/ABS dividers at 150–250 mm pitch; tool-less fit.

## 5. Surface Preparation & Finish

- Pretreatment: multi-stage degrease + phosphate.

- Powder coat: thermoset epoxy-polyester; DFT ≥ 60–80 μm, smooth matte/satin; hardness ≥ HB; adhesion 4B/5B (cross-hatch).

- Boards (if used): E1/E0 low-emission; laminate wear-resistant; no exposed raw edges.

- Colour: as approved (library palette); contrasting edge/bumper optional for visibility.

- VOC/Green: low-VOC coatings/adhesives; materials recyclable; IGBC-aligned.

## 6. Mobility System (Casters)

- Quantity: 4 swivel casters; 2 with total-lock brakes (wheel + swivel).

- Diameter: 125 mm (5") minimum; tread non-marking PU/TPR; shore A 80–95.

- Bearings: sealed precision ball bearings (wheel) + thrust bearing (swivel).

- Load rating: ≥ 120 kg per caster (≥ 480 kg aggregate safety margin).

- Fixtures: top plate with 4-bolt pattern to reinforced base; SS fasteners.

- Debris guards: integral hair/string guards; optional floor-friendly thread guards.

- Performance: zero flat-spot at 24 h standstill under full load; low starting/rolling resistance.

## 7. Stability & Safety

- Anti-tip: CG within wheelbase at full load; tip test on 10° ramp without overturning (brakes applied).

- Brake hold: total-lock must hold at 5° slope with full load.

- Hand safety: crush/pinch protection at handles; shelf lips rounded; no exposed fastener threads.

- Labels: SWL (safe working load), model, serial, month/year of manufacture, maintenance cues.

## 8. Dimensions (reference standard model)

- Double-sided A-frame, 3-tier: 900 (L) × 500 (W) × 1050 (H) mm; shelf clearances 320/320/320 mm; lip 35 mm; slope 10°.

- Single-sided U-frame, 2-tier (oversize): 900 × 520 × 1050 mm; top flat shelf 350 mm deep, bottom flat 350 mm; lip 25 mm.

(Exact sizes to be finalized in shop drawings to suit book formats & aisle widths.)

## 9. Optional Variants / Add-Ons

- RFID/return bins: removable tote boxes (food-grade PP) with barcode/RFID tag.

- Document cradle: soft cradles for folios/rare books (inert foam + fabric).

- Anti-static kit: ESD strap/chain for archives where static control is needed.

- Accessories: clip-on label rails, end-panel signage, handheld scanner holster.

## 10. Testing & Acceptance (factory + site)

- Static load test: 200 kg UDL for 24 h; no permanent deformation (> 2 mm shelf sag) or weld/fastener failure.

- Dynamic roll endurance: 10 km at 3–4 km/h over mixed surface incl. 3 mm thresholds every 10 m at full load; no caster flat-spot, loosening, or structural cracks.

- Brake test: hold on 5° incline for 1 h, no creep.

- Tip test: on 10° side/fore-aft tilt without overturn.

- Finish tests: adhesion (ASTM D3359), impact (ASTM D2794), and 240 h salt-spray (ASTM B117) on coupons—no blistering/rust creepage > 2 mm.

- Noise check: ≤ 65 dB(A) at operator ear height on polished floor, full load, steady run.

## 11. Compliance & References

- Steel sheet/tube: CRCA to IS quality (e.g., IS 513/IS 1079 grade or equivalent), structural MS to IS 2062 or equivalent.

- Powder coating & pretreatment: good industry practice (phosphating before powder); finish free of pinholes/runs.

- Casters & wheels: industrial grade conforming to recognized norms (e.g., ISO/EN industrial caster performance) for load & endurance.

- Ergonomics: anthropometrics per adult workforce; handles within 50th–95th percentile reach; push forces minimized via bearing/tread selection.

- Environment: low-VOC components; boards E1/E0; recyclable steel/aluminum; packaging recyclable.

(Where a cited Indian Standard is not explicit for trolleys, international/industry tests above shall govern.)

## 12. Documentation & Deliverables

- Shop drawings: GA, sections, shelf details, caster plates, bumper profiles, clear dimensions & tolerances.

- QA pack: material mill certificates, finish test reports, caster load ratings, weld maps.

- O&M manual: cleaning, lubrication (if any), caster replacement, brake adjustment, spare parts list.

- Warranty: 3 years comprehensive on structure, welds, casters, and finish (excluding abuse/abrasion from misuse).

- Spares: 2% casters (each type), bumpers, handle grips, fastener kit per batch.

## 13. Execution

- Samples/Mock-up: 1 pre-production trolley of each type for approval (fit/finish, roll test, noise).

- Packing: corner-protected, film-wrapped; KD (knock-down) or fully assembled as approved; all fixings bagged & indexed.

Installation/Commissioning: brake function check, caster torque check, leveling, operator handover & safety briefing.

###### NUMBER OF DOCUMENTS AND COPY RIGHTS:

    - All the documents/drawings, designs, reports, and any other details envisaged under this agreement shall be supplied in six copies. If there is any revision in any drawing/document for any reason, six copies of drawing/document shall be re-issued along with soft copy in CD without any extra charges. All these drawings will become the property of the Engineer- in-Charge. The Engineer-in- Charge may use these drawings in part or full in any other work without any notice to the consultant and without any financial claim of the consultant.

    - The drawings cannot be issued to any other person, firm or authority or used by the Consultant for any other project. No copies of any drawings or documents shall be issued to anyone except the Engineer-in-Charge and / or his authorized representative.

    - The Design should cover the following general requirements:

      - The tentative space management of the library building is enclosed as separate document with this tender along with the tentative floor plans. However, these are indicative and may undergo changes during preparation of detail designing according to the space requirements.

      - To cater for different functional requirements of user with creative indoor spaces, surroundings, better circulation, and flexibility in space planning.

      - Integrated designs of electrical, mechanical, and other services with structural system and construction methodology with low maintenance.

      - Climate responsive Architecture with integration of daylight and electric light, thermal comfort, ventilation, and highest performance standards for workspace efficiency.

      - All the spaces in the building provided shall be adequately ventilated for light and air.

      - Suitable escapes for fire shall be planned as per the requirement specified in NBC and other applicable standard codes of practice

      - The setbacks and height of the building shall conform to all regulatory authority rules.

      - Premium quality materials shall be provided for walls, floors, windows, doors etc. in tune with the industry standards of similar buildings.

      - Proper care shall be taken to plan movement logistics to avoid crisis – crossing of traffic.

      - The façade of the buildings shall be appealing, by judicially mixing the use of energy efficient glass, cladding materials, wall appropriate to the use of the building. The ratio of glass to wall shall vary depending on the direction and as required for 5 STAR GRIHA-LD certification.

      - Space planning for required services shall be given importance.

      - Service routing and ducts shall be planned in coordination with the Architect Consultant appointed by NU and to provide easy access, maintenance, and scalability.

      - Green building materials shall be used as far as possible for obtaining 5 STAR GRIHA-LD rating by the accredited bodies.

      - Preparation of Data sheet showing Room wise and building wise finishing, flooring and Door window, and other high-end inventory schedules.

      - Preparation of Technical Specification for interior, furniture, furnishing etc. for all items and submit the data sheet.

GENERAL CONDITIONS FOR WORKS

  - General specifications, for construction

The order of precedence in case of any confusion/dispute will be as follows:

        - GFC drawings as approved by the University.

        - Description of nomenclature of items in bidding documents

        - Description of user requirement and technical specifications.

        - Particular specifications and special conditions for civil, electrical and horticulture works.

        - CPWD Specifications with up-to-date correction slips for civil, electrical and horticulture works as applicable.

        - Indian Standard Specifications of B.I.S.

        - National Building codes 2016

        - Manufacturer’s specifications

        - CPWD GCC 2020 for EPC project

        - Sound engineering practices

  - A reference made to any Indian Standard Specifications in this NIT, shall imply to the latest version of that standard, including such revisions / amendments as issued by the Bureau of Indian Standards up to last date of receipt of tenders. The Contractor shall keep at his own cost all such publications of relevant Indian Standard applicable to the work at site.

  - Agency shall make his own arrangement of water, electricity & generator to be used in work. The University will not provide the above facility.

  - Samples including brand / quality of materials and fittings to be used in the work shall be got approved from the University, well in advance of actual execution and shall be preserved till the completion of the work.

  - The work shall be executed and measured as per metric dimensions given in the Schedule of quantities, drawings etc. (F.P.S. units wherever indicated are for guidance only).

  - Unless otherwise specified, the rates tendered by the contractor shall be inclusive of all cost, taxes, Royalty payable, seigniorage charges on Royalty and shall apply to all leads and lifts and nothing extra shall be payable on this account.

  - The rates for all items of work shall, unless clearly specified otherwise, include cost of all labour, material, tools and plants and other inputs involved in the execution of the item including the fee of consultancy etc.

  - No foreign exchange shall be made available by the University for importing (purchase) of equipment, plants, machinery, materials of any kind or any other items required to be carried out during execution of the work. No delay and no claim of any kind shall be entertained from the Contractor, on account of variation in the foreign exchange rate.

  - All ancillary and incidental facilities required for execution of work like, stores, fabrication yard, offices for Contractor & Consultants, watch and ward, temporary structure for plants and machineries, temporary fencing around the working sites, installation and consumption charges of temporary electricity, telephone, water etc. required for execution of the work, protection works, testing facilities / laboratory charges, facilities for all field tests and for taking samples etc. during execution or any other activity which is necessary (for execution of work and as directed by Engineer-in-Charge), shall be deemed to be included in rates quoted by the EPC Contractor, for various items in the schedule of quantities. Nothing extra shall be payable on these accounts. Before start of the work, the Contractor shall submit to the Engineer-in-Charge, a site / construction yard layout, specifying areas for construction, site office, positioning of machinery, material stocking yard, fabrication yard, etc. Contractor shall only be permitted for construction of any type of temporary labour hutment in the working site at the designated location as approved by the E-in-C.

  - For completing the work in time, the Contractor might be required to work in two or more shifts (including night shifts). No claim whatsoever shall be entertained on this account, not with-standing the fact that the Contractor may have to pay extra amounts for any reason, to the labourers and other staff engaged directly or indirectly on the work according to the provisions of the labour and other statutory bodies regulations and the agreement entered upon by the Contractor with them. The work in night shift and /or any holiday shall be done only after informing the E-in-C and in presence of EPC contractor technical team.

  - All material shall only be brought at site as per program finalized with the Engineer-in-Charge. Any pre-delivery of the material not required for immediate consumption shall not be accepted and thus may not paid for.

  - Quality Assurance:

  - The contractor shall ensure quality control measures on different aspects of construction including materials, workmanship and correct construction methodologies to be adopted.

  - The contractor shall get the source of various raw materials to be used on the work, approved from the Engineer-in-Charge and the contractor shall stick to the approved source unless it is absolutely unavoidable. Any change shall be done with the prior approval of the Engineer-in-Charge for which tests etc. shall be done by the contractor at his own cost.

  - Similarly, the contractor shall submit brand/make of various materials to be used for the approval of the Engineer-in-Charge along with samples and once approved, he shall stick to it. Any change will have to be got approved from Engineer-in- charge in advance.

  - THIRD PARTY QUALITY ASSURANCE: In order to achieve a high standard of quality, it shall be required to go for Third Party Quality Assurance. For this purpose, a separate agency shall be appointed by the University who will carry out independent testing of materials and checking and ensuring overall quality procedures. The contractor shall be required to fully cooperate with agency and facilitate them in taking samples, transportation and examination of various activities including documentation at no extra time and cost to the University. In case of any adverse findings by the agency, the contractor shall do the needful rectifications at no extra time and cost to the University. The Engineer- in-charge shall be at liberty for getting quality assurance work done through agencies like IIT, NIT, Govt. Engineering College or any other agency approved by Competent Authority, the fee of third-party quality agency shall be borne by the University. The successful tenderer shall include the provisions of Quality Assurance while framing the proposed methodology for tests.

  - Safety Precautions – Contractor shall within two weeks of award of work, submit to the Engineer-in-Charge for his approval, list of measures for maintaining safety of manpower deployed for the work and avoidance of accidents. Contractor will be fully responsible to ensure safety of workers deployed at site.

  - Scaffolding – For façade works, outer finishing and other relevant works at height etc. double steel scaffolding having two sets of vertical supports with steel staircase for inspection of works by Engineer-in-charge or his/her representatives shall be used. The supports shall be sound and strong, tied together with horizontal piece and also secured horizontally with the building as per EHS norms, over which scaffolding planks shall be fixed.

  - SAMPLES FOR TESTING:

  - Samples of all materials required for testing is included in the cost of work. Therefore, all testing charges in house or through external lab shall be borne by the EPC contractor.

  - If any load testing or special testing is to be done for any sample whose strength is doubtful, the cost of the same shall also be borne by the contractor.

  - In case there is any discrepancy in frequency of testing as given in list of mandatory tests and that in individual sub-heads of work as per CPWD Specifications, higher of the two frequencies of testing shall be followed.

- The contractor should submit for approval of Engineer-in-Charge workshop drawings, technical submittals, and samples of the work to be performed under the specified items of work before actually commencing the mass execution of the work under the item. For this they will prepare a sample room / quarters and toilet blocks for each type of building for approval of Engineer-in-charge of work. Nothing extra shall be payable on this account.

- Maintenance of Register of Tests –

  - All the registers of tests carried out at construction site or in outside laboratories shall be maintained by the contractor which shall be issued to the contractor by Engineer-in-Charge.

  - All samples of materials including cement concrete cubes shall be taken jointly with contractor by the representatives of NU Engineering Section and Engineers of PMC (MECON Ltd). All the necessary assistance shall be provided by the contractor. Cost of sampling & testing are to be borne by the contractor and he shall be responsible for safe custody of samples to be tested at site/ outside laboratory.

  - All the tests in field lab at construction site shall be carried out by the Engineering staff deployed by the contractor and shall be 100% witnessed by JE and 50% of tests shall be witnessed by AE –in- Charge.

  - At least 10% of the tests are to be witnessed by the E-in- C. Minimum 25% of all samples should be tested in outside approved laboratory/ Govt. Engineering colleges.

  - All the entries in the registers will be made by the designated Engineering Staff of the contractor and same should be regularly reviewed by NU Engineering Section and Engineers of PMC.

1.1. Contractor shall be responsible for safe custody of all the test registers.

  - Submission of copy of all test registers, Material at site register and hindrance register along with each alternate Running Account Bill and Final Bill shall be mandatory. These registers should be duly checked by NU Engineering Section and Engineers of PMC and receipts of registers should also be acknowledged by the PMC.

  - If all the test registers and reports are not submitted along with alternate R/A Bill & Final Bill, no payment will be released to the contractor.

  - Maintenance of Material at Site (MAS) Register – All the MAS Registers including cement and Steel Registers which shall be issued to the contractor by Engineer-in- Charge shall be maintained by Contractor at site and shall be open for inspection by the

University.

  - The contractor shall submit to the Engineer-in-charge on the 7th day of each month, 2 hard copies and one on soft copy (CD) of monthly progress report of work. Such progress report will include the project progress, summary, work progress (planned vs actual), CPM chart, status of financial progress and achievement of milestone, manpower deployment status, inventory of materials and photographs of important activities. For delay in submission of the report, compensation @ Rs. 2,000/- (Rupees Two Thousand only) per day of delay subject to maximum of Rs. 20, 000/- for each report will be recovered from the amount payable to the contractor.

  - Contractor(s) shall pay all fees, taxes and charges which may be levied on account of any construction activity including transportation of material in executing the contract.

  - The Contractor(s) shall take all precautions to avoid accidents by exhibiting necessary caution boards day and night, speed limit board, red flags, red lights and providing barriers and use of PPE gadgets as per EHS plan approved by E-in-C. He shall be responsible for all damages and accidents caused to existing/new work due to negligence on his part. No hindrances shall be caused to traffic during the execution of the work. In case of any accident of labours / contractual staff’s the entire responsibility will rest on the part of the contractor and any compensation under such circumstances if becomes payable the same shall be entirely borne by the contractor and department shall have no role on this account.

  - The contractor(s) shall take instructions from the Engineer-in- Charge regarding collection and stacking of materials at any place. No excavated earth or building rubbish shall be stacked on areas where other buildings, roads, services and compound walls are to be constructed. The stacking shall take place as per stacking plan however, if any change is required, the same shall be done with the approval of Engineer-in- Charge.

  - Contractor(s) shall provide permanent bench marks, flag tops and other reference points for the proper execution of work and these shall be preserved till the end of the work. All such reference points shall be in relation to the levels and locations, given in the Architectural and plumbing drawings.

  - Contractor shall put up the barricading, at his cost, all around the buildings sites through suitable method for segregating the construction site and also to control the dust pollution in the campus. Entry to all the construction sites shall be controlled for proper security of man and materials and to avoid accidents.

  - On completion of work, the Contractor(s) shall submit at his own cost four prints of “AS BUILT” drawings to the Engineer-in-Charge. These drawings shall have the following information:

    - Run off all piping and their diameters including soil waste pipes and vertical stacks.

    - Ground and invert levels of all drainage pipes together with locations of all manholes and connections, up to outfall.

    - Run off all water supply line with diameters, locations of control valves, access panels etc.

- Water tanks, taps, sanitary, water supply and drainages pipes, fittings and accessories should conform to the specifications provided in bidding documents, if CPWD Specifications are not available, NBC – 2016, IS codes shall be followed. The contractor(s) should engage approved, licensed plumbers for the work and get the materials (fixtures/fittings) tested, by the municipal Body/Corporation authorities wherever required at his own cost. The Contractor shall submit for the approval of the Engineer-in- Charge the name of the plumbing Agency proposed to be engaged by him.

- The contractor shall give performance test of the entire installation(s) as per the specifications & codes in the presence of the Engineer-in-charge or his authorized representative before the work is finally accepted and nothing extra what-so-ever shall be payable to the contractor for the test.

- Any cement slurry added over base surface for continuation of concerting for better bond is deemed to have been built in the items and nothing extra shall be payable and no extra cement considered in consumption on this account.

- The Contractor shall bear all incidental charges for cartage, storage and safe custody of materials issued by department/arranged by the contractor.

- WATER PROOFING TREATMENT OF ALL TYPES OF WORK:

The water proofing work shall be carried out by specialized water proofing agencies. The Contractor shall submit for the approval of the Engineer-in- Charge, the names of specialized agencies, of repute along with their technical capability proposed to be engaged by him, who have executed satisfactorily a minimum of three works of value not less than 40% of corresponding value each or two works of value not less than 60% each or one work of value not less than 80% of corresponding amount in the last five years. For calculation purpose only, cost of waterproofing works will be taken as Rs. 1.00 Cr. The water proofing should be full proof & use of crystalline material may also be done in combination of other water proofing (modern and sound) method to be doubly sure.

- ALUMINIUM/GLASS WORK:

The aluminium/glass work shall be carried out by specialized agency having adequate workshop with necessary equipment and having the experience in aluminium/glass works. The Contractor(s) shall submit for the approval of the Engineer-in-Charge, the names of specialized agencies, of repute along with their technical capability proposed to be engaged by him, who have executed satisfactorily a minimum of three works of value not less than 40% of corresponding value each or two works of value not less than 60 % each or one work of value not less than 80% of corresponding amount in the last five years. For calculation purpose only, cost of aluminium/glass works will be taken as Rs. 3.00 Cr.

- STRUCTURAL GLAZING WORK:

The structural glazing work shall be carried out by specialized agency having adequate workshop with necessary equipment and having the experience in structural glazing works. The Contractor(s) shall submit for the approval of the Engineer-in-Charge, the names of specialized agencies, of repute along with their technical capability proposed to be engaged by him, who have executed satisfactorily a minimum of three works of value not less than 40% of corresponding value each or two works of value not less than 60 % each or one work of value not less than 80% of corresponding amount in the last five years. For calculation purpose only, cost of structural glazing works will be taken as Rs. 1.25 Cr.

- The work shall be carried out in accordance with the approved Architectural drawings and structural drawings, to be prepared and submitted by architectural/structural consultants engaged by the EPC contractor, duly vetted by the proof checking reputed institute and approved by the Engineer-in-Charge. Before commencement of any item of work the contractor shall correlate all the relevant architectural and structural drawings, nomenclature of items and specifications etc. issued for the work and satisfy himself that the information available there from is complete and unambiguous. The figure and written dimension of the drawings shall be superseding the measurement by scale. The discrepancy, if any, shall be brought to the notice of the Engineer-in-charge before execution of the work. The contractor alone shall be responsible for any loss or damage occurring by the commencement of work on the basis of any erroneous and or incomplete information and no claim whatsoever shall be entertained on this account.

- Other agencies/sub-contractor will also simultaneously execute and install the works of & other specialized equipment as indicated in bid document, lifts, fire-fighting etc. of this work and the contractor shall extend necessary facilities for the same. The contractor shall leave such recesses, holes, opening etc. as may be required for the electric/ gas pipe lines and other related works and nothing extra shall be payable on this account.

- The contractor shall conduct his work, so as not to interfere with or hinder the progress or completion of the work being performed by other contractor(s) or by the Engineer-in- Charge and shall as far as possible arrange his work and shall place and dispose of the materials being used or removed, so as not to interfere with the operations of other contractor simultaneously working or he shall arrange his work with that of the others in an acceptable and coordinated manner and shall perform it in proper sequence to the complete satisfaction of others.

- PROGRAMME CHART - The contractor shall submit a Time and Progress Chart for each mile stone. The Engineer-in-Charge may within 15 days thereafter, if required modify, and communicate the programme approved to the contractor failing which the programme submitted by the contractor shall be deemed to be approved by the Engineer-in-Charge. The work programme shall include all details of balance drawings and decisions required to complete the contract along with material/equipment ordering & delivery plan with specific dates by which these details are planned by contractor without causing any delay in execution of the work. The chart shall be prepared in direct relation to the time stated in the contract documents for completion of items of the works. It shall indicate the forecast of the dates of commencement and completion of various trades of sections of the work and may be amended as necessary by agreement between the Engineer-in-Charge and the Contractor within the limitations of time imposed in the Contract documents, and further to ensure good progress during the execution of the work, the contractor shall in all cases in which the time allowed for any work, exceeds one month (save the special jobs for which a separate programme has been agreed upon) complete the work as per mile stones given in Schedule ‘F’.

- In case of non-submission of construction programme by the contractor the program approved by the Engineer-in-Charge shall be deemed to be final.

- The approval by the Engineer-in-Charge of such programme shall not relieve the contractor of any of the obligations under the contract.

- The contractor shall submit the Time and Progress Chart and progress report using the mutually agreed software and format as decided by Engineer-in-charge for the work done during previous month to the Engineer-in-Charge on or before 5th day of each month failing which a recovery Rs. 5,000/- shall be made on per week or part basis in case of delay in submission of the monthly progress report.

- If the work is carried out in more than one shift or during night, no claim on this account shall be entertained.

- Existing drains, pipes, cables, over-head wires, sewer lines, water lines and similar service encountered in the course of the execution of work shall be protected against the damage by the contractor at his own expense. In case the same are to be removed and diverted. The same shall be payable to the contractor. The contractor shall work out the cost and the same shall be approved by Engineer-in-Charge. The contractor shall not store materials or otherwise occupy any part of the site in a manner likely to hinder the operation of such services.

- The contractor shall be responsible for the watch and ward / guard of the buildings’ safety, fittings and fixtures provided by him against pilferage and breakage during the period of installations and thereafter till the building is physically handed over to the University. No extra payment shall be made on this account.

- SAMPLE OF MATERIALS - BIS marked materials except otherwise specified shall be subjected to quality test at the discretion of the Engineer-in-Charge besides testing of other materials as per the specifications described for the item/material. Wherever BIS marked materials are brought to the site of work, the contractor shall, if required, by the Engineer-in-Charge furnish manufacturer’s test certificate or test certificate from approved testing laboratory to establish that the material/ procured by the contractor for incorporation in the work satisfies the provisions of specifications/BIS codes relevant to the material and / or the work done.

- For certain items, if frequency of tests not mentioned in the CPWD Specifications and then relevant IS code shall be followed and tests shall be carried out as per the frequency specified therein.

- The contractor shall render all help and assistance in documenting the total sequence of this project by way of photography, slides, audio-video recording etc. nothing extra shall by payable to the contractor on this account. However, cost of photographs, slides, audio-videography etc. shall be borne by the University.

- The contractor shall be fully responsible for the safe custody of materials brought by him/issued to him even though the materials may be under double lock and key system.

- The contractor shall procure the required materials in advance so that there is sufficient time for testing of the materials and approval of the same before use in the work. The contractor shall provide at his own cost suitable weighing and measuring arrangements at site for checking the weight / dimensions as may be necessary for execution of work. The sealed samples are to be handed over to the testing lab by contractor in the presence of PMC representatives.

GENERAL CONDITIONS OF CONTRACT

(As per relevant clauses of CPWD GCC for EPC Projects 2024 read along with correction slips issued time to time as on date)

SECTION 5

SPECIAL CONDITIONS OF CONTRACT

Definitions

The Contract means the documents forming the tender and acceptance thereof and the formal agreement executed between the Competent Authority, Nalanda University and the Contractor, together with the documents referred to therein including these conditions, the specifications, designs, drawings and instructions issued from time to time by the Engineer-In-Charge/the Competent Authority at NU and all these documents taken together, shall be deemed to form one contract and shall be complementary to one another. In the contract, the following expressions shall, unless the context otherwise requires, have the meanings, hereby respectively assigned to them:-

Work:

The expression work or works shall, mean unless there be something either in the subject or context repugnant to such construction the context otherwise requires be construed and taken to mean the works by or by virtue of the contract contracted to be executed whether temporary or permanent, and whether original, altered, substituted or additional.

Site:

The Site shall mean the land/or other places on, into or through which work is to be executed under the contract or any adjacent land, path or street through which work is to be executed under the contract or any adjacent land, path or street which may be allotted or used for the purpose of carrying out the contract.

Contractor:

The Contractor shall mean the individual, firm or company, whether incorporated or not, undertaking the works and shall include the legal personal representative of such individual or the persons composing such firm or company, or the successors of such firm or company and the permitted assignees of such individual, firm or company.

Owner:

Owner shall mean the Nalanda University, Rajgir, BIHAR.

Project Manager:

Means the authorized Engineer Officer nominated by the Project Management Consultant appointed by Nalanda University. He will report to the Engineer-In-Charge and will be responsible for day to day supervision of project execution and will assist Engineer-in-Charge in Planning, Quality Assurance and Control, Execution and monitoring the Progress of work.

Architect Consultants:

Architect Consultants shall mean the Architect Consultant appointed by NU including its associate architects and their authorized consultants and Engineers to design and assist the Engineer In Charge in monitoring the Architectural, Structural, Electrical, Plumbing, Acoustic, HVAC, Landscape and Infrastructure works and also to check quality, project scheduling, cost control, installation of systems and equipment in the Works and ensure that the same is done as per the drawings approved.

Engineer-in-Charge:

Engineer-in-Charge (EIC) shall mean the authorized Engineer Officer appointed by NU, who shall supervise and be in charge of the work. Engineer-in-Charge will administer the contract with the assistance of his authorized subordinate Engineers.

Accepting Authority:

Accepting Authority shall mean the authority mentioned in Schedule ‘F’.

Excepted Risk:

Excepted Risk are risk due to riots (other than those on account of Contractor’s employees), war (whether declared or not) invasion, act of foreign enemies, hostilities, civil war, rebellion revolution, insurrection, military or usurped power, any acts of Owner damage from aircraft, acts of God, such as earthquake, lightening and unprecedented floods and other causes over which the Contractor has no control and accepted as such by the Accepting Authority (i.e. either the Owner or an Agency appointed by the Owner) or causes solely due to use or occupation by Owner of the part of the Work in respect of which a certificate of completion has been issued or a cause solely due to Owner’s faulty design of works..

Market Rate:

Market rate shall be the rate as decided by the Engineer-in-Charge in consultation with Architect Consultants and Project Management Consultant (PMC) on the basis of the cost of materials and labour at the site where the Work is to be executed plus the percentage mentioned in Schedule ‘F’ to cover all overheads and profits of the Contractor and approved by the NU, provided that no extra overheads and profits shall be payable on the part(s) of work assigned to other agency(s) by the contractor as per terms of contract. Schedule(s) referred to in these conditions shall mean the relevant schedule(s) annexed to the tender documents or the standard Schedule of Rates of the government mentioned in Schedule ‘F’ hereunder, with the amendments thereto issued up to the date of receipt of the tender.

Tendered value:

Tendered value / Contract value means the value of the entire Work as stipulated in the letter of award.

Date of commencement of work:

The Date of commencement of work shall be the date when contract comes into existence i.e. the date the Letter of Acceptance by NU is awarded or the first date of handing over of the site, whichever is later, in accordance with the phasing if any, as indicted in the tender document.

###### GST

GST shall mean Goods and Service Tax – Central, State and Inter State.

Scope & Performance

Where the context so requires, words imparting the singular only also include the plural and vice versa. Any reference to masculine gender shall whenever required include feminine gender and vice versa.

The contractor shall be furnished, free of cost one certified copy of the contract documents including specifications, Schedule of Rates and such other printed and published documents, together with all drawings as may be forming part of the tender papers. None of these documents shall be used for any purpose other than that of this contract.

Works to be carried out

The Work to be carried out under the Contract shall, except as otherwise provided in these conditions shall include all labour, materials, tools and plants, equipment and transport to be provided at the cost and expense of the Contractor which may be required in preparation of and for and in the full and entire execution and completion of the Works. The descriptions given in the Schedule of Quantities (Schedule A) shall unless otherwise stated, be held to include wastage on materials, carriage and cartage, carrying and return of empties, hoisting, setting, fitting and fixing in position and all other labours necessary in and for the full and entire execution and completion of the Works as aforesaid in accordance with good practice and recognized principles.

Sufficiency of Tender:

The Contractor shall be deemed to have satisfied himself before tendering as to the correctness and sufficiency of his tender for the works and of the rates and prices quoted in the Schedule of Quantities and Tender Documents, which rates and prices

shall, except as otherwise provided, cover all its obligations under the contract and all matters and things necessary for the proper completion and maintenance of the works.

Discrepancies and Adjustment of Errors:

The several documents forming the Contact are to be taken as mutually explanatory of one another, detailed drawings being followed in preference to small scale drawing and figured dimensions in preference to scale and special conditions in preference to General Conditions.

In the case of discrepancy between the schedule of Quantities, the Specifications and / or the Drawings, the following order of preference shall be observed:-

I) Description of Schedule of Quantities.

- Particular Specifications

- Special Conditions.

- Additional Conditions

- General Conditions

- Drawings.

Vii) CPWD general specification

- Relevant IS or any other international code in case IS code is not available.

- Indian Standard Specifications of BIS.

- Accepted good Engineering Practice based on experience of Foreign Countries’’ Codes.

If there are varying or conflicting provisions made in any one document forming part of the contract, the Accepting Authority shall be the deciding authority with regard to the intention of the document and his decision shall be final and binding on the contractor.

Any error in description, quantity or rate in Schedule of Quantities or any Omission there from shall not vitiate the Contract or release the Contractor from the execution of the whole or any part of the works comprised therein according to drawings and specifications or from any of his obligations under the contract.

Signing of Contract: The successful bidder/contractor, on acceptance of his tender by the Accepting Authority, shall, within 15 days from the stipulated date of start of the work, sign the contract consisting of:

  - The notice inviting tender, all the documents including drawings, if any, forming the tender as issued at the time of invitation of tender and acceptance thereof together with any correspondence leading thereto.

  - Standard C.P.W.D. Form as mentioned in Schedule ‘F’ consisting of:

  - Various standard clauses with corrections up to the date stipulated in Schedule ‘F’ along with annexures thereto.

  - C.P.W.D. Safety Code

  - Model Rules for the protection of health, sanitary arrangements for workers employed by CPWD or its contractors.

  - CPWD Contractor’s Labour Regulations.

  - List of Acts and omissions for which fines can be imposed.

No payment for the work done will be made unless contract is signed by the contractor

#### General instructions:

The tender shall be written in English language. All other information such as documents and drawings supplied by the bidder shall also be in English language only. Drawings and design shall be dimensioned according to the metric system of measurements. Tender shall be forwarded under cover or a letter typewritten on the bidder’s letterhead and duly signed by the bidder. Signature must be in long hand, executed in ink by a duly authorized principal/representative of the bidder. No oral, telegraphic or telephonic tenders or subsequent modifications there to shall be entertained. If a tender is submitted on behalf of the firm, then all the partners shall sign the Tender or the Tender may be signed by one of the partners in whose favour all the remaining partners have given such signing partner a valid General Power of Attorney. In case of tender submitted by a company, the person who has been authorized by the Board of Directors of the company through a resolution shall sign the tender. Copy of power of attorney/resolution as the case may be, and the authority letter in favour of the person signing must accompany the tender.

The inspection of CEA (Electrical inspector) local electricity department shall be the responsibility of contractor (if applicable).However on submission of the bills the inspection charge shall be released to the contractor. The work must be executed through a valid electrical licence holder and the supply, installation, testing and commissioning job description must be submitted by the contractor on green card as required to the electrical inspector in advance before charging of the system / line

The Bidder shall quote the percentage above/below the total estimated cost given in Schedule of Quantities. Incomplete offer under the tender shall be liable for rejection. Quantities shown in the Schedule of Quantities are approximate and can increase or decrease. No claim shall be entertained for any item of work due to variation in the quantities.

The tender shall be typed or written in indelible ink and shall be signed by the bidder or a person(s) duly authorized to sign under a valid Power of Attorney/Board Resolution on behalf of the Bidder. The person or persons signing the tender shall initial at all pages of the tender document and an initial shall be affixed at all such places where any insertions, corrections or amendments are made by hand.

The bidder shall sign at the bottom right hand corner of every page of the tender documents in token of acceptance of tender conditions and for the purpose of identification.

The Contractor shall give his address for communication along with telephone/e-mail address and all communication made by NU to the said address by registered post acknowledgement due, speed post, courier or any other mode of service shall be deemed to have been effected provided that communication made by mobile, telephone or e-mail shall be followed by a written communication through courier/post. All correspondence addressed to NU shall be in the name of Registrar, Nalanda University, Rajgir, Bihar.

Submission of a tender by a bidder implies that he has read all the terms and conditions contained in this document and all other contract documents and has acquainted himself of the nature, scope and specifications of the Works to be executed. The Contractor shall also be deemed to have acquainted himself of the local conditions and other factors, which might have a bearing on the execution of the Works. NU shall in no event be held liable for any loss/damage/harm for costs or otherwise caused to the Contractor due to such non-acquaintance.

Against the loss/damage/harm caused, NU shall not be liable for any act or omission of the act of the Contractor in so far as any violation of any of the aforementioned act.

Bidders have to compulsorily submit the entire tender Contract Documents. Every Bidder shall submit all documents issued to him for the purpose of this tender after duly filling the same in all respects. Tender Contracts, which are found to be vague, irrelevant and incomplete, shall be summarily rejected.

Unless otherwise stated, the contract shall be for the whole Work as described in the “Schedule of items of Works” and under the drawings. The Contractor shall be bound to complete the whole Work within such stipulated time as set out in the schedule of items of Works and the drawings, including such additional items as may be necessary, if any, as per drawings and instructions. On the Completion of work, inspect the work and certify The Architect Consultant shall The issuance of certificate of completion, as issued by the Engineer-in-Charge on or by the Competent Authority at Nalanda University shall be mandatory for indicating the successful completion of work and such certificate of completion will be conclusive proof of completion of Work. Such certificate shall be issued only after the inspection of work by Architects Consultants and obtaining certificate from him that work is executed as per approved design and specifications

The work to be carried out under the contract shall, except as otherwise provided in these conditions shall include all labour, materials, tools, plants, equipment, transport and royalties, taxes, labour cess etc., which may be required in preparation of the full and entire execution and completion of the Works. The descriptions given in the Schedule of Quantities shall, unless otherwise stated be held to include wastage of

materials, carrying and all other labour necessary for the full and entire execution and completion of the Work as aforesaid in accordance with the specifications, good practice and recognized principles.

The Contractor shall be deemed to have satisfied itself before submission of the tender as to the correctness and sufficiency of the tender for the Works and of the rates and prices quoted in the Schedule of Quantities, which rates and prices shall, except as otherwise provided, cover all his obligations under the Contract and all matters and things necessary for the proper completion and maintenance of the Works. Under no circumstances shall the Contractor withdraw from the Contract once the tender is accepted. In an event of withdrawal or default by the Contractor, suitable penal actions shall be taken as per the Bid Security Declaration Form.

No cost payable for preparing tender: The bidder shall not be entitled to claim any costs, charges, expenses in connection with preparation and submission and subsequent clarification of its tender in the event of withdrawal of the invitation of tenders by NU.

The Contractor shall comply with the provisions of the Apprentices Act, 1961, Minimum Wages Act, 1948, Workmen’s Compensation Act, 1923, Contract Labour (Regulation and Abolition) Act, 1970, Payment of Wages Act, 1936, Owner’s Liability Act, 1938, Maternity Benefits Act, 1961, and the Industrial Disputes Act, 1947 as may be applicable and the rules and regulations issued there under from time to time. The list of the statutes mentioned above is an indicative list and all enactments applicable for workmen shall be complied with by the bidder. Any failure to comply with such statutes shall amount to breach of the Contract and the Engineer-in-Charge may, at his discretion, terminate the Contract without any prior notice being furnished to the Contractor. The Contractor shall also be liable for any pecuniary liability arising on account of violation of any of the said statutes and shall indemnify and keep the NU indemnified for any loss or damage arising on account of the same.

The provisions in the Tender documents shall prevail over the contents of the above paragraphs if there is any contradiction or variation in the conditions mentioned hereinabove and those contained in the Contract Documents.

#### Additional Definitions

Nominated Sub-Contractor:

Nominated Sub-Contractor shall mean any person or agency appointed by NU for the execution of any particular Work or providing any services under the Contract.

Samples:

Samples are physical samples, which illustrate materials, equipment or workmanship and establish standards by which the Work will be judged.

Shop drawings:

Shop drawings means those drawings or other documents which are specifically prepared by or on behalf of the Contractor to illustrate details of construction for the purpose of fabrication or installation or any other construction related works mentioned in the Tender which required by the Architect Consultants and are submitted to the Engineer In charge for the approval/suggestion/verification from Architect Consultants as the Contractor’s intended method of achieving the end result required by the Contract Drawings and Specifications.

Submittals:

Submittals includes progress schedules, setting out drawings, shop drawings, testing and inspection reports, and other information required by the Contract Documents to be submitted by the Contractor for information or approval by the Engineer in Charge.

Virtual Completion:

Virtual Completion shall mean completion of the Works as per drawings and specifications to the entire satisfaction of Engineer in Charge/ Architect Consultants/ NU whose decision shall be final and binding on the parties in this regard and particularly the act of securing by the Contractor to get the virtual completion certificate from the Engineer in charge and submission of the same to the NU to start the Defect liability period of the contract.

Final Completion:

Final Completion shall mean the completion of the Works and any necessary rectification directed to be carried out during the Defects Liability Period and any extension thereof by the Contractor and securing of such final completion certificate from the Engineer in Charge/ Architect Consultants/ NU, and also obtaining the final handing over of the Works to the NU, and the acceptance of the same.

Jurisdiction of Courts: The Court at Bihar Sharif/High Court at Patna in the state of  Bihar shall have the Jurisdiction to decide any dispute arising out of or in respect of  the contract for this work.

### CLAUSE.5.1 EARNEST MONEY

The earnest money is paid by each bidder to enable the NU to ensure that a bidder does not back out of his tender before its acceptance, or refuse to execute the work after it has been awarded to him.

Deposit of earnest money

Earnest Money can be paid in the form of a Deposit at Call receipt/ Demand Draft or Pay order or Banker`s Cheque of any Scheduled Bank drawn in favour of Nalanda University Rajgir, Bihar along with Bank Guarantee of any Scheduled Bank wherever applicable. It shall be scanned and uploaded to the e-Tendering website within the period of tender submission and original should be deposited in office of NU.

A part of earnest money is acceptable in the form of bank guarantee also. In such case, Rs 20.00 Lakhs of earnest money will have to be deposited in the form of a Receipt of Deposit at Call receipt/Demand Draft or Pay order or Banker`s Cheque of any Scheduled Bank drawn in favour of Nalanda University Rajgir, Bihar and balance in the form of Bank Guarantee of any Scheduled bank, which is to be scanned and uploaded by the intending bidders.

The intending bidder has to fill all the details such as Banker's name, Demand Draft/Pay Order/ Banker's Cheque/Bank Guarantee number, amount and date.

The amount of EMD can be paid by multiple Demand Draft / Pay Order / Banker's Cheque / along with multiple Bank Guarantee of any Scheduled Bank.

The Earnest Money will be returned to the unsuccessful bidders within a week from the date of award of tender. Entry of Demand Draft received as earnest money with the tenders may be kept in the Tender Opening Register, and these need not be deposited in the bank except for the successful bidder. The Earnest Money will be returned to the successful bidder after he furnishes Performance Guarantee and duly enters into the contract within such time as per Schedule-F, the successful bidder shall furnish the required Performance Guarantee and attend the office of the NU for execution of the contract documents.

Forfeiture of earnest money

If any bidder withdraws his tender before the expiry of the validity period, or before the issue of letter of acceptance, whichever is earlier, or makes any modification in the terms and conditions of the tender which are not acceptable to the NU, without prejudice to any other right or remedy, be at liberty to forfeit 50% of the said earnest money as aforesaid. Further the bidder shall not be allowed to participate in the retendering process of the work.

If contractor fails to furnish the prescribed performance guarantee within the prescribed period, the earnest money is absolutely forfeited automatically without any notice.

In case the bidder fails to commence the work specified in Schedule-F or such time period as mentioned in letter of award, after the date on which the Engineer-in-charge issues written orders to commence the work, or from the date of handing over of the site, whichever is later, the NU shall, without prejudice to any other right or remedy, be at liberty to forfeit whole of the earnest money absolutely.

If only a part of the work as shown in the tender is awarded, and the contractor does not commence the work, the amount of the earnest money to be forfeited should be worked out with reference to the estimated cost of the work so awarded.

In case of forfeiture of earnest money as prescribed in above, the bidder shall not be allowed to participate in the retendering process of the work.

#### CLAUSE.5.2 : Sub-Contractors

Where and when the appointment of Sub-Contractors for specialized job (in the opinion of Engineer In Charge) is indicated, such Sub-Contractors shall be appointed only with the prior written approval of the Owner upon recommendation of Engineer- in-Charge/PMC/Architect Consultants on the following conditions:

The Contractor shall enter into written agreements with Sub-Contractors and ensure that the Sub-Contractors perform their Work in accordance with and subject to the terms and conditions of these Contract Documents. A copy of each such Agreement shall be furnished to the Engineer-in-Charge and the Owner.

The Contractor shall remain fully responsible to the Owner for the performance and workmanship and all actions of all sub-Contractors and persons directly or indirectly employed by them.

The Contractor shall supply and permit all Sub-Contractors to avail of site facilities and services to enable them to complete their Work safely and without hindrance or delay and conducive to produce the highest quality of Work required.

The Contractor shall upon receipt of instruction from the Engineer-in-Charge, terminate and remove from site forthwith such Sub-Contractor or their person who’s Work may be considered unsatisfactory.

The Contractor shall make regular and prompt payment to each Sub-Contractor not later than one week after receipt of payment from Owner for their measured Works certified by the Engineer-in-Charge. If the Contractor fails to make payments to Sub-Contractors as aforesaid, the Owner may, without any obligation or prejudice to its rights and with prior intimation to the Contractor may make direct payments to Sub-Contractors for and on behalf and from the account of the Contractor and recover such sums from the account of the Contractor out of the amounts due and payable under the bills raised by the Contractor. Such direct payments to Sub-Contractors shall be on behalf of the Contractor and shall in no way relieve the Contractor of his responsibilities or create a contractual relationship between the Owner and Sub- Contractor.

Any Subcontractor that has been approved by the Owner shall neither be removed nor replaced without the prior written consent of the Owner.

#### CLAUSE. 5.4 : Reinforcement steel and structural steel

Use of Indian standard reinforcement steel and structural steel manufactured by Primary steel manufacturers only will be allowed for the work under this contract. For

Non-standard steel sections, the steel manufactured by the producers other than Primary steel manufacturers shall be allowed after prior approval of Engineer – in – Charge in writing.

#### CLAUSE. 5.5: Electricity

The Owner shall not provide electrical power. Contractor has to arrange with necessary permission from relevant authority if any at his own cost electrical power for construction or for general lighting and other usage

The Contractor shall, at his own cost, provide approved temporary electrical connections, cables, distribution boards and related equipment for construction, as required by the Engineer-in-Charge.

The Contractor shall permit and enable all sub-Contractors to use his power & distribution facilities. The Sub-Contractor shall pay such amount for the use of power & distribution facilities as may be determined between the Contractors.

The Contractor shall, at his own cost and in order to prevent interruption of the Work(s) due to power failures, provide for stand-by diesel generators of sufficient capacity as per the requirements to supply adequate electricity for the Works and for other uses.

Disruptions in power supply, whether due to power failures, load shedding, generator breakdowns, non-availability of Electric Supply or any other reason, shall not be accepted as a valid reason for delays and deficiencies in the Work or for claims for additional payments.

The Contractor shall, at his own cost supply the required electrical power supplies to the Site offices of the Owner as directed by the Engineer-In-Charge.

#### CLAUSE.5.6 : Fossils:

All fossils, coins, articles of value or antiquity and structures and other remains or things of geological or archaeological interest discovered on the Site shall, as between the Owner and the Contractor, be deemed to be the absolute property of the Owner. The Contractor shall take reasonable precautions to prevent his workmen or any other persons from removing or damaging any such article or thing and shall, immediately upon discovery thereof and before removal, acquaint the Engineer of such discovery and carry out the Engineer’s instructions for dealing with the same. If, by reason of

such instructions, the Contractor suffers delay and/or incurs costs then the Engineer shall, after due consultation with the Owner and the Contractor, determine:

#### CLAUSE.5.7 : Insurance

The Contractor shall at his own expense arrange for all insurance policies, including workmen compensation policy and Contractor’s All Risk policy (CAR Policy) in the amount of the Contract effective from the date of commencement of Work(s) and until final completion of the Work(s) also taking into consideration the defects liability period, against all of the following risks.

Injuries and damage of persons, property, machinery, equipment, vehicles, animals or things, within or outside the site, arising out of his operations or of any sub-Contractors, nominated or otherwise, or out of any actions of his employees, agents or representatives, limited to Rs. 20,00,000 (Rupees Twenty Lacs) per accident

Injuries to his or any Sub-Contractor’s employees.

Damage to or loss of the property, equipment, and materials at site, of the Owner, Contractor and all Sub-Contractors, as a result of natural causes such as lightning, storm, flood, rain, fire, earthquake, explosion, landslide, etc.

Damage and injuries to persons, property and materials arising out of riot and civil commotion, theft, sabotage malicious acts, terrorist activities, flood, water logging etc.

The insurance policy or policies to cover risks of every nature shall be in the joint names of the Owner and Contractor, and the original of such policy/ policies shall be lodged with the Engineer-in-Charge. The Contractor shall also lodge the premium receipts with the Engineer-in-Charge, such standing jointly in the names of the Owner and the Contractor.

If the Contractor fails to arrange the requisite insurance or fails to renew the policies, the Owner shall arrange for obtaining the requisite insurance policies and or renew them and recover the cost of all premiums from the Contractor and/or subsequently deduct such amount from the payments due and payable by the Owner to the Contractor from the bills raised by the Contractor in respect of the Work(s) executed.

No certificate of payment shall be issued by the Engineer-in-Charge whether for an interim or of the final bill raised by the Contractor if the Contractor fails to arrange for total insurance cover.

The Contractor shall reinstate in a manner approved by the Engineer-in-Charge all damage of every sort caused entirely at his cost so as to deliver up to the Owner the whole of the Work(s) complete and perfect in all respects, and so certified by the Engineer-in-Charge and also make good or otherwise satisfy all claims for damage to property of third parties.

The Contractor shall be responsible for anything within his control and for all risks and consequences, which are not included in the purview of the insurance policies.

The insurance cover shall stand extended until final completion of the Works and the contract and shall also cover the defects liability period.

The Contractor undertakes not to cancel any insurance policy nor reduce its scope without the written consent of the Engineer-in-Charge.

The Contractor undertakes to file necessary insurance claims jointly with the Owner and also to join the Owner in filing any claim the Owner chooses to.

Payments against all insurance claims shall be received in the name of the Owner and commensurate adjustments shall be made in accounts with the Contractor.

The aforesaid insurance policy/policies shall not be cancelled till the Engineer-in- Charge has agreed to their cancellation.

The Contractor shall prove to the Engineer-in-Charge from time to time that it has taken out all insurance policies referred to above and has paid the necessary premium for keeping the policies alive till the end of the defects liability period.

The Contractor shall ensure that similar insurance policies are taken out by his Sub- Contractor (if any) and shall be responsible for any claims of losses to the Owner resulting from their failure to obtain adequate insurance protections in connection thereof. The Contractor shall also obtain the originals of the policies and the premium

receipts from the Sub-Contractor and shall lodge the same with the Engineer-in- Charge.

#### CLAUSE.5.8: Good For Construction Drawings, Details and Clearance to Construct.

After the acceptance of the Letter of Intent, issued by the Owner – Nalanda University, the contractor will be issued “Good for construction” drawings for works. The contractor will arrange to mobilize his resources in very positive and judicious manner to start work on maximum possible fronts. This Execution Plan will be discussed with Project Management Consultants, Architect Consultants and NU in detail and will be finalized subject to changes, if any, and will be agreed to by all the stake holders. If the contractor fails to show reasonable progress for the works for which the drawings and details are already available, Then the Owner Nalanda University, through the Engineer In Charge, will take suitable punitive action like withholding the payments due to contractor, which is equivalent to the amount to be withheld for completion of nearest mile stone as per this Notice Inviting Tender. The withheld amount will be released as soon as the corrective action is taken by the Contractor, for making up the time lost due to delay.

The contractor on receipt of detailed drawings for construction, will be required to prepare the mock ups for various items of work, as soon as possible, and get the same approved from the Owner- Nalanda University and the Architect Consultants. This shall be done at the actual location of each type of sample required. The Contractor is expected to execute entire work of similar nature, exactly similar to the mock up work in terms of materials used and workmanship desired. The work not matching to the above criteria will be removed and redone to the entire satisfaction of the Owner and the Architect Consultants.

#### CLAUSE.5.9 : Protection of Persons, Works and Property Accident or Injury to Workmen

The Owner shall not be liable for or in respect of any damages or compensation payable to any workman or other person in the employment of the Contractor or any Subcontractor, other than death or injury resulting from any act or default of the Owner, his agents or servants.

#### Accident Prevention:

General:

In performing this contract, the Contractor shall provide for protecting the lives and health of employees and other persons preventing damage to or theft or loss of property, materials, supplies, and equipment; and avoiding Work interruptions. For these purposes, the Contractor shall- Provide appropriate safety barricades, signs, and signal lights.

Comply with the standards issued by any statutory bodies having jurisdiction over occupational health and safety and Ensure that any additional measures as required by the Engineer-in-Charge for this purpose.

Records.

The Contractor shall maintain an accurate record of exposure data on all accidents taken place incidental to performance of Work(s) under this contract resulting in death, traumatic injury, occupational disease, or damage to or theft / loss of property, materials, supplies, or equipment. The Contractor shall report this data in the manner prescribed by the Engineer-in-Charge.

Sub-Contractors:

The Sub-Contractors shall be bound to comply with the clause in the same manner as complied with by the Contractor. In the event of non-compliance by the Sub-Contractor of such clause, the Contractor shall be responsible for compliance of the Sub-Contractor.

Written program:

Before commencing the Work, the Contractor shall submit to the Engineer-in- Charge a written proposal for implementing this clause,

Hazardous Material Identification.

Notification:

The Contractor shall notify the Engineer-in-Charge in writing of all hazardous material 5 days before delivery of the material. This obligation applies to all materials delivered under this contract, which will involve exposure to hazardous materials or items containing these materials.

Responsibility of Contractor:

Neither the requirement of this clause nor any act or failure to act by the Owner shall relieve the Contractor of any responsibility or liability for the safety of Owner, Contractor, or Sub-Contractor’s personnel or property.

Compliance with laws:

The Contractor shall comply with applicable laws, including the Public Liability Insurance Act 1991, Fatal Accident Act 1855, codes, ordinances, and regulations (Including the obtaining of licenses and permits) in connection with hazardous materials. Contractor shall pay fees and other expenses for obtaining such permission or licenses.

Sub-Contractors:

The Contractor shall insert these above clauses, relating to hazardous material, with appropriate changes on entering into contracts or agreements with Sub-Contractors and the sub-contractors shall be bound and be liable to comply with the same, and in the event of non-compliance of the same, the Contractor shall be held liable for damages or otherwise on the acts of the Sub-Contractor in this regard.

#### Protection of Property

Vegetation, structures and equipment:

The Contractor shall preserve and protect all structures, equipment, and vegetation on or adjacent to the Work site, and which do not unreasonably interfere with the Work required under this contract and shall not be removed by the Contractor. The Contractor shall only remove trees when specifically authorized to do so, and shall avoid damaging vegetation that will remain in place.

Utilities and improvements:

The Contractor shall protect from damage and have all existing improvements and utilities at or near the Work site and on adjacent property of a third party, the locations of which are made known to or should be known by the Contractor. The Contractor shall be liable to repair any damage caused to those facilities, including those that are the property of a third party, resulting from failure to comply with the requirements of this contract or failure to exercise reasonable care in performing the Work. If the Contractor fails or refuses to repair the damage promptly, the Engineer- in-Charge may have the necessary Work performed and charge the cost to the Contractor or reduce such amounts from the bills of the Contractor due and payable by the Owner.

- Contractor shall be required to work within specified areas and they shall be allowed to use only those areas around the works under their scope, for storage of their materials, construction of site offices, erection of batching plant etc. at predetermined locations as shown on the plans. The area so demarcated, shall be barricaded in such a way that the construction activities or the moving vehicular traffic involved in carriage of construction materials/ construction waste etc. do not create interference with any other areas within the premises or activities of the Institute or the activities of other contractor/s working within the premises.

- The contractor shall provide suitable barricading approximately 2.0 m high, with suitably painted with three rows of G.I. Sheets about 2’-6” to 3’- 0” wide (75 cms to 90 cms.) nailed or bolted with wooden poles spaced 2 to 3 meter apart and each pole 1.6 m to 2 m long 8 cm. to 10 cm. dia. The poles will be

embedded in mobile iron pedestal rings suitably framed for giving stable support as per direction of the Engineer-in-charge. All management (including watch and ward) of barricades shall be the full responsibility of the contractor. The barricades shall be removed only after completion of the work or part of the work. The contractor’s rate shall include all above items of work and nothing extra shall be paid to the contractor over and above his quoted rates.

- The positioning of barricading will be reviewed from time to time and necessary shifting barricading as directed by Engineer - in – charges shall be done forthwith by the contractor. Payment of initial erection of barricading and the shifting thereof at a later date as per requirement shall be made under relevant tender items

- Contractor will make his own arrangements for making temporary roads and approaches to various locations of work under their scope and up to disposal sites marked on the drawing. These internal approaches and temporary roads shall be made in such a way that they do not affect construction activities of permanent roads within the premises at any time.

#### Watchmen and Security

The Contractor shall provide sufficient personnel and materials to provide adequate protection to the property and personnel at the site, in transit and stored goods/materials including but not limited to measures specifically required by and under the Contract Documents and any security requirements under this contract.

#### Corrective Action

Authority to Stop Work:

The Engineer-in-Charge shall notify the Contractor of any non-compliance with the safety and property protection measures as required under this contract of which the Engineer-in-Charge becomes aware and of the corrective action required. This notice, when delivered to the Contractor or the Contractor’s authorized representative at the site of the Work shall be deemed sufficient notice of non- compliance and

corrective action required. After receiving the notice, the Contractor shall immediately take necessary steps to correct the action. If the Contractor fails or refuses to take corrective action promptly, the Engineer-in-Charge shall at his discretion after due consultation with the Owner may issue an order stopping all or part of the Work(s) until satisfactory corrective action has been taken. The Contractor shall not base any claim or request for equitable adjustment for additional time or money on any stop Work order issued under these circumstances.

Rectification:

The Contractor shall be solely responsible to make good at his cost any damage to the Works, property of the Owner and/or any adjacent property, to the satisfaction of the Engineer-in-Charge. In case the contractor fails to do so within a reasonable time the Engineer-in-Charge shall get the same executed at the risk & cost of the contractor & deduct the same from his due payments.

#### CLAUSE.5.10 :Site Security:

Contractor shall be deemed to be in possession of the Works site and shall be responsible for its total security, and shall ensure that all materials, sheds, equipment, plant, tools, etc.; whether his own or belonging to any Sub-Contractor, are well protected.

The Contractor shall at his own cost install and maintain sufficient security fences and gates and employ full time round-the-clock security personnel to prevent the Works site from and against the intrusion of the public or any other unauthorized persons or vehicles.

Total security of the site, property, and materials shall be the sole responsibility of the Contractor. The presence of his consultants representatives, or the NU ’s security personnel shall in no way relieve or absolve the Contractor of his responsibilities in ensuring the security and protection of the site and everything stored or lying thereon

#### CLAUSE.5.11 :Warranty/Guarantee

The Contractor shall be responsible for the proper performance of the Work(s), including installations and systems, as specified under the Contract Documents.

Subject to Clause the Contractor shall, at his own cost and in the shortest possible time, repair and remove any defect or deficiency in the Works, which may appear prior to or during the defect liability period, to the satisfaction of the Engineer-in-Charge.

A guarantee will be given by the Contractor for the complete installation of the Works including its functioning, replacement of parts etc. as specified under the Contract Documents.

The defect liability period will be 24 months for the project from taking over of the system by the University. However, beyond the Defect Liability Period(i.e. after completion of 24 months), if the manufacturer provides the warranty of the appliances for more than two years, in that case, the warranty/guaranty of the system will be valid as per the OEM practice.

All the Guarantees referred above shall commence from the date of Completion Certificate.

### GUARANTEE FOR WATER PROOFING TREATMENT

The contractor shall be fully responsible for and shall guarantee proper performance of the entire water proofing system for a period of 7(Seven) years from the final completion of works. In addition, specific 7 years written guarantee (to be furnished in a non-judicial stamp paper of value not less than Rs.100/-) in approved Performa shall be submitted for the performance of the system, before final payment and shall not in any way limit any other rights the Owner may have under the contract. Guarantee for waterproofing and anti-termite treatment shall comprise of all the items described in particular specification.

All waterproofing work and anti-termite treatment shall be carried out through approved specialist agency as per method of working approved by the Engineer-in-

charge. However the Contractors shall be solely responsible for waterproofing and anti-termite treatment until the expiry of the above guarantee period.

Seven years guarantee in prescribed Performa attached shall be given by the contractor for the waterproofing treatment. In addition10 %( ten percent) of the cost of those items of waterproofing under this subhead shall be retained as guarantee to watch the performance of the work executed. However, half of this amount (withheld) would be released after five years from the date of completion of the work, if the performance of the waterproofing work is satisfactory. They’re meaning withheld amount shall be released after completion of ten years from the date of completion of work, if the performance of the waterproofing work is satisfactory. If any defect is noticed during the guarantee period, it should be rectified by the contractor within seven days of issuing notice by the Engineer-in-Charge and, if not attended to, the same shall be got done through the agency at the risk and cost of the contractor and recovery shall be effected from the amount retained towards guarantee. In any case, the contractor and the specialist agency, during the guarantee period, shall inspect and examine the treatment once in every year and make good any defect observed and confirm the same in writing. The security deposit can be released in full, if bank guarantee of equivalent amount, valid for the duration of guarantee period, is produced and deposited with the Owner.

#### CLAUSE.5.12 :Contractor’s Responsibilities and Work Control

The Contractor shall have complete control of the Works and shall effectively and diligently control, direct and supervise his employees, supervisors, subordinates and Sub-Contractor(s) so as to ensure timely completion of the Works in order and in conformity with the Contract Documents. It shall be the sole responsibility of the Contractor for construction means, methods, techniques, sequences and procedures, and for coordinating the various parts of the Work, whether carried out by the Contractor or any Sub-Contractor.

The Contractor shall provide adequate, qualified and experienced personnel for the proper superintendence and execution of the Works until completion. The category and strength of such personnel shall be determined by the Engineer-in-Charge, and such approved site organization strength shall be maintained by the Contractor at all times until completion of Work(s), and also during defects liability period and as may be decided by the Engineer-in-Charge.

The Contractor shall be responsible for the design, erection, operation, maintenance and removal of temporary structures and other facilities at his own cost during completion of the Works. Any approval sought, given or implied, regarding sufficiency, stability and safety of temporary staging and facilities, shall in any way not relieve the Contractor of his responsibility.

The Contractor shall study all Contract Documents and promptly report to the Engineer-in-Charge any non-conformity, discrepancy, inconsistency or omission he may discover in the same. In the event of such discovery, the Contractor shall not proceed with the affected Works until he has received due corrections and clearances from the Engineer-in-Charge.

The Contractor shall be deemed to have thoroughly studied and satisfied himself regarding Contract Documents and particularly all drawings before commencement of the Work(s). Should any discrepancy or error be discovered during execution of parts of the Work(s) necessitating demolition, repairs or reconstruction, all such remedial measures shall be carried out only with the approval of the Engineer-in-Charge and entirely at the cost of the Contractor. In such an event the Contractor shall neither claim any extra payment nor any extension of time for any delay caused by virtue of such demolition, repairs and reconstruction.

Any instructions given to the Contractor’s supervisory staff by the Engineer-in- Charge shall be deemed to have been given to the Contractor. Instructions that involve any variations in design or specifications and which may have a bearing on time and cost shall be through a written Change Order by the Engineer-in-Charge and at rates agreed in writing prior to implementation

The Contractor shall at his own cost, obtain any permits or authorizations necessary for the execution of the Work and obtaining any permits or approvals for the works executed by him, from all concerned statutory and Owner Authorities/Authority’s, including but not limited to Municipal bodies, Electrical Authority, Fire Service Authorities etc.,

The Contractor shall not be entitled to claim additional sums on account of having to work overtime in order to complete an operation that cannot be interrupted, for working in extended shifts / night shifts /holidays.

In the event the Contractor chooses to work overtime, in extended night shifts as and by way of overtime either by working extended/night shifts or morning or holidays in order to complete the Work(s) within the specified period or on holidays, he shall do so by obtaining prior written approval from the Engineer-in-Charge at least twenty-four hours in advance. The Contractor moreover shall ensure that in any of the above circumstances he maintains the full-agreed strength of his supervisory staff.

The Contractor shall take all necessary precautions to protect the site and Works, materials, plant and equipment, whether his own or belonging to the Owner or any Sub-Contractors, against hazards of fire, rains, floods, landslides, underground water, accidents, etc.

The Contractor shall not be permitted to replace nor remove his Project Manager/Project Engineer / Site Engineer etc. from the site without the prior written approval of the Engineer-in-Charge.

Submittals

“Shop drawings” means those drawings or other documents, which are specifically prepared by or on behalf of the Contractor to illustrate details of construction for the purpose of fabrication or installation and are submitted to the Owner to indicate the Contractor’s intended method of achieving the end result required by the Contract Drawings and Specifications.

“Project data” includes standard drawings, diagrams, layouts, schematics, descriptive literature, illustrations, schedules, performance and test data, and similar

materials furnished by the Contractor to explain in detail specific portions of the Work required by the. Contract.

“Samples” are physical examples, which illustrate materials, equipment or workmanship and establish standards by which the Work will be judged.

“Other submittals” includes progress schedules, setting drawings, testing and inspection reports, and other information required by the Contract Documents to be submitted by the Contractor for information or approval by the Owner.

Schedules of Submittals

Promptly after contract award the Contractor shall submit to the Engineer-in-Charge the submittal schedule showing when product data sheets, samples and other submittals required by and under the Tender Documents would be submitted for the approval of the Engineer-in-Charge.

Review and approval of submittals by Contractor

The Contractor shall co-ordinate and compiles all submittals required by and under the Contract Documents, and thoroughly check them for accuracy, completeness, and compliance in accordance with contract requirements and shall indicate his approval thereon in the form required by the Contract Documents as evidence of such co- ordination and checking. Submittals to the Engineer-in-Charge without the approval of the Contractor shall be returned by the Engineer-in-Charge for resubmission. Submission of shop drawings, product data or samples shall constitute a representation that the Contractor has agreed to, asserted and guaranteed that the assemblies, products or materials indicated therein will be available in a timely manner and in the quantities required for the project as set out under the Contract Documents.

Submission

All submittals shall be in English language, and any system of dimensions (i.e.; English or metric) shown shall be consistent with that used in the Contract Documents. The Contractor shall submit all Submittals in the form and number

required by the Contract Documents within required time limits and sufficiently in advance of construction requirements to permit adequate review by the Owner for correction, approval and resubmission if required. No extension of time shall be allowed on account of any delay by the Owner in approving such submittals, if the Contractor has failed to act promptly and responsively in making his submissions. Each submittal shall be identified as required by the Contract Documents.

Action on Submittals

The Engineer-in-Charge will indicate an approval or disapproval of the Submittals for and on behalf of the Owner requiring approval by the Owner and if not approved as submitted shall indicate the Owner’s reasons thereof. Approval by the Engineer-in- Charge shall not relieve the Contractor from responsibility for any errors or omissions in his submittals, nor from responsibility for complying with the requirements of this contract, except with respect to variations described by the Contractor and approved in accordance with "Variations in Submittals" Clause. The approval of the Engineer-in- Charge on the submittals will be for general compliance with the intent of the Contract Documents and with the information given therein, and shall not be construed

As permitting any departure from the contract requirements

- As relieving the Contractor of responsibilities for any error including details, dimensions, materials, etc. and

- As approving departures from details appearing on Contract ‘Drawings and Specifications.

Where approval of Submittals is required, the Contractor shall perform the Work in accordance with such approved Submittals. Any Work performed by the Contractor prior to such approval by the Engineer-in-Charge shall be at the sole risk and liability of the Contractor.

Variation in Submittals

If Submittals contain any variations from the contract requirements, other than those requested on previous submittals, the Contractor shall specifically describe such

variations in writing and the reasons thereof to the Engineer-in-Charge. If the approval of any such variation affects the Contract Price or the Completion time of the Contract, the Engineer-in-Charge shall issue an appropriate Contract modification. Otherwise, the variation may be approved by the Engineer-in-Charge, only by specific reference thereto in writing. The Contractor shall not be entitled to rely on general approval of a submittal as an approval of variations of requirements of the Contractor. If the Contractor fails to describe such variations and shall not be relieved from the responsibilities of executing the Work in accordance with the contract, notwithstanding a general approval of such submittals. Nothing contained herein shall relieve the Contractor of the responsibility of notifying the Engineer-in-Charge of any part of the Contract Drawings or Specifications, which the Contractor knows or reasonably should have known which could result in defects under construction.

Use of submittals

The Owner may duplicate, use, and disclose in any manner and for any purpose shop drawings, product data and other submittals delivered under this contract.

Placement of Orders

The Contractor shall place orders for items requiring a sample or product data submittal promptly after receiving the written approval of the submittal by the Engineer-in-Charge. No such materials or products shall be ordered or used in the Work until such written approval by the Engineer-in-Charge has been given. In the event such materials or products pre ordered or used in the Works without the written approval of the Engineer-in-Charge, the same shall be at the risks, consequences, liability and costs of the Contractor.

Use and testing of samples

Use:

Approved samples not destroyed in testing will be sent to Engineer-in-Charge. Those samples, which are in good condition, will be marked for identification and may be used in the Works. Materials and equipment incorporated in the Work shall match the approved samples within any specified tolerances. Other samples not destroyed in

testing nor approved will be returned to the Contractor at the expense of the Contractor if so requested at the time of submission.

Failure of samples to pass specified tests:

Failure of any material to pass the specified tests will be sufficient cause for refusal to consider, under this contract, any further samples of the same brand or make of that material or equipment which previously has proved unsatisfactory in service.

Taking and testing of samples:

Samples of various materials or equipment delivered on the site or in place may be taken by the Engineer-in-Charge for additional testing by the Owner outside of those found not to have met contract requirements, unless the Engineer-in-Charge determines it to be in the Owner’s interest to accept the non-conforming materials or equipment with an appropriate adjustment of the Contract Price/Value as determined by the Engineer-in-Charge.

Cost of additional testing:

Unless otherwise specified, when additional tests are made, only one test of each set of sample proposed for use will be made at the expense of the Contractor. Samples, which do not meet contract requirements, will be rejected. Further testing of additional samples, if required, will be made at the expense and costs of the Contractor.

#### CLAUSE.5.14 : Quoted Rates

The rates quoted by the Contractor are deemed to be inclusive of site clearance, setting out work, profile, setting lay out on ground, establishment of reference bench mark(s), installing various signage, taking spot levels, survey with total station, construction of all safety and protection devices, compulsory use of helmet and safety shoes, and other appropriate safety gadgets by workers, imparting continuous training for all the workers, barriers, preparatory works, construction of clean, hygienic and well ventilated workers housings in sufficient numbers as per drawing supplied by Engineer in charge, working during monsoon or odd season, working beyond normal hours, working at all depths, height, lead, lift, levels and location etc. and any other unforeseen but essential incidental works required to complete this work. Nothing extra shall be payable on this account and no extension of time for completion of work shall be granted on these accounts.

The rates quoted by the bidder, shall be firm and inclusive of all taxes and levies as applicable (including works contract tax and GST as per Clause 38). Labour cess applicable as per relevant Central/State laws will be deducted from every bill of the contractor and deposited to respective Government department. TDS for Income- tax and Works Contract tax as applicable will be deducted from contractors’ bills and deposited to respective Government department. The ESI and EPF contributions on the part of employer in respect of this contract shall be paid by the contractor. These contributions on the part of the employer paid by the contractor shall be reimbursed by the Engineer-in-charge to the contractor on actual basis. Effect of any tax due to new legislation shall be adjusted accordingly as per actual on production of documentary proofs of payment of such tax to concerned Government departments.

No foreign exchange shall be made available by the Owner for importing (purchase) of equipment, plants, machinery, materials of any kind or any other items required to be carried out during execution of the work. No delay and no claim of any kind shall be entertained from the Contractor, on account of variation in the foreign exchange rate.

All ancillary and incidental facilities required for execution of work like labour camp, stores, fabrication yard, offices for Contractor, watch and ward, temporary ramp required to be made for working at the basement level, temporary structure for plants and machineries, water storage tanks, installation and consumption charges of temporary electricity, telephone, water etc. required for execution of the work, liaison and pursuing for obtaining various No Objection Certificates, completion certificates from local bodies etc., protection works, testing facilities / laboratory at site of work, facilities for all field tests and for taking samples etc. during execution or any other activity which is necessary (for execution of work and as directed by Engineer-in- Charge), shall be deemed to be included in rates quoted by the Contractor, for various items in the schedule of quantities. Nothing extra shall be payable on these accounts. Before start of the work, the Contractor shall submit to the Engineer-in-Charge, a site / construction yard layout, specifying areas for construction, site office, positioning of machinery, material yard, cement and other storage, steel fabrication yard, site laboratory, water tank, etc.

For completing the work in time, the Contractor might be required to work in two or more shifts (including night shifts). No claim whatsoever shall be entertained on this account, not with-standing the fact that the Contractor may have to pay extra amounts for any reason, to the laborers and other staff engaged directly or indirectly on the work according to the provisions of the labour and other statutory bodies regulations and the agreement entered upon by the Contractor with them.

All material shall only be brought at site as per program finalized with the Engineer-in-Charge. Any pre-delivery of the material not required for immediate consumption shall not be accepted and thus not paid for.

Rate shall including of Liaison work required, if any, in this regard with the local bodies. Nothing extra shall be payable on this account. Statutory charges, fees etc. required to be paid to the local bodies in this connection shall only be payable by the Owner or shall be reimbursable to the contractor on production of proof of actual payment made by him.

It is clarified that the contractor shall be responsible for obtaining all No- objection certificate (NOCs) and relevant licenses for services like lift installation, electrical installation, fire installation and like. Nothing extra shall be payable on this account.

#### CLAUSE.5.15 :Inspection and rectification of Works

Access:

The Owner and their authorized agents and representatives shall at all times have access to the site and other locations where parts of the Work are under preparation.

Contractor tests:

The Contractor shall notify the Engineer-in-Charge well in advance, of tests and inspections to be carried out, and shall obtain his written approval wherever so stipulated before proceeding with the Works.

Inspections:

The Contractor shall maintain an adequate inspection system and perform such inspections from time to time as will ensure that the Work called for by this contract conforms to contract requirements and does not result in any deviation. The Contractor shall maintain complete inspection records and make them available to the Owner. All Work shall be conducted under the general direction of the Contractor and is subject to Owner’s inspection and test at all places and at all reasonable times before final completion and acceptance with a view to ensure strict compliance with the terms and conditions of the contract.

Owner’s inspections and tests:

Inspections and tests conducted by or on behalf of the Owner are for the sole benefit of the Owner and do not: -

Relieve the Contractor of responsibility for providing adequate quality control measures,

  - Relieve the Contractor of responsibility for damage to or loss of the material before final completion and acceptance of the Work;

  - Constitute or imply acceptance. Or.

  - Affect the continuing rights of the Owner after acceptance of completed Work.

Owner inspectors:

The presence or absence of an Owner’s inspector does not relieve the Contractor from any of the obligations under the contract nor is the inspector authorized to change any term or condition of the contract.

Performance of inspections and tests:

The Contractor shall promptly furnish, without an additional charge all facilities, labour, and material reasonably needed for performing such safe and convenient inspections and tests as may be required by the Engineer-in-Charge as per the terms of the Contract. The Owner may charge to the Contractor any additional cost of inspection or testing when work is not ready at the time specified by the Contractor for inspection or testing, when prior rejection makes re inspection or retesting necessary. The Owner shall perform all its inspections and tests in a manner that will not unnecessarily delay the work. Special, full size, and performance tests shall be performed as described in the Contract Documents.

The Contractor shall be solely responsible for the protection of all finished surfaces and Works so as to avoid any repairs and shall deliver to the Owner upon final completion the Works free of any blemish, defect or damage.

  - In addition to the provisions of relevant clauses of the contract, the work shall also be open to inspection by the other agency as and when required by NU in addition of the Engineer-in-charge and his authorized representative. The contractor shall at times during the usual working hours and at all times at which reasonable notices of the intention of the Engineer-in-charge to visit the works shall have been given to the contractor, either himself be present to receive the orders and instructions or have a responsible representative duly accredited in writing, to be present for that purpose.

  - Inspection of the work by Architect Consultants appointed by the NU.

    - The Architect Consultants appointed by NU shall be inspecting the works frequently to ensure that the works are in general being executed according to the design, drawings and specifications laid down in the contract.

    - The Architect Consultants appointed by NU shall certify on completion of particular building that it has been constructed according to the approved drawings design and specifications.

#### CLAUSE.5.16 :Rejected Work

The Architect Consultants and/or Engineer-in-Charge/or Owner shall be authorized to reject any Work, which in their/his opinion is not in conformity to the specifications set out in the Contract Documents. The decision of the Architect Consultants /Engineer-in- Charge in this regard shall be final and binding on the Contractor.

Defective Work whether caused due to poor workmanship, use of sub-standard materials, or on account of damage or for any other reason whatsoever, whether caused by the Contractor and/or the Sub-Contractor may be rejected by the Architect Consultants/Engineer-in-Charge and shall be demolished by the Contractor and removed promptly from the site and replaced or re-executed expeditiously by the Contractor at his own cost. The Owner, Architect Consultants and the Engineer-in- Charge shall in no event be responsible to bear any costs/liability arising on account of such defective workmanship.

If in the opinion of the Engineer-in-Charge/ Architect Consultants, it is not expedient nor feasible to correct the defective Work, the Owner shall be entitled to deduct from monies due to the Contractor, the difference in value between the executed Work and that required under the Contract, such amount of which shall be determined by the Engineer-in-Charge in due consultation with the Owner.

#### Clause. 5.17 :On Account Payment of Interim Bills

The Owner shall release 75% of the value of the interim bill submitted by the contractor immediately on adhoc basis on preliminary scrutiny of the bill by Engineer- in-charge. Such adhoc payment shall be regarded as the advance payments against

the submitted bill. The balance amount shall be released within 15 (Fifteen) working days after the verification in detail by the Engineer-in-charge and getting certification for the quality of the Work(s) from the Architect Consultants and Project Management Consultants (PMC) for which the Contractor shall provide assistance to the Engineer-in-charge in accordance with their direction. All payment will be subject to statutory or permissible deductions to be made by the Owner prior to the release of the amount to the Contractor.

Each bill shall be accompanied by the following documents.

Measurements and quantities of items of Work done since last bill. Physical Progress Report along with relevant Photographs.

Copies of quality control tests in specified format covering the Work done since last bill.

Copies of Material Bills for equipment’s and items procured at site after initial inspecting and approval from Engineer In Charge.

Copies of instructions recorded in the site instruction book containing the instructions and compliance made thereof, covering the Work done since last bill.

#### Clause. 5.18 :Payment of Final Bill

On the intimation to the Engineer-in-charge by the contractor for the completion of the work, the Engineer-in-charge shall arrange for the inspection of works by his representative, PMC and the Architect Consultants. Upon the inspection of works Engineer-in-charge will hand over a list showing the defects in works, if there are any, to the contractor. The final Bill shall not be paid till all such defects are not rectified to the entire satisfaction of the Engineer-in-charge.

The final bill shall be accompanied by:

All-technical documents on the basis of which the Work was carried out.

      - As-built drawings. Three sets of construction and installation drawings for all Works, and Site development, showing therein modifications, corrections and additions signed and confirmed by the Engineer-in-Charge to be “as built” drawings. The entire documentation shall be submitted in Compact Discs (CD), using latest version of AUTOCAD software.

      - Completion certificates for embedded and covered-up Works issued by the Engineer-in-Charge.

      - Certificates for tests carried out for various items of Work.

      - Manufacture’s operating and maintenance manuals as well as guarantee/warrantee papers, commissioning and handing over reports for whatever equipment/Materials installed.

      - Security Deposit of 2.5% (Two Decimal Five per cent) of the Contract Value in the form of bank guarantee valid for a Defect liability period from the date of certificate.

#### CLAUSE.5.19 :Limit of price Adjustment

Provided that, in determining all such price adjustment in accordance with the aforesaid sub-clauses:

No account will be taken of any amount by which any cost incurred by the contractor has been increased by default or negligence of the contractor.

If the contractor fails to complete the work within time for completion, increase or decrease of cost of specified materials shall be made using either the indices or prices relating to prescribed time for completion, or the current indices or prices, relating to prescribed time for completion, or the current indices or prices, whichever is more favourable to the Owner, provided that if an extension of time is granted, the above position shall apply to the adjustment made after expiry of such extension of time.

On completion of the works and before final payment the contractor shall give a certificate that he has made full and complete disclosure to the Engineer-in-Charge of every increase or decrease in price obtained by him on materials affected by this clause.

#### CLAUSE.5.20 :Exemption from price Adjustment

The following items shall not be included in the price adjustment calculation: Liquidated damages:

- Retention withheld and released:

- Advance payments in the form of loans and their repayments:

- The value of any additional or varied work valued at current prices

#### CLAUSE.5.21 :Specifications

In the case of any class of work for which there is no such specifications as referred to in detailed specification, such work shall be carried out in accordance with the Bureau of Indian Standards Specifications. In case there are no such specifications in Bureau of Indian Standards, the work shall be carried out as per manufacturers’ specifications, if not available then as per C.P.W.D. /District/ state P.W.D. / Specifications. In case there are no such specifications as required above, the work shall be carried out in all respects in accordance with the instructions and requirements of the Engineer-in-Charge. Specifications for a type of work are not covered by the detailed specifications/State P.W.D. specifications/I.S.I specifications or particular specifications, the same shall be decided by the engineer-in-charge and shall be binding on the contractor.

The engineer-in-charge shall have the power to insist upon the contractor to purchase and use such materials of particular approved make which may in this opinion be necessary for proper and reasonable compliance with the specifications and execution of work.

In the event of any variation/discrepancy in the drawings, specification and tender documents, the decision of the Engineer In charge shall be final, binding and conclusive on the contractor.

As required by the Engineer-in-charge, the contractor shall provide all facilities at site or at manufactures work or an approved laboratory for testing of materials and/or workmanship, all the expenditure in respect of this shall be borne by the contractor unless specified otherwise in the contract. The contractor shall when required to do so by the engineer-in-charge submit that the materials have been tested in accordance with requirements of the specifications.

Neither the confirmation by the engineer-in-charge not to test the materials nor the production of manufacturer(s) certificate etc. as aforesaid shall affect the right of the engineer-in-charge to reject, after delivery; the materials found not in accordance with the specifications.

#### CLAUSE.5. 22 :Setting Out

The Contractor shall carry out survey of the work area and it’s surrounding with spot levels of the existing ground at every 2.0 mt interval before starting of any work on ground. Contractor has to submit a soft copy of the drawing in CAD format showing the spot levels and contour at every 0.5 mt interval with three set of Hard copy in the same scale of the issued drawing for setting out at his own cost. Contractor has to carry out setting out the layout of building in consultation with the Engineer-in-Charge & proceed further for the approval of Architect Consultants. Any discrepancy between the architectural drawings and actual layout at site shall be brought to the notice of the Engineer-in-charge. It shall be responsibility of the Contractor to ensure correct setting out of alignment. Total station survey instruments etc. shall be used for layout, fixing boundaries, and centerlines, etc. Nothing extra shall be payable on this account.

The Contractor shall establish, maintain and assume responsibility for grades, lines, levels and benchmarks. He shall report any errors or inconsistencies regarding grades, lines, levels, dimensions etc. to the Engineer-in-Charge before commencing work. Commencement of work shall be regarded as the Contractor’s acceptance of such grades, lines, levels, and dimensions and no claim shall be entertained at a later date for any errors found.

If at any time, any error appears due to grades, lines, levels and benchmarks during the progress of the work, the Contractor shall, at his own expense rectify such error, if so required, to the satisfaction of the Engineer-in-Charge. Nothing extra shall be payable on this account.

Though the site levels are indicated in the drawings the Contractor shall ascertain and confirm the site levels with respect to benchmark from the concerned authorities. If required, contractor will have to make his own temporary/permanent benchmark to carry  out  further  work  at  site.  The  Contractor  shall  protect  and  maintain

temporary/permanent benchmarks at the site of work throughout the execution of work. The Engineer-in-Charge or his authorized representatives shall check these benchmarks. The work at different stages shall be checked with reference to bench marks maintained for the said purpose. Nothing extra shall be payable on this account.

The approval by the Engineer-in-Charge, of the setting out by the Contractor, shall not relieve the Contractor of any of his responsibilities and obligation to rectify the errors/ defects, if any, which may be found at any stage during the progress of the work or after the completion of the work.

The Contractor shall be entirely and exclusively responsible for the horizontal, vertical and other alignments, the level and correctness of every part of the work and shall rectify effectively any errors or imperfections therein. Such rectifications shall be carried out by the Contractor at his own cost to the entire satisfaction of the Engineer- in-Charge.

The rates quoted by the Contractor are deemed to be inclusive of site clearance, setting out work (including marking of reference points, center lines of buildings), construction and maintenance of reference bench mark(s), taking spot levels, construction of all safety and protection devices, barriers, barricading, signage, labour safety, labour welfare and labour training measures, preparatory works, working during monsoon, working at all depths, height and location etc. and any other incidental works like making temporary approaches to the place of work required to complete this work. Nothing extra shall be payable on this account.

#### CLAUSE.5.23 : Contractor shall submit fabrication/ shop drawings/ Technical data sheet for obtaining approval

The contractor shall submit technical data sheets as per the design parameters given by the Architect Consultants, in triplicate for obtaining preliminary approval of the Engineer-in-Charge for all design drawing. One copy of this document duly corrected and signed wherever necessary by Architect Consultants will be returned to the contractor for preparing and resubmitting document after incorporating the said

corrections again in triplicate for final approval, if required. Along with the completion and approval of each TDS the contractor shall also submit the sample list for checking and approval to the Engineer-in-charge. NO TDS shall be approved finally without Sample list. Once the TDS, if finally approved no request for any alternative section will be entertained. The contractor shall submit to Engineer in Charge 3 copies of all approved TDS out of which 3 copies will be issued back to the contractor for the execution of work. The contractor shall also submit copies of design calculations. Approval of shop drawings however will not absolve the contractor of his responsibility for the safety and correctness of the fabrication.

#### CLAUSE.5.24 :Quality Assurance

The proposed project is a prestigious project and quality of work is of paramount importance. Contractor shall have to engage well-experienced skilled labour and deploy modern T&P and other equipment to execute the work. Many items like double slab structure with exposed finish form works, coarse rubble stone masonry specialized flooring work, Polysulphide sealant and backer rod fixing in expansion joints, factory made door- window shutters, proper slope maintaining in toilet units, sanitary- water supply installation, textured finishing, water proofing treatment with APP, Extruded Polystyrene insulation boards, china mosaic on terrace and chemical treatment in toilet drops will specially require engagement of skilled workers having experience particularly in execution of such items.

The contractor shall ensure quality construction in a planned and time bound manner. Any sub-standard material / work beyond set out tolerance limit shall be summarily rejected by the Engineer-in-charge & contractor shall be bound to replace / remove such sub-standard / defective work immediately. If any material, even though approved by Engineer-In-Charge is found defective or not conforming to specifications shall be replaced / removed by the contractor at his own risk & cost.

In addition to the supervision of work by project management consultants, the Architect Consultants appointed by the NU shall also be carrying out regular and periodic  inspection  of  the  on-going  activities  in  the  work  and  deficiencies,

shortcomings, inferior workmanship pointed out by them shall be communicated by engineer in charge to the contractor. Upon receipt of instructions from Engineer in Charge these are also to be made good by necessary improvement, rectification, replacement up to his complete satisfaction. Special attention shall be paid towards line and level of internal and external finishing, exposed smooth surface of RCC members by providing fresh shuttering plates, rubberized linings to all the shuttering joints, accurate joinery work in wooden doors and windows, thinnest joints in stone/ tiling / cladding work, non-hollowness in floor and dado tiles work, protection of scratches over flooring by impounding layer of plaster of Paris, water tight pipe linings, proper compaction of filled up earth etc. to achieve an Institution of International standards and up keeping of quality assurance shall be of paramount importance, as such.

The Contractor shall submit, within 20 days after the date of award of work, a detailed and complete method statement for the execution, testing and Quality Assurance, of such items of works, as directed by the Engineer-in- Charge. All the materials to be used in the work, to give the finished work complete in all respects, shall comply with the requirements of the Specifications and shall pass all the tests required as per specifications as applicable or such specifications / standards as directed by the Engineer-in- Charge. However, keeping the Quality Assurance in mind, the Contractor shall submit, on request from the Engineer-in- Charge, his own Quality Assurance procedures for basic materials and such items, to be followed during the execution of the work, for approval of the Engineer-in-Charge.

The Contractor shall procure and provide all the materials from the manufacturers / suppliers as per the list attached with the tender documents, as per the item description and particular specifications for the work. The equivalent brand for any item shall be permitted to be used in the work, only when the specified make is not available. This is, however, subject to documentary evidence produced by the contactor for non-availability of the brand specified and also subject to independent verification by the Engineer-in-

Charge. In exceptional cases, where such approval is required, the decision of Engineer-in-Charge as regards equivalent make of the material shall be final and binding on the Contractor. No claim, whatsoever, of any kind shall be entertained from the Contractor on this account. Nothing extra shall be payable on this account. Also, the material shall be procured only after written approval of the Engineer-in-Charge. All materials shall be got checked by the Engineer- in-Charge or his authorized supervisory staff on receipt of the same at site before use.

The tests, as necessary, shall be conducted in the laboratory approved by the Engineer–in- Charge. The samples shall be taken for carrying out all or any of the tests stipulated in the particular specifications and as directed by the Engineer-in- Charge or his authorized representative.

The Contractor shall at his own risk and cost make all arrangements and shall provide all such facilities including material and labour, the Engineer-in-Charge may require for collecting, preparing, forwarding the required number of samples for testing as per the frequency of test stipulated in the contract specifications or as considered necessary by the engineer-in-Charge, at such time and to such places, as directed by the Engineer-in- Charge. Nothing extra shall be payable for the above.

The Contractor or his authorized representative shall associate in collection, preparation, forwarding and testing of such samples. In case he or his authorized representative is not present or does not associate him, the result of such tests and consequences thereon shall be binding on the Contractor .The Contractor or his authorized representative shall remain in contact with the Engineer-in–Charge or his authorized representative associated for all such operations. No claim of payment or claim of any other kind, whatsoever, shall be entertained from the Contractor.

All the hidden items such as water supply lines, drainage pipes, conduits, sewers etc. are to be properly tested as per the design conditions before covering and their measurements in computerized measurement book duly test checked shall be

deposited with Engineer in charge or his authorized representative, prior to hiding these items.

The contractor shall give performance test of the entire installation(s) as per the standing specifications before the work is finally accepted and nothing extra whatsoever shall be payable to the contractor for the test.

The Contractor shall maintain all the work in good condition till the completion of entire work. The Contractor shall be responsible for and shall make good, all damages and repairs, rendered necessary due to fire, rain, traffic, floods or any other causes. The Engineer-in- Charge shall not be responsible for any claims for injuries to person/workmen or for structural damage to property happening from any neglect, default, want of proper care or misconduct on the part of the Contractor or of any other of his representatives, in his employment during the execution of the work. The compensation, if any, shall be paid directly to the Department / authority / persons concerned, by the Contractor at his own cost.

#### CLAUSE. 5.26 :  Special Conditions – Electrical Works

1.	Minimum Criteria for selecting Electrical Subcontractor.

The contractor must hold Valid Class 1 License issued by appropriate authority.

1.	The Electrical contractor must have completed following under one project head in last 7 years for similar kind of projects etc.

A Supply, Installation, Testing & Commissioning of HV/MV/LV Cabling and termination work.

- Supply, Installation, Testing & Commissioning of all M.V. Switch gear and power panels made by CPRI Tested/ approved panel vendor.

- Supply, Installation, Testing & Commissioning of UPS Systems & AVR / Stabilizer systems.

- Internal wiring with PPI (Per Phase Isolation) type Distribution Boards.

- Supply, Installation, Testing & Commissioning of external lighting including poles, Fixtures and cables.

F Electrical sub-contractor should make necessary arrangement with OEM for Supply, Installation, Testing & Commissioning of Stirling engines under the supervision of supplier and all required cabling and synchronization.

#### CLAUSE.5.29 : Special Conditions – Interior Works

###### SHOP DRAWINGS

The contractor should submit shop drawings for all the works including Partition works, Façade works, False ceiling, wall paneling etc for approval to the Architect.

The Façade works shall be designed as per the technical requirement of the project and the contractor shall submit the shop drawings including all the fixing, joinery and sealing details including flashing and all other accessories as per the technical specifications before commencement of the work.

Mockup for all the interior items shall be prepared for approval at site and on the approval of the same from NU and Architect, the contractor can commence the work.

###### ASSOCIATED CIVIL WORKS

Note that all civil works associated with Interior works are included in the Contractor’s scope of work, like wall chasing by wall chaser, making holes etc. for installation of piping/ducting and making good. These shall be executed in accordance with approved shop drawings of the contractor. The minor civil works & structure support for cable trays etc shall be inclusive in the scope

No extra payment / claim shall be made on this account.

###### CO-ORDINATION WITH OTHER WORKS

The Contractor during the execution of the Works shall co-ordinate with other Works, and other Agencies associated with the Project and shall work in harmony with them without causing any hindrance or obstruction or impeding the progress of their work in any way. Shafts/cut-outs (sizes)

confirmation that specified can be accommodated within the available size of shafts etc.

In respect of the work of other Services and Agencies, where the commencement or progress of such work of any other Service, or Agency is dependent upon the completion of particular portions of the Contractor's Works or generally upon the Contractor maintaining progress in accordance with the approved co-ordinate construction programmed, it shall be the responsibility of the Contractor to complete such portions and maintain such progress.

Should any differences arise between the Contractor and the other Works, and Agencies, these shall immediately be brought to the attention of the Construction Manager/Consultants who after reviewing the matters causing the differences will give his decision which shall be final and binding on the Contractor.

###### CO-ORDINATION WITH FIRE PROTECTION SYSTEM WORKS

To collect all relevant information regarding contracts etc. for the operation condition of CHP under fire conditions.

###### CO-ORDINATION WITH CIVIL WORKS

To prepare detailed shop/ working drawings for related works in accordance with the civil construction drawings.

###### CO-ORDINATION WITH HVAC, ELECTRICAL, PHE WORKS

To co-ordinate all relevant information regarding power and exact location of panels.

###### PROJECT EXECUTION AND MANAGEMENT

The Contractor shall ensure that senior planning and erection personnel from his organization are assigned exclusively for this project. The Contractor shall appoint one Project Manager holding senior management position in the organization. He shall be assisted on full time basis by a minimum of two mechanical & electrical engineer & three senior supervisors. The entire staff shall be posted at site on full time basis. Separate ID card to be given by the Contractor to each worker working on site.

The project management shall be through modern technique. The Contractor’s office at site shall be fully equipped with fax, computers &

plotter and shall prepare proper bar chart and completion schedules to be submitted & ensure timely completion. Erection engineer and supervisors shall be provided with mobile communication system so that they can always be reached.

For quality control & monitoring of workmanship, contractor shall assign at least one full-time engineer who would be exclusively responsible for ensuring strict quality control, adherence to specifications and ensuring top class workmanship for the CHP and related installation. Contractor shall furnish details of licenses of supervisors/workmen to be employed at site

#### CLAUSE. 5.30 : Store :

The site store of the Contractor shall be verified at the end of every month by the concerned University's officials certifying the level of minimum inventory, as specified. The Contractor is required to replace the defective work within 12 hours of its failure. A monthly breakdown report shall be submitted to the owner.

#### CLAUSE 5.31 : Inspection:

Bidder/manufacturer shall indicate the various specified sources of the items being procured. Type of checks, quantum of checks and acceptance norms shall be intimated, and random test and check results should be made available for inspection whenever so desired.

#### CLAUSE 5.32 : Pre-Dispatch Inspection:

The contractor will intimate to owner well in advance or with sufficient notice period for readiness of material at manufactures’ works. The Employer/his representative may carry out stage inspection of the plant/equipment during manufacturing/ assembling stage. The Employer shall have absolute right to reject the raw material/ component/ sub-assemblies or complete equipment not found to be conforming to the specification or being of poor quality/ workmanship. The stage inspection will particularly include tests specified for any particular plant or equipment in the technical specification, general routine tests and physical measurements to be conducted during manufacturing stages as per manufacturer’s standard practice.

The Bidder/manufacturer shall give fifteen (15) days advance notice to enable the Employer to arrange for inspection. Inspection and testing shall be conducted at the place of manufacture. Inspection and testing of any material under this specification by the Employer shall not relieve the Bidder/manufacturer of his obligation to supply the plant/equipment in accordance with the specification and shall not prevent subsequent rejection if it is found to be defective.

The Bidder/manufacturer shall afford the Employer’s representative all reasonable facilities, without charge, to satisfy him that the plant/ equipment is being manufactured in accordance with the specifications. The Bidders/manufacturer must have adequate set of instruments for conducting tests as per IS specification. Instruments for testing shall be duly calibrated and calibration certificate should not be older than one year on the date of inspection. Calibration shall be done by NABL accredited laboratories. A comprehensive list of testing equipment/ instruments indicating make, Sr. No, type, class of accuracy, calibrating agency, calibration date etc. should be furnished, as and when desired. Calibrated instruments shall be duly sealed by calibrating agency to avoid any tampering with calibration and the details thereof shall be clearly mentioned in the calibration certificate(s).

#### CLAUSE 5.33: Material Supply Lot

Only tested material shall be supplied in compliance of relevant IS. The Test should be issued by NABL or NABL accredited laboratory and it should not be older than 3 years.

#### CLAUSE 5.34: Fake Inspection Call

In case of fake inspection call announced by the inspecting officer or University’s authority, a penalty of Rs. 25000/- against Contractor shall be imposed and such amount shall be deducted from the running bill of the contractor.

#### CLAUSE.5.35: Special Conditions For Directives Of Hon'ble National Green Tribunal 04.12.2014 & 10.04.2015 And EIA Guidance Manual

- The contractor shall not store/dump construction material or debris on metalled road.

- The contractor shall get prior approval from Engineer-in-Charge for the area where the construction material or debris can be stored beyond the metalled road. This area shall not cause any obstruction to the free flow of traffic/inconvenience to the pedestrians. It should  be ensured by the contractor that no accidents occur on account of such permissible storage.

- The contractor shall take appropriate protection measures like raising wind breakers of appropriate height on all sides of the plot/area using CGI sheets or plastic and /or other similar material to ensure that no construction material dust fly outside the plot area.

- The contractor shall ensure that all the trucks or vehicles of any kind which are used for construction purposes/or are carrying construction material like cement, sand and other allied material are fully covered. The contractor shall take every necessary precautions that the vehicles are properly cleaned and dust free to ensure that en- route their destination, the dust, sand or any other particles are not released in air/contaminate air.

- The contractor shall provide mask to every worker working on the construction site and involved in loading, unloading and carriage of construction material and construction debris to prevent inhalation of dust particles.

- The contractor shall provide all medical help, investigation and treatment to the workers involved in the construction of building and carry of construction material and debris relatable to dust emission.

- The contractor shall ensure that C&D waste is transported to the C & D Waste site only anddue record shall be maintained by the contractor.

- The contractor shall compulsory use of wet jet in grinding and stone cutting.

- The contractor shall comply all the preventive and protective environmental steps as stated in the MoEF guidelines, 2010.

- The contractor shall carry out on-Road- Inspection for black smoke generating machinery. The contractor shall use cleaner fuel.

- The contractor shall ensure that all DG sets comply emission norms notified by MoEF.

- The contractor shall use vehicles having pollution under control certificate. The emissions can be reduced by a large extent by reducing the speed of a vehicle to 20 kmph. Speed bumps shall be used to ensure speed reduction. In cases where speed reduction cannoteffectively reduce fugitive dust, the contractor shall divert traffic to nearby paved areas.

- The contractor shall ensure that the construction material is covered by tarpaulin. The contractor shall take all other precaution to ensure that no dust particles are permitted to pollute air quality as a result of such storage.

- The paving of the path for plying of vehicles carrying construction material is more permanent solution to dust control and suitable for longer duration projects. The NIT approving authority shall carry out cost benefit ratio analysis of the same.

SECTION 6

## ADDITIONAL CONDITIONS FOR GREEN BUILDING PRACTICES

6.1 The campus is proposed to be registered for obtaining GRIHA LD Rating from GRIHA Secretariat under MNRE scheme. The contractor is required to execute the work in a befitting manner to obtain the targeted GRIHA rating by Owner.

Special conditions for GRIHA rating:-

The contractor shall prepare scheme for the approval of Engineer-in- charge for obtaining GRIHA rating in the criteria relevant to the execution of work.

The contractor shall plan and execute the work in a manner to preserve and protect the landscape during construction and shall arrange the materials/equipment and follow the procedure as per criterion 2 of the GRIHA rating as applicable.

The contractor shall appoint/engage consultant/ consultancies to provide technical guidance and supervise the work, pertaining to the criterion related to the execution of work, so that it finally achieves the targeted GRIHA rating.

The contractor shall preserve the topsoil layer for supporting vegetative growth as per the guidelines given in chapter 4, Section 1, Part 10, NBC 2005.

The contractor shall comply with NBC norms on construction safety, health and sanitation as per criterion.

The construction activity shall be done in a befitting manner and the contractor shall adopt measures to prevent air pollution at site in compliance with criterion 9 of GRIHA rating as applicable.

The contractor shall comply with all the instructions and schemes for execution of green building.

Nothing shall be paid extra for fulfilment of all these conditions except for the items existing in the schedule of quantities. For such items work done shall be paid on the basis of the agreement rates.

6.3	Pre- construction stage

Construction Vehicles, Equipment and Machinery

All vehicles, equipment and machinery to be procured for construction shall conform to the relevant Bureau of India Standard (BIS) norms.

Emission from the vehicles must conform to environmental norms.

Dust produced from the vehicular movement and other site activities is to be mitigated by sprinkling of water. Noise limits for construction equipment shall not exceed 75 dB(A), measured at one meter from the edge of the equipment in free area, as specified in the Environment Protection Act,1986, schedule VI part E, as amended on 9th May,1993. The maximum noise levels near the construction site should be limited to 65 dB (A) Leq (5 min) in project area. Contractor has to make a confined boundary to restrict construction activities within it. Contractor is required to prepare the Construction waste management plan for segregation, storage and safe disposal of construction waste for the approval of engineer in charge.

Construction Stage

    - Construction Wastes Disposal

The pre-identified dump locations will be a part of solid waste management plan OR construction waste management plan or safe handling, storage and recycling of construction waste to be prepared by the Contractor in consultation with Engineer- in-charge. Contractor needs to designate the area for construction waste storage. Inert and hazardous waste should be collected and stored separately on site.

The other construction waste should be either reused on site or safely dispose them off to designed agencies for recycling. The procedure of waste disposal either on- site reuse or selling for recycling purpose should be documented by photographs / log books / receipt copies and evidences needs to be submitted to the Engineer In charge. Contractor shall get approved the location of disposal site prior to commencement of the excavation on any section of the project location.

Contractor shall ensure that any spoils of material will not be disposed of in any municipality solid waste collection bins.

    - Procurement of Construction Materials

All vehicles delivering construction materials to the site shall be covered to avoid spillage of materials and maintain cleanliness of the roads.

Wheel Tires of all vehicles used by of the contractor, or any of his sub-contractor or materials supplies shall be cleaned and washed clear of all dust/mud before leaving the project premises. This shall be done by routing the vehicles through tire washing tracks.

Contractor shall arrange for regular water sprinkling at least twice a day (i.e. morning and evening) for dust suppression of the construction sites and unpaved roads used by his construction vehicles.

    - Water Pollution

The Contractor shall take all precautionary measures to prevent the wastewater during construction to accumulate anywhere. The wastewater arising from the project is to be disposed of in the manner that is acceptable to the Engineer-in- charge.

    - Air and Noise Pollution

Contractor shall use dust screens and sprinkle water around the construction site to arrest spreading of dust in the air and surrounding areas.

Contractor shall ensure that all vehicles, equipment and machinery used for construction are regularly maintained and confirm that emission levels comply with environmental emission standards/norms.

For controlling the noise from Vehicles, Plants and Equipment, the Contractor shall confirm the following:

All vehicles and equipment used in construction will be fitted with exhaust silencers.

Servicing of all construction vehicles and machinery will be done regularly and during routine servicing operations, the effectiveness of exhaust silencers will be checked and if found defective will be replaced.

Noise emission from compactors (rollers) front loaders, concrete mixers, cranes (movable), vibrators and saws should be less than 75 dB(A).

As per the standards/guidelines for control of Noise Pollution from Stationary Diesel Generator (DG) sets, noise emission in dB(A) from DG Set (15-500 KVA) should be less than 94+10 log 10 (KVA). The standards also suggest construction of acoustic enclosure around the DG Set and provision of proper exhaust muffler with insertion loss of minimum 25 dB (A) as mandatory.

    - Personal Safety Measures for Labour

Contractor will provide the following items for safety of workers employed by contractor and associate agencies:

Protective footwear and gloves to all workers employed for the work on mixing, cement, lime mortars, concrete etc. and openings in water pipeline/sewer line.

Welders’ protective eye-shields for all workers who are engaged in welding works. Safety helmet and Safety harness/ belt.

The following provisions shall be maintained by the contractor at

site:

    - Provide adequate sanitation/safety facilities for construction workers to ensure the health and safety of the workers during construction, with effective provisions for

the basic facilities such as sanitation, drinking water and safety equipment or machinery.

All the workers should be wearing helmet and shoes all the time on site. Masks and gloves should be worn whenever and wherever required.

Adequate drinking water facility should be provided at site, adequate number of decentralized latrines and urinals to be provided for construction workers.

Full time workers residing on site should be provided with clean and adequate temporary hutment. Crèche/Day-care facility should be provided to young children of laborers residing on site. Child labour should be banned on site.

First aid facility should also be provided.

Overhead lifting of heavy materials should be avoided. Barrow wheel and hand-lift boxes should be used to transport materials onsite.

Tobacco and cigarette smoking should be prohibited onsite.

All dangerous parts of machinery are well guarded and all precautions for working on machinery are taken.

Maintain hoists and lifts, lifting machines, chains, ropes and other lifting tackles in good condition.

Use of durable and reusable formwork systems to replace timber formwork and ensure that formwork where used is properly maintained.

Ensure that walking surfaces or boards at height are of sound construction and are provided with safety rails and belts. Provide protective equipment such as helmets.

Provide measure to prevent fire. Fire extinguisher and buckets of sand to be provided in fire-prone area and elsewhere.

Provide sufficient and suitable light for working during night.

Ensure that measures to protect workers from materials of construction, transportation, storage and other dangers and health hazards are taken.

Ensure that the construction firm/division/company have sound safety policies.

Comply with the safety procedure, norms and guidelines (as applicable) as outlined in NBC 2005 (BIS 2005c).

Adopt additional best particles and prescribed norms as in NBC 2005 (BIS 2005).

    - Identify roads on-site that would be used for vehicular traffic. Update vehicular roads (if these are unpaved) by increasing the surface strength by improving particle size, shape and mineral type that make up the surface base. Add surface gravel to

reduce source of dust emission. Limit amount of fine particles (smaller than 0.075mm) to 10-20%. Limit vehicular speed on site 10km/h.

    - All material storages should be adequately covered and contained so that they are not exposed to situations where winds on site could lead to dust/particulate emissions.

    - Spills of dirt or dusty materials shall be cleaned up promptly so the spilled material does not become a source of fugitive dust and also to prevent of seepage of pollutant laden water into the ground aquifers. When cleaning up the spill, ensure that the clean-up process does not generate additional dust. Similarly, spilled concrete slurries or liquid wastes should be contained/cleaned up immediately before they can infiltrate into the soil/ground or runoff in nearby areas.

    - Ensure that water spraying is carried out by wetting the surface by spraying water on:

Any dusty material.

Areas where demolition works is carried out. Any unpaved main-haul road and.

Areas where excavation or earth moving activities is to be carried out.

The contractor shall ensure the following:

Cover and enclose the site by providing dust screen, sheeting or netting to scaffold along the perimeter of a building.

Covering stockpiles of dusty material with impervious sheeting.

Covering dusty load on vehicles by impervious sheeting before they leave the site.

Transferring, handling/storing dry loose materials like bulk cement and dry pulverized fly ash inside a totally enclosed system.

Spills of dirt or dusty materials shall be cleaned up promptly so that the spilled material does not become a source of fugitive dust and also to prevent seepage of pollutant-laden water into the ground aquifers. When cleaning up the spill, ensure that the clean-up process does not generate additional dust. Similarly, spilled concrete slurries or liquid wastes should be contained/cleaned up immediately before they can infiltrate into the soil/ground or runoff in nearby areas.

Clear vegetation only from areas where work will start right away. Vegetate/mulch areas where vehicles do not ply.

Apply gravel/landscaping rock to the areas where mulching/paving is impractical.

    - Adopt measures to prevent air pollution in the vicinity of the site due to construction activities. There is no standard reference for this. The best practices should be followed (as adopted from international best practice documents and codes).

    - Provide hoardings of not less than 3m height along the site boundary, next to a road or other public area.

    - The contractor shall provide experienced personnel with suitable training to ensure that these methods are implemented. Prior to the commencement of any work, the method of working, plant equipment and air pollution control system to be used on-site should be made available for the inspection and approval of the Engineer-in-Charge to ensure that these are suitable for the project.

    - Employ measures to segregate the waste on-site into inert, chemical or hazardous wastes. Recycle the unused chemical/hazardous wastes such as oil, paint, batteries and asbestos. The inert waste is to be disposed of to Municipal Corporation/local bodies dump yard and landfill sites.

Preserve and protect landscape during construction

    - Following provisions shall be made at site by the contractor to preserve and protect landscape. Nothing shall be paid on this account unless specifically provided for in the schedule of quantities.

    - To preserve the existing landscape and protect it from degradation during the process of construction. Select proper timing for construction activity to minimize the disturbance such as soil pollution due to spilling of the construction material and its mixing with rainwater. The construction management plan including soil erosion control management plan shall be prepared accordingly for each month. The application of erosion control measures includes construction of gravel pits and tyre washing bays of approved size and specification for all vehicular site entry/ exits, protection of steep slopes. Sedimentation Collection System and run-off diversion systems shall be in place before the commencement of construction activity. Preserve and protect the existing vegetation by not disturbing or damaging to specified site areas during construction. The trees that are identified to be retained on site are protected during the construction period using the following measures:

The damage to roots is prevented during trenching, placing backfill, driving or parking heavy equipment. The dumping of trash, oil, paint and other material is detrimental to plant health. These activities should be restricted to the areas outside of the canopy of the trees.

The trees are not used for support; their trunks should not be damaged by cutting and carving by nailing posters and advertisements or in any other way.

The lighting of fires or carrying out heat or gas emitting construction activity within the ground covered by canopy of the trees is not permitted.

The young trees of saplings identified for preservation within the construction site must be protected using tree guards of approved specification.

The grades of soil should be maintained around existing vegetation. Lowering or raising the levels around the vegetation should not be allowed unless specifically directed by the Engineer –in – Charge.

Maintenance activities should be performed, as and when needed, to ensure that vegetation remain healthy.

  - Staging is dividing a construction area into two or more sections to minimize the area of soil that will be exposed at any given time. Staging should be done to separate undisturbed land from land disturbed by construction activity and material storage. A vector drawing plan to be submitted identifying the areas of sites, which shall be disturbed for the construction activity and apart from these other areas on site should not be disturbed. Measures should be followed for collecting drainage water run-off from construction areas and material storage sites and diverting water flow away from such polluted areas. Temporary drainage channels and perimeter dike/swale should be constructed to carry the pollutant-laden water directly to the treatment device or facility (municipal sewer line). The plan should indicate how the above was accomplished on site well in advance of the commencement of the construction activity.

    - The Contractor should follow the construction plan as proposed by the Architect Consultants/landscape consultant to minimize the site disturbance such as soil pollution due to spilling. Use staging and spill prevention and control plan to restrict the spilling of the contaminating material on site. Protect top soil from erosion by collection storage and reapplication of top soil, constructing sediment basin, contour trenching, mulching etc.

    - Spill prevention and control plans should clearly state measures to stop the source of the spill. Measures to contain the spill and measures to dispose the contaminated material and hazardous wastes. It should also state the designation of personnel trained to prevent and control spills. Hazardous wastes include pesticides, paints, cleaners and petroleum products.

A soil Erosion and Sedimentation Control Plan (ESCP) should be prepared prior to construction and  should be applied effectively. Measures for prevention of top soil are given below:

    - Top Soil should be stripped to a depth of 20 cm from the areas proposed for buildings, roads, paved areas and external services. It should be stockpiled to a

maximum height of 40 cm in designated areas and reapplied during plantation of the proposed vegetation. The topsoil should be separated from the sub soil debris and stones larger than 50 mm in diameter. The stored topsoil may be used as finished grade for planting areas post construction or cordoned off undisturbed areas on site. Stockpiled topsoil should not be compacted to help process of aeration. It should be stabilized on the top by temporary seeding or plastic sheets to avoid wind and water erosion. During construction period contractor has to preserve and protect existing nalas or natural drainage channel at site of work.

    - Sedimentation basin, a temporary dam or basin at the lowest convenient point of the site should be constructed for collecting, trapping and storing sediment produced by the construction activities. A flow detention facility must also be constructed for reducing peak run-off rates. This would also allow most of the sediments to settle before the run-off is directed towards the outfall.

    - Contour trenching is to be provided which an earth embankment or ridge-and- channel arrangement constructed parallel to the contours, along the face of the slope, at regular intervals on the lengths and steep slopes. They are used for reducing run- off velocity, increasing the distance of overland run-off flow. They are also used to hold moisture and minimize sediment loading of surface run-off.

  - Prepare the list of trees to be felled with reference to the tree survey, Compensate the loss of vegetation (trees) due to the construction activity by compensatory plantation. Replant same native and/or non-invasive species, which existed on the site before elimination, in the proportion of 1:3 (as per the suggestion of the landscape consultant).

  - The contractor shall prepare and submit ‘Spill prevention and control plans’ before the start of construction, clearly stating measures to stop the source of the spill, to contain the spill, to dispose the contaminated material and hazardous wastes, and stating designation of personnel trained to prevent and control spills. Hazardous wastes include pesticides, paints, cleaners, and petroleum products.

  - The contractor shall ensure that no construction leaches (Ex: cement slurry) is allowed to percolate into the ground. Adequate precautions are to be taken to safeguard against this including reduction of wasteful curing processes, collection, basic filtering and reuse. The contractor shall follow requisite measures for collecting drainage water run-off from construction areas and material storage sites and diverting water flow away from such polluted areas. Temporary drainage channels, perimeter dike/swale, etc. shall be constructed to carry the pollutant-laden water directly to the treatment device or facility (municipal sewer line).

  - All lighting installed by the contractor around the site and at the labour quarters during construction shall be CFL bulbs of the appropriate illumination levels. This condition is a must, unless specifically prescribed.

  - All paints, adhesives and sealants should comply with the VOC limits prescribed by GRIHA, as follows:

  - All the building materials and systems used on site must be as per the specifications and approved makes by the consultants.

  - All required certificates explaining the properties of the building material/system needs to be obtained from the manufacturer/vendor as required by the green building rating authority.

The final certificates would be produced after the approval of green building consultant with necessary due diligence.

The purchase orders of all the materials made with the manufacturers/authorized vendors should be maintained and shall be provided for the process with due diligence upon request.

  - Water saving measures as suggested by the consultants need to be followed on site.

  - The contractor / subcontractor shall prepare and submit a Site Management Plan (SMP) within 10 days of start, for approval by the Engineer-in-charge. This SMP shall indicate the locations of godown, stockpiles, barricading, waste storage, offices, vehicular movement routes etc. In short this SMP would comprehensively represent how the site activities shall be managed conforming to GRIHA guidelines.

  - Any other site management measures suggested by the Engineer-in-charge / green building consultant shall be followed on site.

  - The contractor shall submit to the Engineer-in-Charge after construction of the buildings, a detailed as built quantification of the following:

Total materials used,

Total top soil stacked and total reused, Total earth excavated,

Total waste generated, Total waste reused, Total water used,

Total electricity consumed, and

Total diesel consumed.

  - The contractor shall submit to the engineer-in-charge, before the start of construction, a site plan along with a narrative to demarcate areas on site from which top soil has to be gathered, designate area where it will be stored, measures adopted for top soil preservation and indicate areas where it will be reapplied after construction is complete.

  - Evidence for the implementation of the all the above required measures shall be provided to the green building consultant in the form of photographs and templates (as provided), which is required for the submission to the green building rating authority (GRIHA).

  - Nothing extra shall be payable for above provisions unless otherwise specified in Schedule of Quantities.

###### SPECIAL CONDITIONS FOR ENVIRONMENT MANAGEMET PLAN

- The contractor shall obtain approval for laying electrical lines from the concerned SE of BERC/BSPHCL/SBPDCL and comply with the provisions as per Terms  and condition for determination of Tariff Regulation, 2007 of for construction purpose as well as for final connection.

- The contractor shall ensure taking necessary steps on urgent basis to improve the living conditions of the labour at site and provide necessary facility to the labour.

- Contractor has to construct housing colony for labour within the site with all necessary infrastructure and facilities such as health facility, sanitation facility, and fuel for cooking, along with safe drinking water, medical camps, and toilets for women, crèche for infants. The housing may be in the form of temporary structures to be removed after the completion of the project. Details of provisions should be submitted to Engineer In charge for them to submit it to Bihar State Pollution Control Board at the time of obtaining Consent to Establish.

- During construction period mobile STP of capacity 60 & 20 KLD shall be provided by the contractor for the labour colony. The drains should be of adequate capacity and be lined till the final disposal points. Provision for disinfection of wastewater after treatment and before reuse to be ensured by the contractor.

- All required sanitary and hygienic measures shall be in place before starting construction activities. The safe disposal of wastewater and solid waste generated during the Construction phase shall be ensured.

- All the laborers engaged for construction shall be screened for health and adequately treated before engaging them to work at the site.

- All the topsoil excavated during the construction shall be stored for use in horticulture/landscape development within the project site.

- Disposal of muck during construction phase shall not create any adverse effect on the neighbouring communities and be disposed taking the necessary precautions for general safety and health aspects of people only in approved sites with approve competent authority.

- The contractor to ascertain that, there is no threat to the ground water quality by leaching of heavy metals and other toxic contaminants during construction will test soil and ground water samples.

- Construction spoils, including bituminous material and other hazardous materials must not be allowed to contaminate water courses and the dump sites for such material must be secured so that they do not leach into the ground water.

- The diesel generator sets to be used during construction phase shall be of low- sulphur diesel type and shall conform to Environment (Protection) Rules for air and noise emission standards.

- Vehicles hired for bringing construction  material and laborers to the site shall be in good conditions and shall conform to applicable air and noise emission standards and shall be operated during non-peak/approved hours.

- Ambient noise levels shall conform to residential standards both during day and night. Incremental pollution loads on the ambient air and noise quality shall be closely monitored during construction phase.

- NOC shall  be obtained  from National  State Disaster  Management Authority, · wherever applicable.

- Water demand during construction shall be reduced by the use of pre-mixed concrete, curing agents and other best practices.

- Total domestic water requirement shall not exceed 94 KLD during construction stage.

- Adequate measures shall be taken to reduce air and noise pollution during construction as per CPCB norms.

- A First Aid Room should be provided at the project site during construction phase of the project.

- Any hazardous  waste generated  during construction  phase shall  be disposed of as per applicable rules and norms with necessary authorization of the BSPCB.

- Regular supervision of the above and other measures for monitoring shall be done by Engineer In charge throughout the construction phase, so as to avoid nuisance to the surroundings.

SECTION 7

## C.P.W.D. SAFETY CODES AND RULES

  - Suitable scaffolds should be provided for workmen for all works that cannot safely be done from the ground, or from solid construction except such short period work as can be done safely from ladders. When a ladder is used, an extra mazdoor shall be engaged for holding the ladder and if the ladder is used for carrying materials as well suitable footholds and hand-hold shall be provided on the ladder and the ladder shall be given an inclination not steeper than ¼ to 1(¼ horizontal and 1 vertical.)

  - Scaffolding of staging more than 3.6 m (12ft.) above the ground or floor, swung or suspended from an overhead support or erected with stationary support shall have a guardrail properly attached or bolted, braced and otherwise secured at least 90 cm. (3ft.) high above the floor or platform of such scaffolding or staging and extending along the entire length of the outside and ends there of with only such opening as may be necessary for the delivery of materials. Such scaffolding or staging shall be so fastened as to prevent it from swaying from the building or structure.

  - Working platforms, gangways and stairways should be so constructed that they should not sag unduly or unequally, and if the height of the platform or the gangway or the stairway is more than 3.6 m (12ft.) above ground level or floor level, they should be closely boarded, should have adequate width and should be suitably fastened as described in (2) above.

  - Every opening in the floor of a building or in a working platform shall be provided with suitable means to prevent the fall of person or materials by providing suitable fencing or railing whose minimum height shall be 90 cm. (3ft.)

  - Safe means of access shall be provided to all working platforms and other working places. Every ladder shall be securely fixed. No portable single ladder shall be over 9m. (30ft.) in length while the width between side rails in rung ladder shall in no case be less than 29 cm.(11½") for ladder up to and including 3 m. (10 ft.) in length. For longer ladders, this width should be increased at least ¼” for each additional 30 cm. (1 foot) of length. Uniform step spacing of not more than 30 cm shall be kept. Adequate precautions shall be taken to prevent danger from electrical equipment. No materials on any of the sites or work shall be so stacked or placed as to cause danger or inconvenience to any person or the public. The contractor shall provide all necessary fencing and lights to protect the public from accident and shall be bound to bear the expenses of defence of every suit, action or other proceedings at law that may be brought by any person for injury sustained owing to neglect of the above precautions and to pay any damages and cost which may be awarded in any such suit; action or proceedings to any such person or which may, with the consent of the contractor, be paid to compensate any claim by any such person.

  - Excavation and Trenching - All trenches 1.2 m. (4ft.) or more in depth, shall at all times be supplied with at least one ladder for each 30 m. (100 ft.) in length or fraction thereof, Ladder shall extend from bottom of the trench to at least 90 cm. (3ft.) above the surface of the ground. The side of the trenches which are 1.5 m. (5ft.) or more in depth shall be stepped back to give suitable slope or securely held by timber bracing, so as to avoid the danger of sides collapsing. The excavated materials shall not be placed within 1.5 m. (5ft.) of the edges of the trench or half of the depth of the trench whichever is more. Cutting shall be done from top to bottom. Under no circumstances, undermining or undercutting shall be done.

  - Demolition - Before any demolition work is commenced and also during the progress of the work,

    - All roads and open areas adjacent to the work site shall either be closed or suitably protected.

    - No electric cable or apparatus which is liable to be a source of danger or a cable or apparatus used by the operator shall remain electrically charged. (iii) All practical steps shall be taken to prevent danger to persons employed from risk of fire or explosion or flooding. No floor, roof or other part of the building shall be so overloaded with debris or materials as to render it unsafe.

  - All necessary personal safety equipment as considered adequate by the Engineer-in-Charge should be kept available for the use of the person employed on the site and maintained in a condition suitable for immediate use, and the contractor should take adequate steps to ensure proper use of equipment by those concerned:- The following safety equipment shall invariably be provided.

- Workers employed on mixing asphaltic materials, cement and lime mortars shall be provided with protective footwear and protective goggles.

- Those engaged in white washing and mixing or stacking of cement bags or any material which is injurious to the eyes, shall be provided with protective goggles.

- Those engaged in welding works shall be provided with welder’s protective eye- shields.

- Stone breaker shall be provided with protective goggles and protective clothing and seated at sufficiently safe intervals.

- When workers are employed in sewers and manholes, which are in active use, the contractors shall ensure that the manhole covers are opened and ventilated at least for an hour before the workers are allowed to get into the manholes, and the manholes so opened shall be cordoned off with suitable railing and provided with

warning signals or boards to prevent accident to the public. In addition, the contractor shall ensure that the following safety measure are adhered to :-

  - Entry for workers into the line shall not be allowed except under supervision of the safety officer or any other higher official.

  - At least 5 to 6 manholes upstream and downstream should be kept open for at least 2 to 3 hours before any man is allowed to enter into the manhole for working inside.

  - Before entry, presence of Toxic gases should be tested by inserting wet lead acetate paper which changes colour in the presence of such gases and gives indication of their presence.

  - Presence of Oxygen should be verified by lowering a detector lamp into the manhole. In case, no Oxygen is found inside the sewer line, workers should be sent only with Oxygen kit.

  - Safety belt with rope should be provided to the workers. While working inside the manholes, such rope should be handled by two men standing outside to enable him to be pulled out during emergency.

  - The area should be barricaded or cordoned off by suitable means to avoid mishaps of any kind. Proper warning signs should be displayed for the safety of the public whenever cleaning works are undertaken during night or day.

  - No smoking or open flames shall be allowed near the blocked manhole being cleaned.

  - The malba obtained on account of cleaning of blocked manholes and sewer lines should be immediately removed to avoid accidents on account of slippery natureof the malba.

  - Workers should not be allowed to work inside the manhole continuously. He should be given rest intermittently. The Engineer-in-Charge may decide the time up to which a worker may be allowed to work continuously inside the manhole.

  - Gas masks with Oxygen Cylinder should be kept at site for use in emergency.

  - Air-blowers should be used for flow of fresh air through the manholes. Whenever called for, portable air blowers are recommended for ventilating the manholes. The Motors for these shall be vapour proof and of totally enclosed type. Non-sparking gas engines also could be used but they should be placed at least 2metres away from the opening and on the leeward side protected from

wind so that they will not be a source of friction on any inflammable gas that might be present.

  - The workers engaged for cleaning the manholes/sewers should be properly trained before allowing to work in the manhole.

  - The workers shall be provided with Gumboots or non-sparking shoes bump helmets and gloves non sparking tools safety lights and gas masks and portable air blowers (when necessary). They must be supplied with barrier cream for anointing the limbs before working inside the sewer lines.

  - Workmen descending a manhole shall try each ladder stop or rung carefully before putting his full weight on it to guard against insecure fastening due to corrosion of the rung fixed to manhole well.

  - If a man has received a physical injury, he should be brought out of the sewer immediately and adequate medical aid should be provided to him.

  - The extent to which these precautions are to be taken depend on individual situation but the decision of the Engineer-in-Charge regarding the steps to be taken in this regard in an individual case will be final.

- The Contractor shall not employ men and women below the age of 18 years on the work of painting with products containing lead in any form. Wherever men above the age of 18 are employed on the work of lead painting, the following precaution should be taken:-

  - No paint containing lead or lead products shall be used except in the form of paste or readymade paint.

  - Suitable face masks should be supplied for use by the workers when paint is applied in the form of spray or a surface having lead paint is dry rubbed and scrapped.

  - Overalls shall be supplied by the contractors to the workmen and adequate facilities shall be provided to enable the working painters to wash during and on the cessation of work.

  - The Contractor shall not employ women and men below the age of 18 on the work of painting with product containing lead in any form, wherever men above the age of 18 are employed on the work of lead painting, the following principles must be observed for such use :

- White lead, sulphate of lead or product containing these pigment, shall not be used in painting operation except in the form of pastes or paint ready for use.

- Measures shall be taken, wherever required in order to prevent danger arising from the application of a paint in the form of spray.

- Measures shall be taken, wherever practicable, to prevent danger arising out of from dust caused by dry rubbing down and scraping.

- Adequate facilities shall be provided to enable working painters to wash during and on cessation of work.

- Overall shall be worn by working painters during the whole of working period.

- Suitable arrangement shall be made to prevent clothing put off during working hours being spoiled by painting materials.

- Cases of lead poisoning and suspected lead poisoning shall be notified and shall be subsequently verified by medical man appointed by competent authority.

- Competent authority may require, when necessary medical examination of workers.

- Instructions with regard to special hygienic precautions to be taken in the painting trade shall be distributed to working painters.

  - When the work is done near any place where there is risk of drowning, all necessary equipment should be provided and kept ready for use and all necessary steps taken for prompt rescue of any person in danger and adequate provision, should be made for prompt first aid treatment of all injuries likely to be obtained during the course of the work.

  - Use of hoisting machines and tackle including their attachments, anchorage and supports shall conform to the following standards or conditions :-

- (a) These shall be of good mechanical construction, sound materials and adequate.

(b) Every rope used in hoisting or lowering materials or as a means of suspension shall be of durable quality and adequate strength, and free from patent defects.

- Every crane driver or hoisting appliance operator, shall be properly qualified and no person under the age of 21 years should be in charge of any hoisting machine including any scaffolding winch or give signals to operator.

- In case of every hoisting machine and of every chain ring hook, shackle swivel and pulley block used in hoisting or as means of suspension, the safe working load shall be ascertained by adequate means. Every hoisting machine and all gear referred to above shall be plainly marked with the safe working load. In case of a hoisting machine having a variable safe working load each safe working load and

the condition under which it is applicable shall be clearly indicated. No part of any machine or any gear referred to above in this paragraph shall be loaded beyond the safe working load except for the purpose of testing.

- In case of Owner machines, the safe working load shall be notified by the Electrical Engineer deputed by the Engineer-in-Charge. As regards contractor’s machines the contractors shall notify the safe working load of the machine to the Engineer-in-Charge whenever he brings any machinery to site of work and get it verified by the Electrical Engineer concerned.

  - Motors, gearing, transmission, electric wiring and other dangerous parts of hoisting appliances should be provided with efficient safeguards. Hoisting appliances should be provided with such means as will reduce to the minimum the risk of accidental descent of the load. Adequate precautions should be taken to reduce to the minimize the risk of any part of a suspended load becoming accidentally displaced. When workers are employed on electrical installations which are already energized, insulating mats, wearing apparel, such as gloves, sleeves and boots as may be necessary should be provided. The worker should not wear any rings, watches and carry keys or other materials which are good conductors of electricity.

  - All scaffolds, ladders and other safety devices mentioned or described herein shall be maintained in safe condition and no scaffold, ladder or equipment shall be altered or removed while it is in use. Adequate washing facilities should be provided at or near places of work.

  - These safety provisions should be brought to the notice of all concerned by display on a notice board at a prominent place at work spot. The person responsible for compliance of the safety code shall be named therein by the contractor.

  - To ensure effective enforcement of the rules and regulations relating to safety precautions the arrangements made by the contractor shall be open to inspection by Engineer-in-Charge or their representatives.

  - WARNING/ CAUTION BOARDS: All temporary warning / caution boards / glow signage display such as "Construction Work in Progress", "Keep Away", “No Parking”, Diversions & protective Barricades etc. shall be provided and displayed during day time by the Contractor, wherever required and as directed by the Engineer-in-Charge. These glow signage and red lights shall be suitably illuminated during night also. The Contractor shall be solely responsible for damage and accident caused, if any, due to negligence on his part. Also he shall ensure that no hindrance, as far as possible, is caused to general traffic during execution of the work. This signage shall be dismantled & taken away by the Contractor after the completion of work, only after approval of the Engineer – in – Charge. Nothing extra shall be payable on this account.

  - SIGN BOARDS: The Contractor shall provide and erect a display board of size and shape as required and paint over it, in a legible and workman like manner, the details about the salient features of the project, as required by the Engineer-in- Charge. The Contractor shall fabricate and put up a sign board in an approved location and to an approved design indicating name of the project, Owner / owner, architects, structural consultants, University etc. besides providing space for names of other Contractors, Sub-Contractors and specialized agencies. Nothing extra shall be payable on this account. Necessary protective and safety equipment shall be provided to the Site Engineer, Supervisory staff, labour and technical staff of the contractor by the Contractor at his own cost and to be used at site. No inflammable materials including

P.O.L shall be allowed to be stored in huge quantity at site. Only limited quantity of

P.O.L may be allowed to be stored at site subject to the compliance of all rules / instructions issued by the relevant authorities and as per the direction of Engineer -in- Charge in this regard. Also all precautions and safety measures shall be taken by the Contractor for safe handling of the P.O.L products stored at site. All consequences on account of unsafe handling of P.O.L shall be borne by the Contractor.

  - The PMC of the project has prepared an EHS manual and exhaustive process and guidelines for the EHS implementation plan at site. The documents and the manual containing these process and guidelines are enclosed separately with the tender documents as “EHS Manual”. Contractors are expected to go through all the provisions of this EHS manual and apprise themselves of these requirements before submitting their tenders as they would be required during the execution of the project to strictly adhere to those guidelines, provisions and processes.

SECTION 8

Model Rules for the Protection of Health and Sanitary Arrangements for Workers Employed by Nalanda University (As per Standard Central P.W.D. Procedure)or its Contractors

###### APPLICATION

These rules shall apply to all buildings and construction works in charge of Nalanda University (Central Public Works University/ PWD (DA) )in which twenty or more workers are ordinarily employed or are proposed to be employed in any day during the period during which the contract work is in progress.

###### DEFINITION

Work place means a place where twenty or more workers are ordinarily employed in connection with construction work on any day during the period during which the contract work is in progress.

###### FIRST-AID FACILITIES

- At every work place, there shall be provided and maintained, so as to be easily accessible during working hours, first-aid boxes at the rate of not less than one box for 150 contract labour or part thereof ordinarily employed.

- The first-aid box shall be distinctly marked with a red cross on white back ground and shall contain the following equipment:-

  - For work places in which the number of contract labour employed does not exceed 50-

Each first-aid box shall contain the following equipment’s:-

- 6 small sterilized dressings.

- 3 medium size sterilized dressings.

- 3 large size sterilized dressings.

- 3 large sterilized burn dressings.

- 1 (30 ml.) bottle containing a two per cent alcoholic solution of iodine.

- 1 (30 ml.) bottle containing salvolatile having the dose and mode of administration indicated on the label.

- 1 snakebite lancet.

- 1 (30 gms.) bottle of potassium permanganate crystals.

- 1 pair scissors.

- 1 copy of the first-aid leaflet issued by the Director General, Factory Advice Service and Labour Institutes, Government of India.

- 1 bottle containing 100 tablets (each of 5 gms.) of aspirin.

- Ointment for burns.

- A bottle of suitable surgical antiseptic solution.

- For work places in which the number of contract labour exceed 50. Each first-aid box shall contain the following equipment’s.

  - 12 small sterilized dressings.

  - 6 medium size sterilized dressings.

  - 6 large size sterilized dressings.

  - 6 large size sterilized burn dressings.

  - 6 (15 gms.) packets sterilized cotton wool.

  - 1 (60 ml.) bottle containing a two per cent alcoholic solution iodine.

  - 1 (60 ml.) bottle containing salvolatile having the dose and mode of Administration indicated on the label.

  - 1 roll of adhesive plaster.

  - 1 snake bite lancet.

  - 1 (30 gms.) bottle of potassium permanganate crystals.

  - 1 pair scissors.

  - 1 copy of the first-aid leaflet issued by the Director General Factory Advice Service and Labour Institutes /Government of India.

  - A bottle containing 100 tablets (each of 5 gms.) of aspirin.

  - Ointment for burns.

  - A bottle of suitable surgical antiseptic solution.

Adequate arrangements shall be made for immediate recoupment of the equipment when necessary.

Nothing except the prescribed contents shall be kept in the First-aid box.

The first-aid box shall be kept in charge of a responsible person who shall always be readily available during the working hours of the work place.

A person in charge of the First-aid box shall be a person trained in First-aid treatment in the work places where the number of contract labour employed is 150 or more.

In work places where the number of contract labour employed is 500 or more and hospital facilities are not available within easy distance from the works. First-aid posts shall be established and run by a trained compounder. The compounder shall be on duty and shall be available at all hours when the workers are at work.

Where work places are situated in places which are not towns or cities, a suitable motor transport shall be kept readily available to carry injured person or person suddenly taken ill to the nearest hospital.

###### DRINKING WATER

- In every work place, there shall be provided and maintained at suitable places, easily accessible to labour, a sufficient supply of cold water fit for drinking.

- Where drinking water is obtained from an intermittent public water supply, each work place shall be provided with storage where such drinking water shall be stored.

- Every water supply or storage shall be at a distance of not less than 50 feet from any latrine drain or other source of pollution. Where water has to be drawn from an existing well which is within such proximity of latrine, drain or any other source of pollution, the well shall be properly chlorinated before water is drawn from it for drinking. All such wells shall be entirely closed in and be provided with a trap door which shall be dust and waterproof.

- A reliable pump shall be fitted to each covered well, the trap door shall be kept locked and opened only for cleaning or inspection which shall be done at least once a month.

###### WASHING FACILITIES

In every work place adequate and suitable facilities for washing shall be provided and maintained for the use of contract labour employed therein

Separate and adequate cleaning facilities shall be provided for the use of male and female workers.

Such facilities shall be conveniently accessible and shall be kept in clean and hygienic condition

###### LATRINES AND URINALS

Latrines shall be provided in every work place on the following scale namely :-

Where female are employed, there shall be at least one latrine for every 25 females.

Where males are employed, there shall be at least one latrine for every 25

males

Provided that, where the number of males or females exceeds 100, it shall be sufficient if there is one latrine for 25 males or females as the case may be upto the first 100, and one for every 50 thereafter.

Every latrine shall be under cover and so partitioned off as to secure privacy, and shall have a proper door and fastenings.

Construction of latrines: The inside walls shall be constructed of masonry or some suitable heat-resisting nonabsorbent materials and shall be cement washed inside and outside at least once a year, Latrines shall not be of a standard lower than borehole system.

(A) Where workers of both sexes are employed, there shall be displayed outside each block of latrine and urinal, a notice in the language understood by the majority of the workers “For Men only” or “For Women Only” as the case may be.

  - The notice shall also bear the figure of a man or of a woman, as the case may be.

There shall be at least one urinal for male workers upto 50 and one for female workers upto fifty employed at a time, provided that where the number of male or female workmen, as the case may be exceeds 500, it shall be sufficient if there is one

urinal for every 50 males or females upto the first 500 and one for every 100 or part thereafter.

    - The latrines and urinals shall be adequately lighted and shall be maintained in a clean and sanitary condition at all times.

    - Latrines and urinals other than those connected with a flush sewage system shall comply with the requirements of the Public Health Authorities.

Water shall be provided by means of tap or otherwise so as to be conveniently accessible in or near the latrines and urinals.

Disposal of excreta: - Unless otherwise arranged for by the local sanitary authority, arrangements for proper disposal of excreta by incineration at the work place shall be made by means of a suitable incinerator. Alternately excreta may be disposed of by putting a layer of night soil at the bottom of a pucca tank prepared for the purpose and covering it with a 15 cm. layer of waste or refuse and then covering it with a layer of earth for a fortnight (when it will turn to manure).

The contractor shall at his own expense, carry out all instructions issued to him by the Engineer-in-Charge to effect proper disposal of night soil and other conservancy work in respect of the contractor’s workmen or employees on the site. The contractor shall be responsible for payment of any charges which may be levied by Municipal or Cantonment Authority for execution of such on his behalf.

###### PROVISION OF SHELTER DURING REST

At every place there shall be provided, free of cost, four suitable sheds, two for meals and the other two for rest separately for the use of men and women labour. The height of each shelter shall not be less than 3 metres (10 ft.) from the floor level to the lowest part of the roof. These shall be kept clean and the space provided shall be on the basis of 0.6 sq.m. (6 sft) per head.

Provided that the Engineer-in-Charge may permit subject to his satisfaction, a portion of the building under construction or other alternative accommodation to be used for the purpose.

###### CRECHES

At every work place, at which 20 or more women worker are ordinarily employed, there shall be provided two rooms of reasonable dimensions for the use of their children under the age of six years. One room shall be used as a play room for the children and the other as their bedroom. The rooms shall be constructed with specifications as per clause 19H (ii) a,b& c.

The rooms shall be provided with suitable and sufficient openings for light and ventilation. There shall be adequate provision of sweepers to keep the places clean.

The contractor shall supply adequate number of toys and games in the play room and sufficient number of cots and beddings in the bed room.

The contractor shall provide one ayaa to look after the children in the crèche when the number of women workers does not exceed 50 and two when the number of women workers exceed 50.

The use of the rooms earmarked as creches shall be restricted to children, their attendants and mothers of the children.

###### CANTEENS

In every work place where the work regarding the employment of contract labour is likely to continue for six months and where in contract labour numbering one hundred or more are ordinarily employed, an adequate canteen shall be provided by the contractor for the use of such contract labour.

The canteen shall be maintained by the contractor in an efficient manner.

The canteen shall consist of at least a dining hall, kitchen, and storeroom, pantry and washing places separately for workers and utensils.

The canteen shall be sufficiently lighted at all times when any person has access

to it.

The floor shall be made of smooth and impervious materials and inside walls

shall be lime washed or colour washed at least once in each year. Provided that the inside walls of the kitchen shall be lime-washed every four months.

The premises of the canteen shall be maintained in a clean and sanitary condition.

Waste water shall be carried away in suitable covered drains and shall not be allowed to Accumulate so as to cause a nuisance.

Suitable arrangements shall be made for the collection and disposal of garbage.

The dining hall shall accommodate at a time 30 per cent of the contract labour working at a time.

The floor area of the dining hall, excluding the area occupied by the service counter and any furniture except tables and chairs shall not be less than one square metre (10 sft) per diner to be accommodated as prescribed in sub-Rule 9.

  - A portion of the dining hall and service counter shall be partitioned off and reserved for women workers in proportion to their number.

  - Washing places for women shall be separate and screened to secure privacy.

Sufficient tables stools, chair or benches shall be available for the number of diners to be accommodated as prescribed in sub-Rule 9.

    - 1. There shall be provided and maintained sufficient utensils crockery, furniture and any other equipment’s necessary for the efficient running of the canteen.

2. The furniture utensils and other equipment shall be maintained in a clean and hygienic condition.

    - 1. Suitable clean clothes for the employees serving in the canteen shall be provided and maintained.

- A service counter, if provided, shall have top of smooth and impervious material.

- Suitable facilities including an adequate supply of hot water shall be provided for the cleaning of utensils and equipment’s.

The food stuffs and other items to be served in the canteen shall be in conformity with the normal habits of the contract labour.

The charges for food stuffs, beverages and any other items served in the canteen shall be based on ‘No profit, No loss’ and shall be conspicuously displayed in the canteen.

- In arriving at the price of foodstuffs, and other article served in the canteen, the following items shall not be taken into consideration as expenditure namely:-

  - The rent of land and building.

  - The depreciation and maintenance charges for the building and equipment’s provided for the canteen.

  - The cost of purchase, repairs and replacement of equipment’s including furniture, crockery, cutlery and utensils.

  - The water charges and other charges incurred for lighting and ventilation.

  - The interest and amounts spent on the provision and maintenance of equipment’s provided for the canteen.

- The accounts pertaining to the canteen shall be audited once every 12 months by registered accountants and auditors.

###### ANTI-MALARIAL PRECAUTIONS

The contractor shall at his own expense, conform to all anti-malarial instructions given to him by the Engineer-in-Charge including the filling up of any borrow pits which may have been dug by him.

- The above rules shall be incorporated in the contracts and in notices inviting tenders and shall form an integral part of the contracts.

###### AMENDMENTS

Government may, from time to time, add to or amend these rules and issue directions

- it may consider necessary for the purpose of removing any difficulty which may arise in the administration thereof.

SECTION 9

Nalanda University Contractor’s Labour Regulations as per Standard (C.P.W.D.) Regulation

### C.P.W.D. Contractor’s Labour Regulations

###### SHORT TITLE

These regulations may be called the C.P.W.D./PWD (DA) Contractors Labour Regulations.

###### DEFINITIONS

Workman means any person employed by the Owner or its contractor directly or indirectly through a subcontractor with or without the knowledge of the Owner to do any skilled, semiskilled or unskilled manual, supervisory, technical or clerical work for hire or reward, whether the terms of employment are expressed or implied but does not include any person :-

Who is employed mainly in a managerial or administrative capacity : or

Who, being employed in a supervisory capacity draws wages exceeding five hundred rupees per mensem or exercises either by the nature of the duties attached to the office or by reason of powers vested in him, functions mainly of managerial nature: or3

Who is an out worker, that is to say, person to whom any article or materials are given out by or on behalf of the principal employers to be made up cleaned, washed, altered, ornamental finished, repaired adopted or otherwise processed for sale for the purpose of the trade or business of the principal employers and the process is to be carried out either in the home of the out worker or in some other premises, not being premises under the control and management of the principal employer.

No person below the age of 14 years shall be employed to act as a workman.

Fair Wages means wages whether for time or piece work fixed and notified under the provisions of the Minimum Wages Act from time to time.

Contractors shall include every person who undertakes to produce a given result other than a mere supply of goods or articles of manufacture through contract labour or who supplies contract labour for any work and includes a subcontractor.

Wages shall have the same meaning as defined in the Payment of Wages Act.

  - Normally working hours of an adult employee should not exceed 9 hours a day. The working day shall be so arranged that inclusive of interval for rest, if any, it shall not spread over more than 12 hours on any day.

  - When an adult worker is made to work for more than 9 hours on any day or for more than 48 hours in any week, he shall be paid over time for the extra hours put in by him at double the ordinary rate of wages.

  - a)   Every worker shall be given a weekly holiday normally on a Sunday, in accordance with the provisions of the Minimum Wages (Central) Rules 1960 as amended from time to time irrespective of whether such worker is governed by the Minimum Wages Act or not.

- Where the minimum wages prescribed by the Government under the Minimum Wages Act are not inclusive of the wages for the weekly day of rest, the worker shall be entitled to rest day wages at the rate applicable to the next preceding day, provided he has worked under the same contractor for a continuous period of not less than 6 days.

- Where a contractor is permitted by the Engineer-in-Charge to allow a worker to work on a normal weekly holiday, he shall grant a substituted holiday to him for the whole day on one of the five days immediately before or after the normal weekly holiday and pay wages to such worker for the work performed on the normal weekly holiday at overtime rate.

###### DISPLAY OF NOTICE REGARDING WAGES ETC.

The contractor shall before he commences his work on contract, display and correctly maintain and continue to display and correctly maintain in a clear and legible condition in conspicuous places on the work, notices in English and in the local Indian languages spoken by the majority of the workers giving the minimum rates of wages fixed under Minimum Wages Act, the actual wages being paid, the hours of work for which such wage are earned, wages periods, dates of payments of wages and other relevant information as per Appendix ‘III’.

###### PAYMENT OF WAGES

The contractor shall fix wage periods in respect of which wages shall be payable No wage period shall exceed one month.

The wages of every person employed as contract labour in an establishment or by a contractor where less than one thousand such persons are employed shall be paid before the expiry of seventh day and in other cases before the expiry of tenth day after the last day of the wage period in respect of which the wages are payable.

  - Where the employment of any worker is terminated by or on behalf of the contractor the wages earned by him shall be paid before the expiry of the second working day from the date on which his employment is terminated.

  - All payment of wages shall be made on a working day at the work premises and during the working time and on a date notified in advance and in case the work is completed before the expiry of the wage period, final payment shall be made within 48 hours of the last working day.

  - Wages due to every worker shall be paid to him direct or to other person authorized by him in this behalf.

  - All wages shall be paid in current coin or currency or in both.

  - Wages shall be paid without any deductions of any kind except those specified by the Central Government by general or special order in this behalf or permissible under the Payment of Wages Act 1956.

  - A notice showing the wages period and the place and time of disbursement of wages shall be displayed at the place of work and a copy sent by the contractor to the Engineer-in-Charge under acknowledgment.

  - It shall be the duty of the contractor to ensure the disbursement of wages in the presence of the authorized representative of the Engineer-in-Charge who will berequired to be present at the place and time of disbursement of wages by the contractor to workmen.

  - The contractor shall obtain from the authorized representative of the Engineer-in- Charge as the case may be, a certificate under his signature at the end of the entries in the “Register of Wages” or the “Wage-cum-Muster Roll” as the case may be in the following form:-

“Certified that the amount shown in column No ......................has been paid to the workman concerned in my presence on ........................... at	“

###### FINES AND DEDUCTIONS WHICH MAY BE MADE FROM WAGES

- The wages of a worker shall be paid to him without any deduction of any kind except the following:-

  - Fines

  - Deductions for absence from duty i.e. from the place or the places where by the terms of his employment he is required to work. The amount of deduction shall be in proportion to the period for which he was absent.

  - Deduction for damage to or loss of goods expressly entrusted to the employed person for custody, or for loss of money or any other deduction which he is required to account, where such damage or loss is directly attributable to his neglect or default

  - Deduction for recovery of advances or for adjustment of overpayment of wages, advances granted shall be entered in a register.

  - Any other deduction which the Central Government may from time to time allow.

No fines should be imposed on any worker save in respect of such acts and omissions on his part as have been approved of by the Chief Labour Commissioner.

Note: -  An approved list of Acts and Omissions for which fines can be imposed is enclosed at Appendix-X

No fine shall be imposed on a worker and no deduction for damage or loss shall be made from his wages until the worker has been given an opportunity of showing cause against such fines or deductions.

The total amount of fine which may be imposed in any one wage period on a worker shall not exceed an amount equal to three paise in a rupee of the total wages, payable to him in respect of that wage period.

No fine imposed on any worker shall be recovered from him by instalment, or after the expiry of sixty days from the date on which it was imposed.

Every fine shall be deemed to have been imposed on the day of the act or omission in respect of which it was imposed.

###### LABOUR RECORDS

The contractor shall maintain a Register of persons employed on work on contract in Form XIII of the CL (R&A) Central Rules 1971 (Appendix IV)

The contractor shall maintain a Muster Roll register in respect of all workmen employed by him on the work under Contract in Form XVI of the CL (R&A) Rules 1971 (Appendix V).

- The contractor shall maintain a Wage Register in respect of all workmen employed by him on the work under contract in Form XVII of the CL (R&A) Rules 1971 (Appendix VI).

- Register of accident - The contractor shall maintain a register of accidents in such form as may be convenient at the work place but the same shall include the following particulars:

  - Full particulars of the labourers who met with accident.

  - Rate of Wages.

  - Sex

  - Age

  - Nature of accident and cause of accident.

  - Time and date of accident.

  - Date and time when admitted in Hospital,

  - Date of discharge from the Hospital.

  - Period of treatment and result of treatment.

  - Percentage of loss of earning capacity and disability as assessed by Medical Officer.

  - Claim required to be paid under Workmen’s Compensation Act.

    - Date of payment of compensation.

- Amount paid with details of the person to whom the same was paid.

- Authority by whom the compensation was assessed.

- Remarks

- The contractor shall maintain a Register of Fines in the Form XII of the CL (R&A) Rules 1971 (Appendix-XI)

The contractor shall display in a good condition and in a conspicuous place of work the approved list of acts and omissions for which fines can be imposed (Appendix-X)

- The contractor shall maintain a Register of deductions for damage or loss in Form XX of the CL (R&A) Rules 1971 (Appendix-XII)

- The contractor shall maintain a Register of Advances in Form XXIII of the CL (R&A) Rules 1971 (Appendix-XIII)

- viii) The contractor shall maintain a Register of Overtime in Form XXIII of the CL (R&A) Rules 1971 (Appendix-XIV)

###### ATTENDANCE CARD-CUM-WAGE SLIP

The contractor shall issue an Attendance card-cum-wage slip to each workman employed by him in the specimen form at (Appendix-VII)

The card shall be valid for each wage period.

The contractor shall mark the attendance of each workman on the card twice each day, once at the commencement of the day and again after the rest interval, before he actually starts work.

The card shall remain in possession of the worker during the wage period under reference.

The contractor shall complete the wage slip portion on the reverse of the card at least a day prior to the disbursement of wages in respect of the wage period under reference.

The contractor shall obtain the signature or thumb impression of the worker on the wage slip at the time of disbursement of wages and retain the card with himself.

- EMPLOYMENT CARD

The contractor shall issue an Employment Card in Form XIV of the CL (R&A) Central Rules 1971 to each worker within three days of the employment of the worker (Appendix-VIII).

###### SERVICE CERTIFICATE

On termination of employment for any reason whatsoever the contractor shall issue to the workman whose services have been terminated, a Service certificate in Form XV of the CL (R&A) Central Rules 1971 (Appendix-IX)

###### PRESERVATION OF LABOUR RECORDS

All records required to be maintained under Regulations Nos. 6 & 7 shall be preserved in original for a period of three years from the date of last entries made in them and shall be made available for inspection by the Engineer-in-Charge or Labour Officer or any other officers authorized by the Ministry of Urban Development in this behalf.

###### POWER OF LABOUR OFFICER TO MAKE INVESTIGATIONS OR ENQUIRY

The Labour Officer or any person authorized by Central Government on their behalf shall have power to make enquires with a view to ascertaining and enforcing due and proper observance of Fair Wage Clauses and the Provisions of these Regulations. He shall investigate into any complaint regarding the default made by the contractor or subcontractor in regard to such provision.

###### REPORT OF LABOUR OFFICER

The Labour Officer or other persons authorized as aforesaid shall submit a report of result of his investigation or enquiry to the Engineer-in-Charge indicating the extent, if any, to which the default has been committed with a note that necessary deductions from the contractor’s bill be made and the wages and other dues be paid to the labourers concerned. In case an appeal is made by the contractor under Clause 13 of these regulations, actual payment to labourers will be made by the Engineer-in-Charge after the Competent Authority has given his decision on such appeal.

The Executive Engineer shall arrange payments to the labour concerned within 45 days from the receipt of the report form the Labour Officer or the Superintending Engineer as the case may be.

###### APPEAL AGAINST THE DECISION OF LABOUR OFFICER

Any person aggrieved by the decision and recommendations of the Labour Officer or other person so authorized may appeal against such decision to the Superintending Engineer concerned within 30 days from the date of decision, forwarding simultaneously a copy of his appeal to the Executive Engineer concerned but subject to such appeal, the decision of the officer shall be final and binding upon the contractor.

###### PROHIBITION REGARDING REPRESENTATION THROUGH LAWYER

A workman shall be entitled to be represented in any investigation or enquiry under these regulations by:-

  - An officer of a registered trade union of which he is a member.

  - An officer of a federation of trade unions to which the trade union referred to in clause (a) is affiliated.

  - Where the employer is not a member of any registered trade union, by an officer of a registered trade union, connected with the industry in which the worker is employed or by any other workman employed in the industry in which the worker is employed.

An employer shall be entitled to be represented in any investigation or enquiry under these regulations by :-

- An officer of an association of employers of which he is a member.

- An officer of a federation of associations of employers to which association referred to in clause (a) is affiliated.

- Where the employers is not a member of any association of employers, by an officer of association of employer connected with the industry in which the employer is engaged or by any other employer, engaged in the industry in which the employer is engaged.

No party shall be entitled to be represented by a legal practitioner in any investigation or enquiry under these regulations.

###### INSPECTION OF BOOKS AND SLIPS

The contractor shall allow inspection of all the prescribed labour records to any of his workers or to his agent at a convenient time and place after due notice is received or to the Labour Officer or any other person, authorized by the Central Government on his behalf.

###### SUBMISSIONS OF RETURNS

The contractor shall submit periodical returns as may be specified from time to time.

###### AMENDMENTS

The Central Government may from time to time add to or amend the regulations and on any question as to the application/Interpretation or effect of those regulations the decision of the Superintending Engineer concerned shall be final.

SECTION 10

## LIST OF ACTS AND OMISSONS FOR WHICH FINES CAN BE IMPOSED

LIST OF ACTS AND OMISSONS FOR WHICH FINES CAN BE IMPOSED

In accordance with rule 7(v) of the CPWD Contractor’s Labour Regulations to be displayed prominently at the site of work both in English and local Language

- Willful insubordination or disobedience, whether alone or in combination with other.

- Theft fraud or dishonesty in connection with the contractors beside a business or property of CPWD.

- Taking or giving bribes or any illegal gratifications

- Habitual late attendance.

- Drunkenness lighting, riotous or disorderly or indifferent behavior

- Habitual negligence.

- Smoking near or around the area where combustible or other materials are locked

- Habitual indiscipline.

- Causing damage to work in the progress or to property of the CPWD or of the contractor.

- Sleeping on duty.

- Malingering or slowing down work.

- Giving of false information regarding name, age father’s name, etc.

- Habitual loss of wage cards supplied by the employers.

- Unauthorized use of employer’s property of manufacturing or making of unauthorized particles at the work place.

- Bad workmanship in construction and maintenance by skilled workers which is not approved by the University and for which the contractors are compelled to undertake rectifications.

- Making false complaints and/or misleading statements.

- Engaging on trade within the premises of the establishments.

- Any unauthorized divulgence of business affairs of the employees.

- Collection or canvassing for the collection of any money within the premises of an establishment unless authorized by the employer.

- Holding	meeting	inside	the	premises	without	previous	sanction	of	the employers.

- Threatening or intimidating any workman or employer during the working hours within the premises.

## TENTATIVE SPACE MANAGEMENT PLAN

(Separate Document)

## ARCHITECTURAL PLAN OF THE BUILDING

(Separate Document)

PAYMENT – SCHEDULE FOR ORIGINAL WORK

All running / intermediate & final payments shall be made to the agency in accordance with the following schedule:

- Unless explicitly stated otherwise in the Tender Documents, the Contract shall be for the whole Works, based on the milestones and payment shall be as accepted in the Contract.

- The design notes, calculations, specifications, dimensioned drawings and milestone schedules prepared by the tenderer in respect of his technically acceptable proposal shall be for limited purpose of tender evaluation and for enabling its technical acceptability, price and construction time to be prima facie assessed.

- Irrespective of the estimated quantities and /or dimensioned details for various items of work as furnished in the design notes, calculations, specifications or outline /dimensioned drawings accompanying the tender for the work, the successful tenderer shall carry out all changes, modifications or alterations that may, during the scrutiny of the detailed designs and working drawings, or during construction be considered necessary in the opinion of the Engineer for compliance with the Employer’s Requirements.

- All duties, taxes, fees, octroi and other levies, payable by the Contractor under the Contract shall be included in the total Contract Price submitted by the Tenderer. The evaluation of the Tender by the Employer shall be made on the basis of quoted price only.

- The Payment shall be in accordance with the provisions of Clauses of the schedule of payment.

SCHEDULE OF PAYMENT FOR EPC WORK

All running / intermediate & final payments shall be made to the EPC Contractor in accordance with the following schedule

| Sl. No | Activity | Stage Payment percentage against total contract value |
| --- | --- | --- |
| 1 | Submission and approval of floor plans, conceptual drawings, 3-D views/walkthrough, etc. | 1% |
| 2 | Submission of Detailed Project Report (DPR) and Design Basis and material selection report for all works | 1% |
| 3 | Approval of final Interior Drawings along with plans, elevations, sections for all quadrants G+5 floors including 3D views of all segments | 1% |
| 4 | Submission & approval of macro & micro level work completion schedule in primavera/MS Project software | 1% |
| 5 | Submission & approval of detailed Quality Assurance Plan for all works | 0.5% |
| 6 | Submission of Good for Construction Drawings for Execution of Works duly vetted by PMC & third party proof checking consultant appointed by the University | 0.5% |
| 7 | Completion of all partition and glazing works | 15% |
| 8 | Completion of all wall paneling, acoustic works service bays, false ceiling, workstations for digital library, research cubical, exhibition hall displays, magazine stands, newspaper stands, etc. | 20% |
| 9 | Completion of all other items including windows, doors, flooring, carpeting, tack board, white board, signage and wayfinding, etc. for all areas, Decorative & task lighting, wiring, controls, etc. | 20% |
| 10 | Completion of supply and installation of all movable furniture including book racks, reading chairs, reading tables, customized tables, conference tables, classroom furniture, book trolleys, lounge seating, etc. all complete. | 30% |
| 11 | Final Handing over of all areas along with all documentation, as built drawings, guarantee / warrantee certificates, test reports, etc. all complete. | 10% |
|  | Total percentage = | 100% |

Note:

  - For above all Interior, Furnishing and Furniture supply & installation works pro rata payment will be made as assessed during execution by Engineer in Charge and will be final.

  - The payment stages is tentative and  it can be rescheduled depending upon work exigency/requirement, as recommended by concerned Engineer for Civil & E/M services work in consultation with Engineer-in-Charge and agency.

### LIST OF APPROVED MAKES

### (INTERIOR WORKS AND ASOCIATED CIVIL WORKS)

| 1 | STRUCTURAL STEEL | · TATA STEEL LTD. |  |  |
| --- | --- | --- | --- | --- |
|  |  | · SAIL |  |  |
|  |  | · RINL |  |  |
|  |  | · JSW STEEL LTD. |  |  |
|  |  | . JINDAL STEEL & POWER LTD. |  |  |
| 2 | WOOD | . APPROVED QUALITY SEASONED TEAK WOOD |  |  |
| 3 | AUTOCLAVED AERATED CONCRETE BLOCK | · AEROCON |  |  |
|  |  | · BRIXO |  |  |
|  |  | · ECOLITE |  |  |
|  |  | · BILTECH |  |  |
|  |  | · MAGICRETE |  |  |
|  |  | · ULTRATECH X-TRALITE |  |  |
|  |  | . SIPOREX |  |  |
| 4 | CONSTRUCTION CHEMICALS/ WATERPROOFING COMPOUND/ GROUT/ ADHESIVE/ READY MIX MORTAR | · FOSROC |  |  |
|  |  | · SIKA |  |  |
|  |  | . MYK LATICRETE |  |  |
|  |  | · BASF |  |  |
|  |  | · PIDILITE |  |  |
|  |  | · ARDEX ENDURA |  |  |
|  |  | · CICO |  |  |
|  |  | . KERAKOLL |  |  |
|  |  | . BOSTIK INDIA |  |  |
| 5 | VITRIFIED TILES (GLAZED/ MATT/ ANTISKID) | · KAJARIA |  |  |
|  |  | · JOHNSON-ENDURA |  |  |
|  |  | · ORIENT BELL CERAMICS |  |  |
|  |  | . RAK |  |  |
|  |  | · SOMANY |  |  |
|  |  | · NITCO |  |  |
|  |  | . RESTILE |  |  |
| 6 | VINYL / LINOLEUM FLOOR | . WONDERFLOOR (BRAVO) |  |  |
|  |  | . FORBO |  |  |
|  |  | . MARMOLEUM |  |  |
|  |  | . TARKETT |  |  |
|  |  | . ARMSTRONG |  |  |
|  |  | . PERGO |  |  |
| 7 | WOODEN FLOOR | . WONDERFLOOR (BRAVO) |  |  |
|  |  | . ARMSTRONG |  |  |
|  |  | . MARMOLEUM |  |  |
|  |  | . GREEMLAM MIKASA |  |  |
|  |  | . HAVWOODS |  |  |
|  |  | . EPITONE |  |  |
|  |  | . TARKETT |  |  |
|  |  | . PERGO |  |  |
| 8 | EPOXY FLOORING | . CIPY |  |  |
|  |  | . FLOWCRETE |  |  |
|  |  | . TUFF COAT |  |  |
|  |  | . FORBO |  |  |
| 9 | CARPET | . TANDUS WILDFLOWER TONES |  |  |
|  |  | . ROSETTA |  |  |
|  |  | . FORBO |  |  |
|  |  | . TARKETT |  |  |
| 10 | FLUSH DOORS/ MULTI FUNCTIONAL DOOR | · GREENPLY |  |  |
|  |  | · CENTURY |  |  |
|  |  | · KITPLY |  |  |
|  |  | . ARCHIDPLY |  |  |
|  |  | . DURO |  |  |
| 11 | PLYWOOD/ BLOCKBORD/ PARTICAL BOARD/ MDF | · GREENPLY |  |  |
|  |  | · CENTURY |  |  |
|  |  | · KITPLY |  |  |
|  |  | . ARCHIDPLY |  |  |
|  |  | . DURO |  |  |
| 12 | HIGH PRESSURE LAMINATES | · GREENLAM DECO MIKA |  |  |
|  |  | . FORMICA |  |  |
|  |  | · CENTURY |  |  |
|  |  | · KITPLY |  |  |
|  |  | . ARCHIDPLY |  |  |
|  |  | . AICA SUNMICA |  |  |
|  |  | · MERINO |  |  |
| 13 | NATURAL WOOD VENEERS | · GREENLAM DECOWOOD |  |  |
|  |  | . CENTURY |  |  |
|  |  | · KITPLY |  |  |
|  |  | . DURO |  |  |
| 14 | BIRCH PLY | · GREENLAM DECOWOOD |  |  |
|  |  | . SVEZA |  |  |
|  |  | · LUCO |  |  |
|  |  | . BALTIK |  |  |
| 15 | FIRE RATED DOOR/ AUTOMATION DOOR/ ROLIING SHUTTER / ACCOUSTICAL DOOR | · SHAKTI HORMANN |  |  |
|  |  | · DOORWIN TECHNOLOGIES PVT LTD |  |  |
|  |  | · RADIANT |  |  |
|  |  | · NAVAIR |  |  |
|  |  | . GANDHI AUTOMATION PVT LTD |  |  |
|  |  | . PROMAT |  |  |
|  |  | . ECOTONE |  |  |
| 16 | HARDWARE & ACCESSORIES | · KICH |  |  |
|  |  | · DORMA |  |  |
|  |  | · HAFELE |  |  |
|  |  | · ENOX ASSA ABLOY |  |  |
|  |  | . OZONE |  |  |
|  |  | . DORSET KABA |  |  |
|  |  | . HETTICH |  |  |
|  |  | . ELECTRO PLATING |  |  |
| 17 | GLASS DOOR FITTING/ MISCLLANEOUS FITTINGS /KITCHEN ACCESSORIES | · DORMA |  |  |
|  |  | · HAFELE |  |  |
|  |  | · ENOX ASSA ABLOY |  |  |
|  |  | . OZONE |  |  |
|  |  | . HETTICH |  |  |
|  |  | . HENDERSON |  |  |
|  |  | . GEZE |  |  |
| 18 | S.S. BASKET | · FITWELL |  |  |
|  |  | · KICH |  |  |
|  |  | · GEZE |  |  |
|  |  | · CARLEFE |  |  |
| 19 | SS RAILING | . DORMA |  |  |
|  |  | . OZONE |  |  |
|  |  | · D LINE |  |  |
|  |  | . TECHNORAIL |  |  |
|  |  | . GEZE |  |  |
|  |  | GODREJ, CONNECT ARCHITECTURAL PRODUCTS PVT.LTD |  |  |
|  |  | JINDAL STAINLESS STEEL LTD. |  |  |
| 20 | ALUMINIUM EXTRUSIONS | · JINDAL |  |  |
|  |  | · HINDALCO |  |  |
|  |  | · BIANCO |  |  |
|  |  | · INDALCO |  |  |
|  |  | . BHORUKA |  |  |
| 21 | ALUMINIUM EXTRUSIONS (HEAVY) | · GIESSE |  |  |
|  |  | · ALUALPHA |  |  |
|  |  | . PULSE |  |  |
|  |  | . KINLONG |  |  |
| 22 | ALUMINIUM HARDWARE (HEAVY DUTY) | · GIESSE |  |  |
|  |  | · ALUALPHA |  |  |
|  |  | . PULSE |  |  |
|  |  | · DORMA |  |  |
| 23 | GLASS | · SAINT GOBAIN (SGGI) |  |  |
|  |  | · GUJARAT GUARDIAN(MODIGUARD) |  |  |
|  |  | · HINDUSTAN PILKINGTON(HPG) |  |  |
|  |  | · ASAHI INDIA (AIS) |  |  |
|  |  | · HINDUSTAN NATIONAL (HNGIL) |  |  |
| 24 | DASH FASTNERS/ANCHOR BOLTS | · HILTI |  |  |
|  |  | · FISHER |  |  |
|  |  | · BOSCH |  |  |
| 25 | ADHESIVES | . FEVICOL PIDILITE |  |  |
|  |  | . VAMICOL |  |  |
|  |  | . ARALDITE |  |  |
|  |  | . HENKEL |  |  |
|  |  | . FORMICA |  |  |
| 26 | SCREWS | . RK |  |  |
|  |  | . Nettlefold |  |  |
|  |  | . GRK |  |  |
| 27 | SEALANT | · WAKER |  |  |
|  |  | · GE |  |  |
|  |  | · DOW CORNING |  |  |
| 28 | WHITE CEMENT BASED PUTTY | · BIRLA WALL CARE |  |  |
|  |  | . JK WHITE |  |  |
|  |  | · ASIAN PAINTS |  |  |
|  |  | · AKZO NOBEL DULUX |  |  |
|  |  | · KANSAI NEROLAC |  |  |
|  |  | . JOTUN |  |  |
| 27 | PAINTS/ POLISH | · ASIAN PAINTS |  |  |
|  |  | · KANSAI NEROLAC |  |  |
|  |  | · AKZO NOBEL DULUX |  |  |
|  |  | . JOTUN |  |  |
|  |  | . NIPPON |  |  |
| 28 | TEXTURED PAINT | · SPECTRUM PAINT |  |  |
|  |  | · BACKALITE HYLAM HERRITAGE PAINT |  |  |
|  |  | . LUXTURE |  |  |
|  |  | . RENOVO |  |  |
|  |  | . SIDDHIKA SKK |  |  |
|  |  | . BALAJI WALL TEXTURE |  |  |
|  |  | . DURASCHILD |  |  |
| 29 | FIRE RETARDANT / MARINE/ INDUSTRIAL PAINT | · ASIAN PAINTS |  |  |
|  |  | · KANSAI NEROLAC |  |  |
|  |  | · AKZO NOBEL DULUX |  |  |
|  |  | . JOTUN |  |  |
|  |  | . NIPPON |  |  |
|  |  | . NOBEL |  |  |
|  |  | . PROMAT |  |  |
| 30 | WATER REPALLENT | · WACKER |  |  |
|  |  | · AKZO NOBEL DULUX |  |  |
|  |  | · FOSROC |  |  |
|  |  | . DOW CORNING |  |  |
|  |  | . BASF |  |  |
| 31 | GYPSUM PLASTER | . FERROUS CRETE |  |  |
|  |  | . VANS GYPSUM PVT LTD |  |  |
| 32 | NON ASBESTOS FIBRE CEMENT BOARD | · SHERA |  |  |
|  |  | · VISAKA -V BOARD |  |  |
|  |  | . EVEREST |  |  |
|  |  | . ZYKRON |  |  |
|  |  | . SAINT GOBAIN |  |  |
| 33 | GYPSUM BOARD | · SAINT GOBAIN |  |  |
|  |  | · ARMSTRONG |  |  |
|  |  | . GYPSTONE |  |  |
|  |  | . DIAKEN |  |  |
|  |  | . INDIA GYPSUM |  |  |
|  |  | . LAFARGE |  |  |
| 33 | CALCIUM SILICATE BOARDS | . HYLUX |  |  |
|  |  | . AEROLITE |  |  |
|  |  | . SAINT GOBAIN |  |  |
| 34 | FALSE CEILING SYSTEM | . ARMSTRONG |  |  |
|  |  | . SAINT GOBAIN |  |  |
|  |  | . ANUTONE |  |  |
|  |  | . USG BORAL |  |  |
|  |  | . HUNTER DOUGLAS |  |  |
|  |  | . AEROLITE |  |  |
|  |  | . INTERARCH |  |  |
|  |  | . LAFARGE |  |  |
| 35 | ACCOUSTICAL WALL PANELLING | · ARMSTRONG |  |  |
|  |  | · ECOTONE |  |  |
|  |  | . ANUTONE |  |  |
|  |  | .SAINT GOBAIN |  |  |
| 36 | ACCOUSTICAL DRY PARTITION | · SAINT GOBAIN |  |  |
|  |  | · VISAKA |  |  |
|  |  | . LAFARGE |  |  |
|  |  | . ECOTONE |  |  |
| 37 | GLASS WOOL/ROCK WOOL INSULATION | · SAINT GOBAIN |  |  |
|  |  | . UP TWIGA |  |  |
|  |  | . ANUTONE |  |  |
|  |  | . ROCKWOOL INDIA |  |  |
|  |  | · NATIONAL IND. CO. |  |  |
| 38 | TACK BOARD PANELING | · ARMSTRONG |  |  |
|  |  | · ECOTONE |  |  |
|  |  | . ANUTONE |  |  |
|  |  | .SAINT GOBAIN |  |  |
| 39 | WHITE BOARD/TACK BOARD | .SIGN MARK |  |  |
|  |  | .ALKOSIGN |  |  |
| 40 | STRETCH FABRIC | . ORANGE |  |  |
|  |  | . SERGE FERRARI |  |  |
|  |  | . DRAPE SHOPPE |  |  |
|  |  | . ANUTONE |  |  |
| 41 | BLINDS/CURTAINS | . SERGE FERRARI |  |  |
|  |  | · HUNTER DOUGLAS |  |  |
|  |  | . VISTA |  |  |
|  |  | . MARVEL |  |  |
| 42 | FIRE RATED BOARD | . SPARTAN |  |  |
|  |  | . LAFARGE |  |  |
| 43 | FIRE & SMOKE INTUMSCENT SEAL | . VIPER |  |  |
|  |  | . TWINO |  |  |
|  |  | . RAVEN |  |  |
| 44 | ACOUSTICAL SEAL/ GASKET | . LORIENT |  |  |
|  |  | . RAVEN |  |  |
|  |  | . ENVIRO |  |  |
| 45 | BONDED BAMBOOWOOD PLANT FLOOR | . INOVAR |  |  |
|  |  | . WFI (WOOD FLOORS INDIA) |  |  |
|  |  | . XYLOS |  |  |
|  |  | . CALI BAMBOO |  |  |
| 47 | RAISED ACCESS FLOOR SYSTEM | . UNIFLOOR |  |  |
|  |  | . ANTARES |  |  |
|  |  | . UNITILE |  |  |
| 48 | ACOUSTIC TREATMENT: WALL PANELLING | ARMSTRONG / ECOPHON/ DEXUNE / ANUTONE/ ECOTONE / HIMALAYA / CREDENZE |  |  |
| 49 | SOUND ISOLATION & VIBRATION TREATMENT FOR CEILING: | BSW/ OCEANZ ACOUSTICS/ REGUPOL |  |  |
| 50 | DOUBLE SKIN INSULATED ROOFING SYSTEM : | MULTICOLOR/ KIRBY/ TIGER STEEL |  |  |
| 51 | CARPET FLOORING: | BLOOMSBURG/ OCEANZ ACOUSTICS/ STANTON |  |  |
| 52 | ACOUSTIC DOOR | ICA ACOUSTICS/ OCEANZ ACOUSTICS/ |  |  |
|  |  | ACOUSTICAL SOLUTION INC. |  |  |
| 53 | WHITE CEMENT | J.K. WHITE, BIRLA WHITE. |  |  |
| 54 | WATER PROOFING COMPOUND | TAPCRETE, CICO, ACCOPROOF, IMPERMO, |  |  |
|  |  | FOSROC. |  |  |
| 55 | LOCKS/LATCH | GODREJ, HARRISON, LINK, DOORSET. |  |  |
| 56 | WIRE MESH | STERLING ENTERPRISES, TRIMURTY WELDED |  |  |
|  |  | MESH. |  |  |
| 57 | PRELAMINATED PARTICLE BOARD | GREENPLY, CENTURY |  |  |
| 58 | DASH FASTNERS | HILTI, FISHER, CANON, BOSCH. |  |  |
| 58 | FLUSH DOOR SHUTTERS (DECORATIVE/ NON-DECORATIVE). | SWASTIC, CORBETT ,CENTURY, GREENPLY, ARCHID, JENA |  |  |
| 59 | PVC DOOR FRAME |  | POLYLINE, DUROPLAST, POLLYWOOD, ACCURA. |
| 60 | GLASS FRP DOOR SHUTTERS |  | POLYLINE, DUROPLAST, CACTUS. |
| 61 | BOARD & PLYWOOD |  | DURO, KITPLY, CENTURY. GREEN PLY |
| 62 | HYDRAULIC DOOR CLOSER/ FLOOR SPRING |  | HARDWYN, GODREJ. DORMA |
| 63 | SMOKE SEAL STRIP |  | PROMAT/ASTRO FLAME. |
| 64 | DOOR CLOSER LOCK |  | INGERSOLL RAND/DORMA.,HARDWYN |
| 65 | PANIC EXIT DEVICE |  | INGERSOLL RAND/MONARCH/DORMA. |
| 66 | DOOR COORDINATOR |  | UL LISTED /MONARCH/DORMA. |
| 67 | ANODISED ALUMINIUM HARDWARE (HEAVY DUTY) |  | GODREJ, HAFLE, HARDIMA, EVERITE, SIGMA (ISI MARKED). |
| 68 | TEMPERED / CLEAR / FLOAT / TOUGHENED GLASS |  | PILKINTON, SAINT GOBAIN, ASAHI. |
| 69 | POLYSTER POWER COATING SHADES |  | NEROLAC, BERGER, J&N. |
| 70 | ALUMINIUM SECTIONS |  | JINDAL, HINDALCO, INDALCO, NALCO |
| 71 | FRICTION STAY HINGES |  | EARL-BIHARI. |
| 72 | NUTS, BOLTS AND SCREWS, STEEL |  | KUNDAN.PRIYA ATUL. |
| 73 | EPDM GASKET |  | HANU/ANAND. |
| 74 | STRUCTURAL SILICON |  | DOW CORNING/WACKER. |
| 75 | WEATHER SILICON |  | DOW CORNING/WACKER. |
| 76 | GLAZED CERAMIC TILES |  | KAJARIA, SOMANY, JOHNSON, ORIENT BELL CERAMICS, /NITCO |
| 77 | VITRIFIED TILES DOUBLY CHARGED QUALITY |  | KAJARIA, JOHNSON (MARBONITE), AGL, RAK CERAMIC,SOMANY |
| 78 | TILE ADHESIVE |  | CICO, PIDILITE,FOSROC, SIKA.,ENDURA,LATICRETE |
| 79 | CHEQUERED TERRAZO TILES |  | NITCO, BHARAT, PODDAR. |
| 80 | GRASS PAVER |  | UNISTONE, ULTRA. |
| 81 | WATER-PROOF CEMENT PAINT |  | SNOWCEM, ASIAN. |
| 82 | SYNTHETIC ENAMEL PAINT |  | BERGER, NEROLAC, ASIAN, ICI |
| 83 | ACRYLIC EMULSION PAINT |  | ASIAN, BERGER, NEROLAC, ICI, DULUX. |
| 84 | VITREOUS CHINA SANITARYWARE |  | PARRYWARE, HINDWARE, JOHNSON & JOHONSON, JAGUARE, CERA |
| 85 | STAINLESS STEEL SINKS |  | NILKANTH, AMC, CORBA, NIRALI, JYNA |
| 86 | C.P.BRASS FITTINGS |  | JAQUAR, MARC, PARRYWARE, JOHNSON, HINDWARE, CERA |
| 87 | LA( CI) PIPES |  | RIF, NECO,KAPILANSH |
| 89 | G.I.PIPES |  | TATA, JINDAL, HISSAR,PRAKASH SURYA |
| 90 | G.I.FITTINGS (MALLEABLE CAST IRON) |  | UNIK, ICS. |
| 91 | GUNMETAL VALVES |  | LEADER, SANT, ZOLOTO, KIRLOSKAR |
| 92 | UPVC DOORS &WINDOWS (PROFILE MAKERS & THEIR AUTHORISED FABRICATORS ONLY) |  | DUROPLAST, ALUPLAST, KOMMERLING, FINISTA |
| 93 | UPVC DOORS &WINDOWS HARDWARE |  | ROTTO, DORSET, KINLONG |
| 94 | HEAT RESISTANCE TITLES |  | SWASTIK, THARMA TECH |
| 95 | MIRROR GLASS |  | ATUL, MOGIGUARD, JAGUAR, CERA |
| 96 | TEXTURE PAINT |  | ASIAN,NEROLAC, |
| 97 | METAL GRID FALSE CEILING |  | ARMSTRONG, HUNTER DOUGLS. |
| 98 | SS ARCHITECHRAL HARDWARE |  | EBCO, HETTICH, HAFFLE |
| 99 | FACTORY MAKE WOODEN FLOORING |  | GREENLAM, ACTION TESSA |